From: garyf@puente.jpl.nasa.gov (Gary Friedman)

Newsgroups: comp.sys.hp48

Subject: Infrared Remote Control Learn/Transmit Source Code for the HP48

Summary: Direct from the Corvallis conference proceedings

Keywords: IR, remote, learning, infrared, world is drooling over this...

Date: 9 Aug 91 18:26:27 GMT

Distribution: comp.sys.handhelds, comp.sys.hp48

Organization: Jet Propulsion Laboratory, Advanced Engineering Group

Nntp-Posting-Host: puente

Okay, okay!  Here is the article on the IR Remote control for the 48!!!!

Remember, this was literally the first cut, and there is gobs of room

for improvment and polishing!

Enjoy!

-Gary Friedman

------------------------------------------------------------------

IR Sampling of TV Remotes

by Dave Marsh  (Typed in by Gary Friedman)

Many users have experimented with the HP 48 to make it capable of

being a TV remote control unit.  With its built-in IR capability

numerous schemes have been tried to give the HP 48 additional power.

Previously these schemes involved knowing the digital codes used to

modulate the 40 KHz IR signal commonly used.

With the recent release by HP of their Software Development Tools it

is possible to write assembly programs for the HP 48.  Faster speed

and greater control of the calculator are gained using this approach.

A method that occurred to me during lunch one day - I do development

work on the HP 48 as part of my job - is implemented in the two

programs on the following pages.  The basic idea is to sample the

incoming IR as fast as possible to digitize the signal and record it.

This takes lots of memory, but it actually works.  Using this method

of "learning" the codes instead of knowing them allows the HP 48 to

be used by any IR device.  The inverse is used with the second program

using the stored sampled signal (stored as a text string) to control

the transmitter.

This approach is still in the experimental stages and there is much

work to be done.  Applications include controlling systems such as the

BSR system that has in IR controller - at least they used to offer an

IR controller.  Your HP 48 could, with off-the-shelf components and a

simple program, control the lights and appliances of your home or

factory.

The sampling rate is estimated at about 60 kilohertz, still short of

the desired twice the carrier frequency.  Still it works quite well.

I have written preliminary programs that convert the digitized signal

into a graphics object that may be displayed or printed.  This aids

the decoding of the signal if that is of interest.

Once the optimum signal is determined it may be stored in a more

memory-efficient form.  A complete set of commands for any IR

controller could be made available to any user.  This aspect of the

programming ahs yet to be done.  Copies of the program will be

available at the conference and time does not allow a complete

description of the program here.  The listings are commented and there

is always room for improvement.  One problem is the memory usage of

using one nibble where only one bit is needed.  Perhaps other

programmers will find additional techniques to make the program

faster.

Dave Marsh

18335 Jakes Way #103

Canyon Country, CA 91351

(818) 362-6822 x3007 (W)

[Poster's note: I will try to post an ASC-> version of the binary this

weekend.   -GF]

-----------------------------------------------------------------

*	LOOK

*	A PROGRAM TO SAMPLE I.R. REMOTE CONTROL SIGNALS

*

*	1:	STRING ->    1:    STRING

*

*	(C) 1991 DAVE MARSH - THIS PROGRAM MAY BE DISTRIBUTED ONLY IF

*	IT'S FREE OF CHARGE

*

*	(Insert your favorite disclaimers here)

*

ASSEMBLE

NIBASC  \HPHP48-F\

RPL

::

CK1NOLASTWD			( CHECK FOR 1 ARG )

CK&DISPATCH1		( CHECK ARG IS STRING )

THREE

::

TOTEMPOB

CODE

GOSBVL #01115		DisableIntr (FOR SMOOTH INPUT)

INTOFF

A=DAT1 A			SET A TO POINT AT BEGINNING OF STRING

GOSBVL #0679B		SAVE REGISTERS (SAVEPTR)

D1=HEX #0011A		11Ah IS ADDRESS OF I.R. INPUT

P= 0

C=0 P

DAT1=C P			SET 'TRIGGERED' BIT TO 0

D0=A

D0=D0+ 5			D0 POINTS TO LENGTH FIELD OF STRING

C=DAT0 A

C=C-CON A, 5		SUBTRACT 5 FROM LENGTH IN ORDER TO GET

DATA LENGTH ONLY

D0=D0+ 5			D0 POINTS TO DATA AREA OF STRING

A=A+CON A, 10		SET A = D0

C=C+A A			SET C= ADDRESS OF END OF STRING

B=A A

LOOP1 A=DAT1 P			LOOP UNTIL TRIGGER BIT IS 1 (I.R. SIGNAL REC'D)

?ABIT=0 0

GOYES LOOP1

LOOP2 A=DAT1 P			GET I.R. DATA AND STORE AS NIBBLE OF STRING

DAT0=A P

D0=D0+ 1

B=B+1 A

?B#C A			LOOP UNTIL D0=END OF STRING (STORAGE AREA)

GOYES LOOP2

GOSBVL #010E5		AllowIntr ( re-enable interrupts )

GOVLNG #05143		RESTORE REGISTERS (GETPTRLOOP)

ENDCODE

;

;

------------------------------------------------------------------

*	XMITREM

*	A PROGRAM TO EMULATE AN I.R. REMOTE CONTROL TRANSMISSION

*	REQUIRES OUTPUT OF 'LOOK' PROGRAM AS INPUT

*

*

*	1:   STRING ->  EMPTY STACK

*

*

*	(C) 1991 DAVE MARSH- THIS PROGRAM MAY BE DISTRIBUTED ONLY IF

*	IT'S FREE OF CHARGE

*

*

ASSEMBLE

NIBASC \HPHP48-F\

RPL

::

CK1NOLASTWD

CK&DISPATCH1			( CHECK FOR 1 ARG- STRING )

THREE

::

TOTEMPOB			( MAKE NEW COPY OF STRING IN TEMPORARY MEMORY )

CODE

GOSBVL #01115		DisableIntr (FOR SMOOTH TRANSMISSION)

INTOFF

A=DAT1 A			A POINTS TO INPUT STRING (OUTPUT FROM LOOK)

D1=D1+ 5

D=D+1 A

GOSBVL #0679B		SAVEPTR

P= 0

C=0 P

D1=HEX #0011A		11Ah IS ADDRESS OF I.R. INPUT & CONTROL

DAT1=C P			SET IR CONTROL BITS TO 0

D0=A

D0=D0+ 5			D0 POINTS TO LENGTH FIELD OF STRING

D1=HEX #0011C		11Ch IS ADDRESS OF I.R. LED CONTROL (BIT 3)

DAT1=C P			MAKE SURE LED IS OFF

C=DAT0 A

C=C-CON A, 5		C IS LENGTH OF DATA STRING ONLY

D0=D0+ 5			D0 POINTS TO DATA FIELD OF STRING

A=A+CON A, 10		SET A = D0

C=C+A A			  SET C= ADDRESS OF END OF STRING

B=A A

R1=A

LOOP2 A=DAT0 P

A=A-1 P		SUBTRACT 1 FROM NIBBLE (FORMS BIT 3 ON OR NOTHING)

DAT0=A P			AND PUT BACK

D0=D0+ 1			INCREMENT NEXT NIBBLE

B=B+1 A

?B#C A			LOOP UNTIL END OF STRING

GOYES LOOP2

A=R1

B=A A

D0=A A

LOOP3 A=DAT0 P

DAT1=A P			SEND DATA TO LED (OFF OR ON)

D0=D0+ 1			INCREMENT TO NEXT NIBBLE

B=B+1 A

?B#C A			LOOP UNTIL END OF STRING

GOYES LOOP3

A=0 P			MAKE SURE LED IS OFF

DAT1=A P

GOSBVL #010E5		RESTORE INTERRUPTS

GOVLNG #05143		GETPTRLOOP

ENDCODE

;

--

----------------------------------------------------------------

Gary Friedman                   Jet Propulsion Laboratory - NASA

4800 Oak Grove Drive, Pasadena, CA.  91109        (818) 354-1447

garyf@puente.JPL.NASA.GOV || {cit-vax,elroy,psivax}!devvax!garyf