From: robin@ireq-robot.hydro.qc.ca (Christian Pronovost)

Subject: modification to SC ram cards

This is an article that I took on this newsgroup a few months ago. This

article is very interesting if you want to modify your SC card. It

seem that the Epson Ram card and the SC ram card are identical.

The article was writen by Allan Petersen, i have added some comments

in the text between the (())...

I have also putted some comments at the end of this message on how to

modify the SC just by adding a 43khoms resistor.

---->By the way i think i wrote 33khoms in my last article, i was wrong

but it can also work since the value of the resistor is not critical.

I use a 43k resistor because the highest it is, the less current it

consumes(P=VI ;). The resistor value can`t be mutch higger than 47k. I think

43k is the less conssuming and safest value...

>-------------------------------------------------------------------<

Epson RAM card surgery ...  or:  All about RAM cards.

------------------------------------------------------------

by   Allan Petersen

Dansk Data Elektronik A/S,   Copenhagen

------------------------------------------------------------

Shortly after  the introduction of the HP48sx it was mentioned here

on c.s.h. that Epson RAM cards possibly could be used in the 48 and

that  HP RAM cards in fact were custom versions of Epson cards.  It

was later pointed out that the standard Epson RAM cards should  not

be  used  in  the 48 because of different electrical specifications

than the HP custom version.

I followed this discussion with interest and later when  I  got  my

48sx,  I  looked  for further info on the specs of Epson cards.  I

found some specs and studied them carefully;  recently I  found  a

supplier of cards and asked for prices and delivery times.

I  have now got a RAM card,  have made some tests on it,  and have

made some comparisons to the CMT RAM card and the HP Eq.  Lib. card

(as mentioned recently by a colleague of mine Arne Christensen here

at DDE).

Here I will describe the test results; but first the prices:

In Denmark the price of the 32k Epson RAM card is 72 $ and the 128k

Epson  card  is  157  $.  Both  prices  are  based on single piece

quantities and are excl. sales tax.

These  prices  more  or  less  correspond to the EduCalc prices of

HP/CMT cards but as the 32k/128k Epson card cost  respectively  81%

and 56% of the 32k/128k HP card here in Denmark, I would expect the

Epson cards to be slightly cheaper in the States than the mentioned

prices from here.

However  from  a  cost point of view there is not much to hope for,

the best advice seems to be: Buy from EduCalc.

Well, being of a curious mind, I could not resist the challenge and

finally got a 32k Seiko-Epson RAM card (type  no.  RBC032IE10,  the

128k  card  is  called RBC128IE10) hoping to be able to change the

voltage detect level and/or add some extra RAM chips to  the  card.

Other types (OTP EPROM/EEPROM/Flash EEPROM/MASK) are also available

from Seiko-Epson.

It was indeed somewhat of a challenge but so far I have managed  to

change  the detection level,  which I will describe to you a little

later; first concerning the specs:

I can in every respect confirm that every piece of  information  on

the  HP  cards given here by Jim Dickie,  Steve Harper and Preston

Brown  (all  from  HP  Corvallis)   is   in   fact   correct   and

considered:

I  have  found  that the standard Epson RAM card that I have clamps

the address and control lines to ground until  the  supply  voltage

reaches 4.20 volts and again clamps the lines when the voltage goes

lower than 4.15 volts.

The data lines are not clamped.

These  measurements were made with the card inserted in an original

Seiko-Epson card socket,  not the 48.

The  voltage supplied from my 48 to the card is 4.24 volts when the

48 is  running  programs  (battery  midt-life  or  so,  version  E

calculator).

This  means  that  there is typically  a  chance that the card will

work if inserted, but the margin is very limited. As earlier stated

here the powersupply of the 48 is stabilized and not very dependent

of battery state; however when the batteries become almost empty at

some  point  the  stabilizer wil not be able to sustain the needed

voltage and the RAM card will clamp its inputs long before  the  48

stops  operating.  This  has a very high chance of causing a memory

lost (or something worse...) as the clamps are rather effective: On

my  card it takes about 10 mA per line to pull the line to 0,4 volt

and 25 mA to pull to 1 volt.  This corresponds  to  about  40  ohm

on-resistance,  which  makes  me  guess  that  the controller uses

on-biased MOSFETs on the inputs in the clamp-state.

There is no way the 48 will be able to pull-up all  these  lines,

therefore:  DO NOT USE A STANDARD EPSON RAM CARD IN YOUR HP48 !!

((OR SC CARDS))

It may work for a time, but you are on very, very thin ice here !

The only way to safely use an otherwise standard Epson RAM card  is

to change the clamp voltage detect level on the card and at the end

of this article I will describe how to do this;  but if you do  not

feel  absolutely  sure  of  what  you  are doing,  I will strongly

recommend that you order a HP card from eg.  EduCalc (I can  highly

recommend EduCalc for their excellent service,  I speak of personal

experience). The cost saving - if any - on changing a standard card

is  not  worth  the change of a possibly damaged card - or damaged

calculator :-).

BTW:  No,  I have no connection in any way to HP,  I just happen to

love their products and quality!

Some more specifications:

The  card  has  of  course the same mechanical dimensions as the HP

card,  although  it has no plastic handle to help in  removing  the

card  from  the calculator,  it has 40 pin connections on a contact

pitch of 1.27 mm,  pattern width 0.8  mm,  pattern  gap  0.47  mm,

contacts have min.  0.3 micrometer gold over min.  5 micrometer Ni,

and the shutter mechanism is guaranteed for 10,000 insertions.  The

front  and  rear  are covered  by stainless steel panels to protect

against statics, noise and shock. The card and connector are keyed,

preventing incorrect insertion.

The data interface is 8 bits wide.

I  have  made some measurements of current consumption,  but I will

not give you any figures,  as there is often a factor of up to  100

between the max. RAM chip current over temp and the typical current

at room temp and it is in fact this current that  will  decide  the

lifetime  of the back-up battery;  the current consumed by the card

at 4.4 volt (or other voltage) from the  calculator  is  irrelevant

from a back-up point of view,  although it in some cases may have a

small influence on the lifetime of the batteries in the  calculator

itself.

The main part of the card's standby current consumption from the 48

is probably caused by the controller chip (gate array) in the Epson

card;  the standby current is specified as max.  1.15  mA,  but  a

current this high is NOT taken from the back-up battery when the 48

is off.  Only the RAM chip data retention current and a very  small

current  for  the  controller to keep its clamps on-biased is taken

from the Lithium battery.

The CMT card is claimed to use much less than the 1.15 mA, which is

very  likely,  as  the CMT card is probably designed using standard

CMOS chips, but this difference does not need to have much relation

to the actual back-up currents from the Lithium battery.

I agree that the Epson card controller will consume a small current

to keep the clamp MOSFETs on and to bias the  gatearray,  but  this

should not be that significant.

The standard Epson card is specified for  a  typ.  data  retention

periode of 4 years at room temp.

When you look down at the gold contacts and hold the battery end of

card away from you, the leftmost pin is pin number one.

The pin connections are:

PIN:   Signal:                    PIN:   Signal:

1      Power supply input         21     Card enable (active HIGH)

2      Batt. out/ EPROM Vpp       22     Output enable (active low)

3      Addr. 0                    23     Data 0

4      Addr. 1                    24     Data 1

5      Addr. 2                    25     Data 2

6      Addr. 3                    26     Data 3

7      Addr. 4                    27     Data 4

8      Addr. 5                    28     Data 5

9      Addr. 6                    29     Data 6

10     Addr. 7                    30     Data 7

11     Addr. 8                    31     nc     How are these

12     Addr. 9                    32     nc     lines connected

13     Addr. 10                   33     nc     in the 48?

14     Addr. 11                   34     nc     Could really be

15     Addr. 12                   35     nc     nice to have

16     Addr. 13                   36     nc     waitrequest here!!

17     Addr. 14                   37     Write prot. out (act low)

18     Addr. 15                   38     Card present out(act high)

19     Addr. 16                   39     Device type (low for RAM)

20     Write enable (active low)  40     Ground

It  can  be  seen  that the card enable signal is active high;  on

standard EPROMs the enable signal is active low,  so if you try  to

make  an  adaptor for an EPROM programmer,  you need to invert this

signal in your adaptor.

Some signal lines are used to indicate card present,  writeprotect,

and ROM/RAM type.  If the writeprotect switch on the card is set to

protect position,  the 48 will not be able to change the card data,

not even in a memory lost/lock-up situation.

The card battery voltage is measured via a 1M resistor.

And now to the exciting part:    OPENING THE CARD.

Remove the battery and use a scalpel to very carefully lift and cut

loose the stainless steel  panel  on  the  upper  side  (the  side

opposite  to  the  side  where  the  contacts  and the shutter are

located). Be careful not to damage the inside of the card - or your

fingers :-).

Near  one  edge  by  the  battery  there is a metal spring used to

connect the two steel panels to keep them at  the  same  electrical

potential.  You  will probably damage this spring when you open the

card (so did I).  You must remember to put in a new spring when you

assemble  the  card  again (I used a piece of conductive rubber) in

order to keep the protection against statics.  Also be careful  not

to  damage  the springs for the shutter mechanism;  put a piece of

tape over them until you assemble the card again.

When you have removed the panel, you will (on the 32k card) see the

following:

A position for a 2016 Lithium battery (coin cell, not supplied with

the card) and close to this the write protect switch; positions for

4  pieces  of  32k  RAM  chips,  with  one position occupied.  One

decoder/controller chip (Seiko-Epson proprietary gate  array),  and

another  smaller  chip which is the voltage detector chip.  Several

surface mount capacitors used for  decouplings  and  some  surface

mount chip resistors. Four gold plated test points.

The  PCB  is  of good quality with gold plated contacts and bonding

pads.  Also the plastic card frame and the shutter mechanism  seems

to  be of high quality,  so in my opinion the card will stand up to

the usual high quality that we associate with HP products.

There is no plastic handle like the one found on the HP version, so

the card can be difficult to remove if inserted into the 48, but it

is easy to glue some kind of grip onto the card if you wish to.

Unfortunately the chips are fastened directly to the PCB, bonded to

bondingpads  on  the PCB and covered with drops of epoxy,  so it is

not possible to use standard small-outline RAM chips to extend  the

memory.  The chips actually used are standard dies (you can see the

familiar bonding pad layout known  from  eg.  27C256  EPROMs  with

quartz window).

You  can't  solder out the detector chip either :-(;  but the test_

point located away from the three others happens to be  the  output

of the voltage detector.

((NOT SO EVIDENT ON THE SC CARD, THIS CHECK POINT IS ABOUT IN THE

MIDDLE OF THE SC CARD ON THE COMPONENTS SIDE ABOUT 1.5 CM OF THE

CONNECTOR PINS. THE VOLTAGE DETECTOR IS ALSO IN THE MIDDLE OF THE

CARD: IF YOU LOOK AT THE OPENED CARD, PINS DOWN (the connector) THE

VOLTAGE DETECTOR IS THE FIRST COMPONENT IN THE MIDDLE OF THE CARD FROM

THE BOTTOM OF THE CARD))

If you cut the PCB trace (be careful...) from the smaller IC to the

testpoint,  you can now via the testpoint access the  clamp  enable

input of the gate array!

(( --> don't do this you can change the hysteresis of the voltage

detector just by addind a resistor!! ))

This  is  very interesting,  because you can now add a new voltage

detector IC and thereby change the detection  level  at  your  own

choice (provided you can get the correct detector IC).

The  detector  IC requires three signals:  Supply voltage from the

card connector,  ground,  and a detector output, which is connected

to the clamp enable input of the gate array.

The three testpoints close together are ground (closest to the card

edge),  some kind of detector checkpoint,  and card supply voltage.

It is not possible to change the original detector  level  via  the

detector  checkpoint,  but  the detector hysteresis may be changed

(although this just makes things worse :-)).

((FALSE: PLACING A RESISTOR CHANGES THE HYSTERESIS AND LOWER THE

CLAMPING VOLTAGE.))

I know of two types of voltage detectors that can be used:

Seiko Epson Corp.  type SCI7701YHA  (typ. 3.2 volt)

or  Seiko Instruments Inc.  type S-8053ALR-LJ-X  (typ. 3.25 volt).

These are SOT89 surface mount packages and can be connected to  the

required testpoints with thin, isolated wirewrap wire.

I do not know the price of the detectors, I just got a few samples.

I have placed the detector in one of the three empty positions  for

RAM  chips  and  with  a  fine  solder tip I melted grooves in the

plastic frame to make room for the wires.

If you can get a detector IC,  you will probably also be  able  to

find  the  pinconnections  of the IC,  so I will not go into these

details.

You may use a different voltage version of the detector (when using

an  open  drain  version  instead  of  a CMOS version a pull-up is

required),  but the voltage should be between 3.2 and 3.9 volts for

best  results.  If  the  voltage  is too low,  you may end up in a

situation where the RAM chips may  take  power  from  the  back-up

battery  while  they  are  active;  this  will exhaust the back-up

battery very fast and you may even risk to loose data  memory  when

your main batteries are down.

Under  normal  battery  conditions  this will not occur with a low

detector voltage as the supplied voltage from  the  48  will  drop

rapidly below the clamp voltage when the 48 is switched off;  so if

you just switch off your 48 immediately when a low main battery  is

detected,  you  can  safely get away with using a detector voltage

down to about 2 volts. I have tried 2.6 volt with no problems.

After these changes carefully glue the panel in place  again,  glue

on a plastic handle, put on some nice labels on the card, install a

2016 lithium battery,  set a  battery  alarm,  and  you  have  got

yourself a RAM card for your 48.

Sounds complicated?     Well,  then  don't do it;  but if you tried

something like the memory expansion of the HP28C  as  described  by

Wlodek, I think you will find this very interesting also.

Some comments on the CMT card:

I have found that the CMT card does not clamp the signal lines,  so

in this respect you won't get problems. However this could make the

pins more sensitive to static electricity, especially when there is

no shutter mechanism to cover the contacts.  CMT also warns you  to

keep  the  card  in the anti-static bag when the card is not in the

48.

It has been claimed that CMT uses tranzorbers on the  signal  lines

for protection; sounds good, that will give you adequate protection,

but then why does CMT put so much emphasis on  the  protective  bag

issue?  I  have  not  tested  the  card  with voltages outside the

specified normal range,  and I have not seen the inside of the card

(Arne  would  not  let  me open his CMT card :-) :-)) so I can not

really judge how protection is made.

On the other hand I  have  not  yet  seen  commercially  available

tranzorbers  for  surface  mounting - but I may be wrong here;  if

someone will give me a CMT card,  I certainly will make  a  closer

examination :-).

The  mechanical dimensions of the CMT and the HP cards are somewhat

different:  the CMT card has a thickness of 2.0 mm at  the  contact

area  and  a thickness from 2.42 to 3.0 mm on the body,  where the

measures of the HP card are 1.80 mm and 2.3 mm.  Thus the  required

insertion force for the CMT card is rather high.

It  is  very difficult to see if the CMT card corresponds correctly

with the connector keying or not,  but when inserted in a  separate

card  connector  you can see that the contact fingers are not fully

depressed,  there is still a clearance of about 0.2 - 0.3 mm behind

them,  so I assume that the contacts themselves will not be damaged

by inserting the CMT card, but I can't see if that is also the case

with the sockets in the 48 itself, as I have not taken the 48 apart

to see if the sockets are standard Epson sockets.

The contacts of the CMT card does not seem to be of the same quality

as  on the original cards;  the PCB traces have blurred edges,  not

straight and sharp lines as you usually see on PCB's for professional

use,  but if the traces on the board are intact, you may argue that

these are only cosmetic flaws.

After all,  I do not see any really serious reasons why you  should

not use a CMT card in your 48 calculator as long as you mainly keep

the card in your calculator instead of using  it  as  an  exchange

medium; but I don't like the high insertion force. If you are going

to make a card for the 48,  why not make it  the  same  mechanical

dimensions as the original cards?

I think the price difference is not enough to justify the electrical

and mechanical differences.

What is the next project here?     Well,  some form  of  programmer

for Epson EPROM or EEPROM cards,  I guess.  I have noticed that the

HP Equation Library card - and  therefore  also  the  Epson  EPROM

cards,  I  hope  -  do  not clamp the signal lines on a low supply

voltage.  It will then be easy to design a  programmer  for  these

cards and use them in the 48.

Some  time  ago HP released a very useful MSDOS program USRLIB that

enables you to build libraries for the 48.

However I do not have a PC,  so I was very,  very pleased  to  see

Frank  Ochoa's  TOOLS.48  that Friedrich Schroeder sent to c.s.h.

recently.  Very nice. I can recommend the >LIB function. Now we can

build  libraries  in  the  48 itself.  Great.  Then we just need a

convenient way of programming EPROM/EEPROM cards (I know  that  CMT

has a programmer,  but it is priced at 280 $ ...,  I also know that

EduCalc has a programming service,  but I would like to do this  at

home).

I'll  try to work something out,  so that I can make my own library

cards, so stay tuned.

Now,  after  going through all this,  what would I really recommend

you to do concerning RAM cards? In my opinion it goes like this:

- You CAN modify standard Epson cards but it is  hardly  worth  the

effort,  so  buy the HP cards and buy them where you can get them

at the best price (I can recommend EduCalc).  This will give  you

the best value for your money in the long run.

Disclaimer:   Don't blame me if something goes wrong  if  you  use

some of my ideas;  these opinions are mine alone, but

you may have them if you want to :-).

I do not speak on behalf of anybody else and have no

connections to HP, Educalc or Epson.

Even  my  employer  may have opinions different from

mine and can not be held responsible either.

Allan

-------------------------------------------------------------------

Allan Petersen     HP25/HP19C/HP15C/HP28S/HP48SX/HP??    ap@dde.dk

-------------------------------------------------------------------

NOTES ON HOW TO ADD THE 43k RESISTOR:

So if you want to change the hysteresis of the voltage resistor,

you have to add a 43 khoms resistor between the input voltage (pin #1)

and the outpout of the voltage resistor witch is about in the middle

of the card (as described in this article).

Christian