BCD numbers (binary coded decimal) are represented by a mantissa,

sign and exponent fields.  There are two kinds of BCD numbers in

the HP 48SX, 12-form and 15-form BCD numbers, called BCD12 and

BCD15 in the field descriptions.

Both kinds have several things in common:

*    A mantissa.  Each digit of the number is represented by

one nibble in the mantissa.  12-form BCD numbers have a

12 digit mantissa and 15-form BCD numbers have a 15

digit mantissa.  There is an implied decimal point after

the Most Significant Digit (msd) with respect to the

exponent.

*    A sign nibble.  Positive numbers are represented by a

0 and negative numbers are represented by a 9.

*    An exponent.  The exponent is in 10's compement.

12-form BCD numbers have a 3 nibble exponent and 15-form

BCD numbers have a 5 digit exponent.

12-form BCD numbers are the basis for Real Number, Complex

Number, Real Array and Complex Array data types.

15-form BCD numbers are the basis for Long Real and Long

Complex data types.

BCD12:

15 14                     3  2   0

+--+------------------------+------+

|S | msd..  Mantissa  ..lsd | Exp  |

+--+------------------------+------+

1             12              5

BIN15:

21 20                           5  4       0

+--+------------------------------+----------+

|S | msd...    Mantissa    ...lsd |   Exp    |

+--+------------------------------+----------+

1                15                   5

------------

This is from my 'HP 48SX Internals' document that is getting close to its

first posting.  Hope it helps.

Take a look at the Real Number and Long Real objects starting with

0 (Real Number) at 2A2B4 to 10 (Long Real) at 2A596.  There are 34

objects in that range.  Most are simple.  But includes PI and MAXR...

Derek S. Nickel