HP48 feature list

Alonzo Gariepy has pointed out to me that people might like to see

a list of HP 48SX features, sent out by the firmware design team

leader, Bill Wickes, to the reviewers before we got the calculators

themselves. Here is a copy of that list. Many thanks, Alonzo and Bill.

Wlodek Mier-Jedrzejowicz,

Space Physics, Imperial College, London

BITNET: MIER @ SPVA.PH.IC.AC.UK

Disclaimer: This information is presented as is. NO CLAIMS about

its sutitability for any purpose are made by HP, my employers or

myself. It is simply a copy of a private communication whose author

has allowed me to make it available to a wider audience.

----------------------------------------------------------------------

Here's a slightly edited, although still not exhaustive list of features

(in no particular order) that I made recently:

HP-48SX Feature Summary

Hardware

7.1"x3.2" footprint with tapered .7" - 1.1" thickness

single vertical format 49-key keyboard

64 x 131 pixel STN LCD display

256K ROM, 32K RAM built in

Saturn CPU @ 2MHz (note--throughput ~1.5MHz compared to 28S @1MHz)

2 memory card ports

--battery backed RAM (32K or 128K)

--ROM or OTP (OneTime Programmable)

Bidirectional IR I/O at 2400 baud

Wired serial port at 1200, 2400, 4800, 9600 baud

cables available for IBM or Apple computers (~$35)

3 AAA cells

List price $350

Two part-manual (800p) plus quick reference guide

Brushed nylon case with velcro fastening, pocket for cards and guide

Firmware

(Most HP 28 features not explicitly mentioned are included; "new" means "in

addition to that provided by the HP 28S.")

HP-28-style multi-object-type RPN calculator, superset of 28 language

Interactive stack environment, for point-and-shoot stack operations

Prompts and menus to help beginners

Coexistent graphics and stack/edit display memory

Graphics display can be larger than physical display,

with hardware scrolling

2-dimensional graphical entry and display of algebraic expressions

Spreadsheet-like matrix editor

Redefinable keyboard

Real-time clock & date display

ECMA94 character set, 3 display font sizes

Appointment and control alarms

Plotting features:

Eight types of automatic plots:	function, polar,

conic sections, parametric, "truth", scatter,

bar charts, histograms

Autoscaling option

Interactive plot environment

zoom conrol, recentering, cursor readouts, labelling,

etch-a-sketch, line & circle drawing,

copying and erasing

integration, differentiation, root-finding

Graphics features:

Dot and line drawing

Boxes and Circles

Graphics objects of any size

OR or XOR superposition

Sub-region extraction

Pixel or logical addressing

Unit Management

"Unit objects"--real numbers with physical dimensions

allowed in algebraic expressions

Automatic unit conversion in unit object operations

arithmetic, integration, plotting, solving

Interactive unit menus for simple conversions

I/O

Kermit protocol built in for calc-to-calc (IR) or

calc-to-calc (serial)

Server mode

Low level serial commands

Print to serial or IR

Plugin Management

Merged or independent RAM (max RAM 288K)

Automatic menu of plugin applications and commands

Archiving and restoring (also via I/O)

Mathematics

Common transcendentals on hard keyboard

New:

Symbolic integration (pattern matching)

Summation function

Programmable pattern match/replace

"Where" function for sticky local variable bindings

Generation of user-defined functions from

algebraic assignments

decimal to rational fraction conversion

New supported object types:

Graphics (replaces HP28 graphic strings)

Tagged (for labeling objects)

Unit

XLIB Name (plug-in command)

Directory

Library (plug-in application/commands)

Backup	(Independent RAM variable)

Library Data (for use by plug-in applications)

New programming features:

CASE structure

Programmable OFF and CONT

Autostart for single step; step through subroutines

Memory size and checksum for objects

INPUT command (programmable command line)

PROMPT (HALT with display string)

Wait-for-key with timeout

Custom error generation

One-key typing aids for entire structures

Extended custom and solver menu control

Increment and decrement (variable) commands

Storage arithmetic for all object types

Global variable commands extended to local variables

Decompile stack objects into command line

Wlodek Mier-Jedrzejowicz