HP48SX Modeming

In general, one can keep a modem well-fed and happy by connecting

pin 6 (data set ready) to pins 4 (ready to send) and 20 (data terminal

ready).  When the modem is on, it should assert DSR, but it may wait

for RTS or DTR to be asserted before doing anything else.

Upon satisfying pins 4 and 20, connecting pins 2, 3, and 7 to the

calculator will suffice to use the modem.  To avoid (or create)

confusion, I will letter the pins on the calculator as A, B, C, and D,

starting with the pin closest to the IR led/detector pair.  Then

connect A to 7, B to 3, and C to 2 (and if you want, connect D to

the cable shield, if any).  The pin 2 and 3 connections are opposite

what you would use to connect to a computer, since computer serial

ports are wired like terminals ("data terminals"), not modems ("data

sets").

Mark Adler

madler@tybalt.caltech.edu