Subject: Re: HP48SX Plug in Card Pinouts

>Does anyone have a pinout for the Plug in cards for the

>'48 (IE the Equation card, 41 Emulator)?

I just happen to have a copy here.  The document is titled "HP48SX

Expansion Port - Technical Guide and Specification", dated November

12, 1990.

When looking at the card contacts, pin one is on the right:

PIN #	Signal  I/O	Description

-----------------------------------------------------------------

1	VDD	PI	Power

2	VBB	O	Card battery check.  If VBB is low then 48SX

will signal 'low battery'.

3	A0	I	Pins 3-19, 31, and 32 are the address lines.  (Pin

32 is inverted.)  My guess is the 20th address

line is actually the Card Enable (pin 21)

input in disguise.

4	A1	I

5	A2	I

6	A3	I

7	A4	I

8	A5	I

9 	A6	I

10	A7	I

11	A8	I

12	A9	I

13	A10	I

14 	A11	I

15	A12	I

16	A13	I

17	A14	I

18	A15	I

19	A16	I

20	NWE	I	Not write enable

21	CE	I	Card Enable

22	NOE	I	Not output enable

23	D0	I/O	Pins 23-30 are the eight data lines.

24	D1	I/O

25	D2	I/O

26	D3	I/O

27	D4	I/O

28	D5	I/O

29	D6	I/O

30	D7	I/O

31	A17	I

32	NA18	I	A18 is inverted (negative logic)

33	XSCL	I	Display data clock

34	LP	I	Display data horizontal sync

35	LD[0]	I	Display data

36	LD[1]	I	Display data

37	CDT	O	Card detect/type, must be tied high for writes

to operate.

38	NC		No connection.

39	NC

40	GND	PI	Ground

The CDT pin (37) helps the 48 determine which of the following four valid

states applies:

1) Empty - The CDT pin is open (Hi Z).

2) ROM - The CDT pin is low.  All write operations are disabled.

3) RAM - The CDT pin is high and the card passes the RAM size test.

4) UNKNOWN DEVICE - The CDT pin is high and the card fails the RAM

size test.  User-level reads and writes are disabled (as if nothing

were present) but machine level reads and writes work.

Lines 33-36 are responsible for those overhead projector thingys

(that's the technical term) which frame grab the 48's display and

uses the bitmap for its own purposes.  The data is two serial bit

streams in sync with the shift clock XSCL.  A pulse on the LP signal

provides horizontal sync information.  The 131 columns of display data

for a particular row are contained in the 64 shifts before each LP

pulse and the 2 shifts immediately following the LP.  The left-most

column of data is output first.  There is no vertical sync.

*Whew!*

-Gary Friedman