HP48sx serial pinout

The following table shows how the 48SX 4 pin, the cable's 9 pin

connector and the 25 pin adapter provided with the MSDOS PC link

package are wired.

HP48SX                  IBM 9 PIN               IBM 25 PIN

(From the outside->in)

SHIELD-------------------1 SHIELD----------------  SHIELD

TX (Output)<-------------2 RX (Input)------------3 RX (Input)

RX (Input)-------------->3 TX (Output)---------->2 TX (Output)

SGND---------------------5 GND-------------------7 GND

Preston

Here are the pinouts for the 48SX serial port:

4 3 2 1

/------------|-|-|-|----\

|   48SX                  |    Pin   Function    Standard RS232 pin eqv.

|   ___________________   |                       (25 pin connector)

|  |                   |  |     1  Chassis Ground       1

|  | 4:         "THIS" |  |     2  Transmit Data        2

|  | 3:           "IS" |  |     3  Receive Data         3

|  | 2:          "THE" |  |     4  Signal Ground        7

|  | 1:       "SCREEN" |  |

|   -------------------   |

|                         |

| [A] [B] [C] [D] [E] [F] |

| MTH PRG CST VAR  ^  NXT |

|     .  .  .  .  .  .    |

If you're making a cable, just make a standard null modem cable.

(reverse RX and TX)

48SX pin           RS232 pin

CHG   1  - - - - - - - -  1  CHG     Chassis ground does not

actually have to be connected.

TX   2  ------\ /------  2  TX

X

RX   3  ------/ \------  3  RX

GND   4  ---------------  7  GND

THORNTON JOHAN A