WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVERS

FOR USE WITH NOVELL ADVANCED NETWARE v2.0a

RELEASE INFORMATION

Version 2.06        7 May 1988

This version of Western Digital's driver for Novell Advanced

NetWare v2.0a has been released and fully tested by Western

Digital, but formal certification by Novell has not yet been

completed.

VERSION HISTORY

-- Version 2.06 (Current Version) --

Two problems that were present in driver versions 2.05 and 2.06a

have been corrected:

o A server or workstation would occasionally stop

functioning when the network was under extremely

heavy load.

o Certain application programs that used the Novell

NETBIOS interface would not operate properly.

-- Version 2.06a (Field Test Version:  March, 1988) --

This version identifies itself on the screen as "2.A6".

There were no known problems with version 2.05, but the following

improvements have been made:

o The RAM base address in the workstation shell is

changed from C4000 (hex) to D0000 to minimize conflicts

with other adapters (especially VGA, EGA).

o The driver operates with LAN adapter boards that have

32 Kbyte RAM buffers as well as the original adapter

boards with 8 Kbyte RAM buffers.

o Workstation transmit speeds are improved.

--Version 2.05 (September, 1987)--

Modifications have been made that correct throughput degredation

encountered under heavy loading.

--Version 2.02--

This release of the WD8003 driver contains a fix for a problem

encountered in earlier versions that resulted in a workstation

being logged off the server after 15 minutes of no interaction

with the server.

TECHNICAL SUPPORT

Please direct any questions to your dealer or to:

Western Digital Corporation

Communications Products Technical Support

(800) 638-5323

or

(714) 474-2033  ext. 4900

#####################