From almes@rice.edu Mon Mar 25 09:28:07 1991

Received: Mon, 25 Mar 91 09:28:04 EST by rivendell.merit.edu (5.51/1.6)

Return-Path: <almes@rice.edu>

Received: from rice.edu by merit.edu (5.65/1123-1.0)

id AA20725; Mon, 25 Mar 91 09:25:59 -0500

Received: from vesuvius.rice.edu by rice.edu (AA28299); Mon, 25 Mar 91 08:25:28 CST

Received: by vesuvius.rice.edu (AA25679); Mon, 25 Mar 91 08:25:54 CST

Date: Mon, 25 Mar 91 08:25:54 CST

From: almes@rice.edu (Guy Almes)

Message-Id: <9103251425.AA25679@vesuvius.rice.edu>

To: skh@merit.edu

Subject: Re:  ES-IS and CLNP tutorial's ready for anonymous ftp

Status: RO

Sue:

I've formatted esis.intro.doc to have more uniform margins.

Cheers,

-- Guy

A tutorial on the ES-IS protocol

by: Robert Hagens

University of Wisconsin

hagens@cs.wisc.edu

Introduction

On a typical OSI subnetwork, computers (systems) generally fall

into two classes: End Systems (ES) and Intermediate Systems (IS).

This classification is made from a layer point of view: both

systems contain the OSI layers 1 (physical) through 3 (network).

However, an ES also contains OSI layers 4 - 7. An IS does not. ****

[**** Footnote: The distinction between ESs and ISs is gray when

real systems are considered. For example, although an IS may

primarily route packets, it may also have layers 4 - 7 present in

order to operate network management protocols.]

The basic idea is that ESs are systems that run applications that

send and receive packets of information and ISs are systems that

route the packets from source to destination. This general concept

can be readily subdivided into two cases. The first case occurs

when the source and destination ES are connected to the same

subnetwork (see Figure 1, host A communicating with host B). Since

A and B are on the same subnetwork, packets may flow directly

between them without the intervention of an IS.  The second case

occurs when the source and destination ES are not connected to the

same subnetwork (Figure 1, host A communicating with host C). It is

important to note that in the second case, packets flowing from

A to C must be fowarded from one subnetwork to the other by IS D.

The End System to Intermediate System routing exchange protocol

(ISO 9542) is designed to aid the two types of communication

between ESs and ISs described above. The protocol provides a means

for ESs and ISs on a subnetwork to learn of each other's existence.

This process, known as *configuration*, allows ESs and ISs to

dynamically determine the existence of each other. The dynamic

quality of the protocol eliminates the need for manual

configuration of the different types of systems on a subnetwork.

In addition to promoting the exchange of configuration information,

the protocol provides a means for route *redirection* information

to be sent from an IS to an ES. This allows an IS to inform an ES

of a potentially better route towards a destination.

As explained below, the ES-IS protocol distinguishes different

types of subnetworks.  The operation of the protocol is quite

different on each type of subnetwork.  The difference between the

broadcast subnetwork (i.e., 802.3) version and the general topology

subnetwork (i.e., X.25) version is so large that there are two

distinct versions of the protocol defined.  This article will not

describe the operation of ES-IS over X.25. Rather, it will concern

itself with the operation of the ES-IS protocol when it is used in

conjunction with the OSI connectionless-mode network protocol

(ISO 8473).

Configuration Information

Configuration information is transmitted by exchanging two types of

packets: the End System Hello (ESH) packet is generated by ESs and

transmitted to every IS on the subnetwork.  The Intermediate System

Hello (ISH) packet is generated by ISs and transmitted to every ES

on the subnetwork. The primary purpose of the hello packets is to

convey the subnetwork and network layer address of the system that

transmitted the packet. The ESH and the ISH packets are generated

by a system when the system's configuration timer expires.  When

the timer expires, a hello packet is transmitted and the timer

reset to its initial value. The initial value of the timer is

maintained locally by the system.  The interval between

transmission of hello packets may be modified by adjusting this

timer.

The configuration interval must be coordinated with the value of

the holding timer that is transmitted with the hello packet.  The

holding timer indicates to the receiving system the number of

seconds that the configuration information should be kept.  When

the holding timer expires, the associated configuration information

must be deleted. If the holding timer is smaller than the

configuration timer, systems may "disappear" from the subnetwork

for periods of time.

Redirection Information

Redirection information is transmitted via a RD PDU. These PDUs are

only generated by ISs. The purpose of an RD PDU is to inform an ES

of a potentially better route towards its destination.

During the forwarding process, an IS will determine the next system

to which the packet should be sent. If an IS can determine that

this "next hop" system can be reached directly, without the IS's

intervention, then a RD PDU will be sent from the IS to the

originator of the packet.

The RD PDU may redirect an ES to either the destination ES or to a

different IS. The lifetime of the information conveyed by the RD

PDU is limited by a holding timer, just like the hello PDUs.

Normally, the redirection information delivered in an RD PDU is

specific to a single destination address.  However, as an option,

it is possible for an RD PDU to indicate that the redirection

information should be applied to a larger number of destination

addresses.  This is accomplished by including an address mask which

indicates the larger population of destination addresses.

Subnetwork Topology Considerations

The ES-IS protocol identifies 3 common subnetwork topologies: a

point-to-point subnetwork, a broadcast subnetwork, and a general

topology subnetwork. The point-to-point subnetwork supports

exactly two systems. The broadcast and general topology subnetworks

support an arbitrary number of systems. The crucial difference

between the latter two types is the cost of sending a packet to a

large subset of the subnetwork population. If the cost of such an

*n-way* transmission scales directly with the subset size, then the

subnetwork topology is considered general (a common example is an

X.25 subnetwork).  If the cost of an *n-way* transmission is close

to the cost of a transmission to a single system, then the topology

is broadcast (a common example is 802.3).  This difference affects

the transmission of configuration information.

Ideally, configuration information is sent to many systems on the

subnetwork simultaneously.  When operating on a broadcast

subnetwork, ESHs are sent to a special multicast address "all

intermediate systems".  ISHs are sent to a special multicast

address "all end systems".  This multicast address may be realized

physically as a true multicast or as a broadcast to every system.

When operating on a general topology subnetwork, configuration

information is generally not transmitted due to the high cost of a

multicast transmission.

Network and Subnetwork Addresses

The addresses described so far have been network layer addresses.

There are actually two different kinds of addresses that the ES-IS

protocol conveys: network layer addresses and subnetwork addresses.

Network layer addresses identify either the interface between layer

3 and layer 4 in an OSI ES (called a Network Service Access Point,

NSAP), or the network layer entity itself in an OSI IS (called a

Network Entity Title, NET).  NSAP addresses and NETs are part of a

name space which spans all OSI networks.

These network layer addresses can be contrasted with a subnetwork

point of attachment address (SNPA address).  The SNPA is the point

where an ES or IS is physically attached to a subnetwork.  The SNPA

address is used by the subnetwork to uniquely identify each system

attached to the subnetwork.  For example, in an 802.3 subnetwork,

the SNPA address corresponds to the 48 bit MAC address.

When an ES or IS transmits a packet, it is necessary to determine

the destination SNPA address.  This is generally accomplished by

associating a NSAP address or NET with a specific SNPA address.

Part of the configuration information transmitted by the ES-IS

protocol is this NSAP address (or NET) to SNPA address mapping.

Operational Scenarios

It is often useful to consider real-life scenarios in order to get

an intuitive idea of how a protocol operates. In the following

examples, please refer to Figure 1. Furthermore, assume that the

subnetworks in Figure 1 are broadcast subnetworks and that capital

letters are NSAP & NET addresses, whereas lower letters are

corresponding SNPA addresses.

Example 1:

ES A wishes to send a packet to ES C.  ES A knows about IS D

because it receives periodic ISH packets from IS D. Since D is an

IS, and ISs should know how to route packets toward any

destination, ES A will send its packet directly to IS D.  In

addition to knowing that IS D is an intermediate system, ES A also

knows the SNPA for IS D.  This information was taken from the ISH

received from IS D.  Result: ES A sends the packet to IS D at SNPA

d and IS D forwards the packet to ES C.

Example 2a:

ES A wishes to send a packet to ES B.  As in example 1, ES A knows

about IS D due to receipt of ISH packets from D. Therefore, ES A

will send its packet to IS D at SNPA d. IS D has knowledge about

both ES A and ES B because it receives ESH packets from both.  When

IS D receives the data packet from ES A, it computes that ES B is

reachable via the same subnetwork as ES A, and forwards the data

packet to ES B.  In addition, IS D computes that ES A could have

sent the packet directly to ES B.  IS D notifies ES A of this fact

by sending a RD PDU to ES A.  This RD PDU indicates that ES B may

be reached by sending packets directly to SNPA b.  Further packets

sent by ES A to ES B are directed toward ES B.

Example 2b:

This is the same scenario as 2a, but with a twist.  Assume that ES

A's routing information states that ES B is on the *same

subnetwork* as ES A. The only problem is that ES A does not know

the SNPA for ES B.  ES A has two choices.  As in 2a, it could send

the packet to IS D and receive a redirect.  Or, as an optimization,

ES A could listen to ESH PDUs sent on the subnetwork.  If ES A

eavesdropped, it would know the SNPA of B. ES A could then transmit

the packet directly to ES B at SNPA b.

Example 3:

Similar to scenario 2b, assume that ES A wishes to send to ES B.

In addition, ES A knows that ES B is on the same subnetwork.

Furthermore, assume that ES A does not eavesdrop and IS D does not

exist.  How can ES A determine the SNPA of ES B? In this situation,

ES A will invoke a function of the ES-IS protocol called query

configuration.  To do this, ES A will *broadcast* its *data* packet

to all ESs on the subnetwork.  When ES B receives this broadcasted

packet, it will note: 1) that the packet is a data packet sent to a

broadcast SNPA address; 2) that the destination NSAP address of the

broadcasted packet is ES B.  In this case, ES B will generate an

ESH packet and send it directly to ES A.  This packet will inform

ES A of the SNPA of ES B.

A commonly asked question, when considering example 3, is "If ES A

does not eavesdrop ESH packets, how can it receive the ESH sent

directly by ES B ?" The key to the answer is SNPA addresses.  When

an ES eavesdrops, it *enables* the reception of packets addressed

to the multicast address "all intermediate systems" (because end

systems send their ESH packets to the multicast address "all

intermediate systems").   When ES B sends an ESH to ES A, it sends

it directly to SNPA a. ES A will receive this packet even if the

"all intermediate systems" multicast address is not enabled.

PDU formats

Figure 2 and 3 show the format of the ESH and ISH packets.  The

format of these packets is very similar: they both have a fixed

header which is almost identical to the fixed part of an ISO 8473

(CLNP) packet.  Following the fixed part, each hello packet has

space to store a network address.  Note that an ESH has space for

more than 1 NSAP address, (since an ES may support several NSAPs)

whereas an ISH has space for only 1 NET.

The discerning reader will note that there is no place in either

the ESH or ISH packet to store an SNPA address.  How can this

information be transmitted from system to system?  The answer lies

in the subnetwork service definition. By fiat, when a packet is

delivered by a subnetwork service, the delivery will include not

only the data packet but also the subnetwork source and destination

address.  Thus, when processing an ESH, the SNPA associated with

the ES is taken from the *source SNPA address* which accompanies

the delivery of the ESH.

Comparison to DoD Protocols

The ES-IS routing exchange protocol does not have an exact

counterpart in the DoD protocol suite.  The configuration

information exchange function is similar to the function of the

Address Resolution Protocol (ARP, RFC 826).  However, ARP was

designed around an "on demand" approach, that is, query when the

information is needed.  This is in contrast to ES-IS where the

design is based around continual update messages (hello packets).

It should be noted that the query configuration function (shown in

example 3) is somewhat analogous to the ARP "on demand" approach.

However, unlike ARP, query configuration requires that the entire

data packet, rather than an ARP request, be broadcast on the

subnetwork.

The counterpart to the ES-IS redirection information function in

the DoD protocol suite is the redirect message defined by the ICMP

protocol (RFC 792). These two approaches to redirect differ in that

ES-IS allows the target of a redirect to be either an ES or IS

whereas an ICMP redirect can only refer to a gateway.

=============================================================================

Figure 1

+-------+

|       |

| ES A  |------|

|       |      |

+-------+      |

|

|   +-------+   |

|   |       |   |

|---| IS D  |---|

|   |       |   |

|   +-------+   |     +-------+

|               |     |       |

+-------+      |               |-----| ES C  |

|       |      |                     |       |

| ES B  |------|                     +-------+

|       |

+-------+

=============================================================================

Figure 2

The ESH Packet Format

+-------------------------------------------------------------+

|               Network Layer Protocol Identifier

+-------------------------------------------------------------+

|				Length Indicator

+-------------------------------------------------------------+

|				Version/Protocol Id extension

+-------------------------------------------------------------+

|				Reserved

+-------------------------------------------------------------+

|  0  | 0 | 0 |   Type of Packet

+-------------------------------------------------------------+

|               Holding

|               Time

+-------------------------------------------------------------+

|               Checksum

|

+-------------------------------------------------------------+

|               Number of Source Addresses

+-------------------------------------------------------------+

|               Source Address Length

+-------------------------------------------------------------+

/

Source Address

/

|

+-------------------------------------------------------------+

.

.

+-------------------------------------------------------------+

=============================================================================

Figure 3

The ISH Packet Format

+-------------------------------------------------------------+

|               Network Layer Protocol Identifier

+-------------------------------------------------------------+

|				Length Indicator

+-------------------------------------------------------------+

|				Version/Protocol Id extension

+-------------------------------------------------------------+

|				Reserved

+-------------------------------------------------------------+

|  0  | 0 | 0 |   Type of Packet

+-------------------------------------------------------------+

|               Holding

|               Time

+-------------------------------------------------------------+

|               Checksum

|

+-------------------------------------------------------------+

|               Network Entity Title Length

+-------------------------------------------------------------+

/

Network Entity Title

/

|

+-------------------------------------------------------------+