Benchmarking Storage Systems

White Paper

by

Ohad I. Unna

DynaTek Automation Systems Inc.

September, 1992

Benchmarking Storage Systems

Abstract

Existing  disk  benchmarking  programs  fall  short  in  measuring  the

performance  of the  storage  subsystem as  a whole,  in  the sense  of

missing  the forest  for the  trees. In  order to  obtain a  meaningful

performance  index of a  storage subsystem that  is congruent with  the

user-perceived  usefulness of that system,  a benchmark has to  measure

the  performance in an  application-level manner, simulating  real-life

computing environments.  A method for constructing such a  benchmark is

presented in this paper.

Forward

or

Who needs another disk benchmarking program?

It is  said that if you have a watch  you know what time it is,  but if

you  have two --  you are never  sure. If  you have half  a dozen  disk

benchmarking  utilities,  you  probably  have  six  different  sets  of

results,  and there's  a good  chance none  of these  results bear  any

resemblance to the way your system performs under its normal workload.

Back in the old days (1982-3) things were  pretty simple - PC hard disk

drives were invariably spinning at 3600  revolutions per minute, almost

always had 17 sectors per  track, and built-in bad-sector remapping and

multi-megabyte cache  were practically unheard of in  the microcomputer

world. When the IDE and SCSI disk interfaces  were introduced to the PC

arena,  things  got  really complicated;  disk  drive  vendors  started

employing  sector-mapping schemes in order  to avoid the  1024-cylinder

restriction, controllers were enhanced with a  large onboard cache, and

on-the-fly disk compression adapters and software  become more and more

popular, rendering most of the old disk benchmarking software useless.

If  you  are  not familiar  with  terms such  as  "sector-mapping"  and

"compression  adapters," don't worry. As  a user, you shouldn't  really

care  whether your  drive has  "skewed-track interleave"  or "low  BIOS

overhead latency," as long as you get good performance out of it.

Application Level Benchmarks

or

Why "your mileage may vary"

Have you  ever been tempted to buy  one of those new "effective  access

time: 0.5  milliseconds" disk drives?  Look before you  leap: obtaining

such  a  figure  for  a   current  technology  drive  means  the  drive

manufacturer is  using a less-than-adequate disk benchmarking  program.

Such a program can easily be fooled by  the presence of a small onboard

cache;  by continuously reading  and writing data to  the same area  of

the disk, the benchmark merely updates  the RAM on the disk controller,

while  very little disk activity  actually takes place. Updating a  few

Page 2

kilobytes of  RAM in one half  of a millisecond isn't that  impressive,

and  this will become  evident as soon as  you start running  real-life

applications.  If  you are  using  a software  disk  cache, as  is  now

standard  on  most  operating  systems,  you  will  hardly  notice  the

presence  of an  additional 256KB  controller-based  cache, unless  you

have a nonstandard configuration or device.

Actual  storage   system  performance  depends  on  an   assortment  of

parameters:  bus bandwidth, cache  strategy, scatter-gather  algorithm,

cluster  size  and  fragmentation,  to   name  a  few.  None  of  these

parameters  are  influenced  by the  physical  characteristics  of  the

specific hard drive  used, and each of them has more impact  on overall

storage system performance than the proverbial "average seek time."

The  more advanced  disk  benchmarking programs  try to  eliminate  the

effects of operating system level  parameters by communicating directly

to the  disk. Unfortunately, results obtained in this way are  about as

useful as knowing the top RPM of a car engine when running in neutral.

Two cases  present even vaguer prospects: networks and  disk-arrays. In

both  cases the  same  hardware can  easily be  configured  to work  in

different modes, producing a vast range  of performance. With networks,

network  topology may  be redefined,  and server  configuration may  be

altered, resulting  in an overall system throughput that has  little to

do  with the actual  performance of the  drives themselves. The  second

special  case  is  disk-arrays  configured  as   a  RAID  cluster;  the

performance of the array as a unit can  be as high as twice or more the

performance  of  a  single  drive  or as  low  as  a  fraction  of  the

performance of a single drive.

The  experienced  user will  try  to estimate  the importance  of  each

performance  feature of  the storage  system, weigh  them against  each

other, and make a decision based more on intuition than on hard facts.

Eventually, the  user will run the ultimate benchmark: a test  drive of

the system  with their favorite application. As unscientific as  it may

seem,  the results of  this test are  more meaningful  than any of  the

standard benchmarks.

The  application level benchmarking  approach is  defined as testing  a

system in a manner that resembles  actual usage as closely as possible.

This  seems  to be  the  most consistent  and  useful approach  to  the

problem of measuring storage system performance.

Page 3

Shortcomings of Existing Benchmarks

or

Why results are only as good as the measurement tool

*    Cache

Since  most existing disk benchmarking  programs were designed  to

give  results within  a few  seconds of  activation, they  usually

perform  their tests by  moving very small  amounts of data.  With

modern  high-end storage devices  you could  have anywhere from  a

few  kilobytes to 64  megabytes or more of  cache between the  CPU

and  the storage system. If  a benchmarking program uses  standard

file  accesses, most or  all of the data  wouldn't even reach  the

drive  by  the  time  the measurement  period  is  over,  yielding

ludicrous  results, such as  a transfer rate  of 20 megabytes  per

second.  If  the cache  is  embedded within  the hardware,  as  is

typical  of high-end  SCSI  adapters, it  would present  the  same

problem for benchmarking software which uses  direct access to the

drive.  In  order to  measure  the true  effect  of a  cache,  the

benchmark  should move vast amounts  of data during the test,  use

huge  files for disk access,  and generally behave like a  typical

application.  A  good  benchmark  shouldn't  eliminate  the  cache

effect, but rather measure the true  benefit an actual application

will gain by the presence of that cache.

*    Compression

Disk  compression hardware and  utilities are  new players in  the

small-computer  arena. Before data is  written to the drive it  is

compressed  either by a software  device driver or by a  dedicated

hardware adapter.  During reads, data is uncompressed back  to its

original  form. Best  performance  is achieved  when the  data  is

highly  regular,  such  as  is common  in  large  spreadsheets  or

databases.  However,  as the  ratio  between  CPU speed  and  disk

throughput  grows,  new  applications  will   have  built-in  data

compression,  especially those  dealing  with highly  compressible

data, such as video and music.  When this happens, the benefits of

disk-based compression will diminish. A  typical disk benchmarking

program  uses either fixed  data blocks  or whatever garbage  data

happens  to be in  memory at the  time. In  either case, the  data

tends to  be highly compressible, resulting in  exceptionally high

test  scores for transfer rate.  In order to obtain true  results,

the benchmark  should use data patterns which resemble  the actual

data as  closely as possible. Since this is  virtually impossible,

the next best thing is to select a  reference pattern which can be

easily  defined and whose  maximum compression  ratio is known  in

advance. The  only data pattern that meets both of  these criteria

is an  incompressible pattern. Information theory assures  us that

a  suitable pseudorandom  sequence has  zero  redundancy and  thus

cannot  be  compressed.  As  far  as   the  compression  logic  is

concerned,   a  pseudorandom   sequence  seems   identical  to   a

pre-compressed sequence.

Page 4

*    Multi-threading

A  drive  with  a  long  seek  time  may  perform  very  well  for

sequential  accesses,  but  it will  cease  doing so  as  soon  as

several  threads are  running concurrently,  even  if all  threads

access  the same file, and  all do so sequentially. The  necessity

to  switch tasks results  in multiple seeks up  to a point  where,

with a  sufficiently large number of threads, the behavior  of the

drive  will resemble totally  random access.  The extent to  which

this effect  will degrade performance cannot be  computed a-priori

using  only  the  knowledge  of average  random  access  time  and

transfer rate,  and thus has to be  tested per se. A small  number

(3-6)  of concurrent threads  is usually enough  to feel the  full

effect of sequential access degradation.

*    Sequential vs. Random

Existing  benchmarks generally  divide the type  of disk  accesses

into  two   categories:  sequential  access  and  random   access.

However, in real life, only  very few of the applications actually

behave  this way. A modern  disk controller with a high  bandwidth

and  deferred-write  caching  can almost  totally  eliminate  seek

latency  for  applications  performing only  a  small  portion  of

random  accesses. The same drive,  when used with a  write-through

cache,  might perform almost as  slowly as fully random access.  A

good benchmark should present the system  with the correct mixture

of sequential and random accesses in order to get valid results.

*    Combining reads and writes

Much  like  combining sequential  and  random accesses,  again,  a

mixture of read and write  requests may yield results which differ

substantially  form the  'expected' weighed  average  of the  two.

With  the  proper logic  programmed  in  the drive  controller  or

device   driver,  and   the   appropriate  scatter/gather   method

implemented in  the operating system, read and write  requests can

be overlapped from the CPU's  point of view, resulting in superior

throughput.  Some  of  the  more   rudimentary  disk  benchmarking

programs  do not perform  any writes at all  during the test,  and

simply  assume the transfer  rate is the same  as for reads.  This

assumption  falls apart  when RAID  class  devices are  concerned.

Depending  on RAID  level  and architecture,  the time  needed  to

write a block  may differ from the time it takes to read  the same

block  by a factor  of two  or more. An  inherently sluggish  RAID

device  may seem exceptionally  fast when  tested by only  reading

existing  data, or when the  mixture of read and write  operations

does not match the actual application mixture.

Page 5

Defining a Reference Disk Benchmark

or

What's a RAIDmark?

The  DynaTek RAIDmark  utility is  intended to  measure the  integrity,

performance,  and  cost-efficiency  of  any  read/write  random  access

(blocked  I/O) storage subsystem in  terms that are meaningful for  the

layman, and that depict the apparent  usefulness the user gets from the

system. RAIDmark is also intended  to measure some low-level metrics of

the storage  device, which can be utilized for testing  and fine-tuning

such devices throughout the phases of on-site installation.

The  RAIDmark  utility  defines  a  set of  six  standard  tests,  each

simulating a  typical I/O-bounded computer environment. For  each test,

the  tested system receives a  grade proportional to its throughput.  A

unit of 'One RAIDmark' is defined  as the measured throughput (for each

test) of an  IBM PS/2 model 95 with the standard IBM 150 MB  hard disk.

Integrity  is  tested  on  a  go/no-go   basis  using  32-bit  checksum

verification on each record.

*    Low-level performance

The  utility measures raw indexes  of storage device  performance:

average  access time,  read/write transfer  rate, effective  cache

size and cache strategy.

*    High-level performance

The utility tests the performance  of the storage subsystem on the

application level, checking the subsystem's  ability to respond to

harsh real-world data traffic of  typical heavy load environments.

The   utility  simulates  data  flow   of  several  typical   work

environments and  measures system response and throughput  in each

of  the test  cases, giving  a combined  figure of  merit for  the

system's performance in each scenario.

The following scenarios are simulated and tested:

-    Typical  workstation applications  -  multiuser (3-10  users)

environment, many  small- to medium-size file requests,  most

of  which are  read-only accesses  with  less frequent  large

file  read/write  operations  (as in  CAD/CAM  file  save  or

memory swap).

-    Transaction processing  environment - 5-50 users, many  small

record   read  and  update   requests,  reports  and   backup

processes running  in the background, transaction logbook  as

a large sequential file.

-    Video/audio/multimedia  applications - single  or few  users,

huge  sequential  access  files, with  both  read  and  write

requests.

Page 6

-    Database  query  systems -  5-200  users, many  small  record

accesses,  almost  all  of  which   are  read-only;  updates,

reports and backup are executed during system off-line time.

-    Data  logging - up  to several hundreds  of sites, many  very

small  random   write  accesses  (typically  20-100   bytes),

reports and backup are executed during system off-line time.

-    Archiving  -  few  users  with  large  sequential  read/write

accesses.

*    Degraded subsystem performance

When the subject of the tests  is a RAID-class storage system, the

user can  disable one of the subsystem elements, and  then proceed

to measure the degraded performance in this hindered setup.

*    Design constraints and limitations

-    Free disk space

In  order for the  RAIDmark utility to  conduct the tests  it

should be able to create  files of several megabytes each, or

even   larger,  if  caching  effects   are  to  be   measured

accurately.  For best accuracy, at  least 150MB of free  disk

space should be available on the tested drive.

-    Operating system interface

When  using a  multi-processing  environment, all  benchmarks

should be run as a sole  process, otherwise time measurements

will be of little value.

-    Integrity and stability

The  ability of RAID-class devices  to operate with parts  of

the  subsystem  inoperable  (or   altogether  missing)  might

create a  situation in which the  user is not aware that  the

system  is  operating  under  highly  unfavorable  conditions

which adversely  affect the benchmark. Moreover, modern  disk

controllers can  overcome some physical mishaps, such  as bad

sectors, unaligned heads, etc., relying  on firmware logic to

correct   these   problems  at   the  expense   of   degraded

performance.  The  user  should  identify  and  isolate  such

conditions, possibly  by testing for stability over  time and

over different areas of the disk platters.

-    DOS FAT Latency

When  using top-level MS-DOS file  I/O requests, each  random

file  seek requires  the operating  system to  scan the  file

allocation  table (FAT)  from the  location  pointing to  the

beginning  of  the file,  to  the  location pointing  to  the

Page 7

requested cluster.  Although the FAT is usually kept  in RAM,

for  large   files  this  sequential  scanning  process   may

actually take  longer than the physical movement of  the disk

arm.  Some advanced  applications  (database managers,  etc.)

overcome this  delay by keeping several logical  entry points

to  the file ("file handles")  scattered at different  points

in the  file. Access is then achieved using the  nearest file

handle, reducing  FAT latency by a factor of 10 to  20. Other

applications,  such  as  MS-Windows  3.0/3.1  Virtual  Memory

Manager, implement their own file system  to achieve zero FAT

latency.  More modern operating systems,  such as OS/2  HPFS,

achieve  the same favorable  results using advanced  built-in

random seek methods.

For  the  sake of  being  as  general as  possible,  RAIDmark

measures  the  exact FAT  latency  on  the tested  system  by

comparing the average access times for  clusters located in a

limited  region near the  beginning of the  file to those  of

clusters  located in a  similar size region  near the end  of

the file.  The average FAT latency is then deducted  from all

random  access test times, giving  the theoretical limit  for

an 'optimally tuned' application.

*    Test files

Test  files are  homogeneous streams of  random values,  logically

accessed  as an array of  32-bit unsigned integers. The stream  is

logically   subdivided    into   records   of   16    double-words

(long-words), each containing an embedded checksum.

Each  record in a  test file contains 16  double-words of 32  bits

each.   Fifteen  of   these  double-words  are   generated  by   a

pseudorandom  number generator.  The 16th value  is calculated  in

such  a way  that the  sum of  all words  in the  record plus  the

offset  of the record  from the beginning of  the file (in  bytes)

equals  zero (modulo 2^32). This  file structure enables the  user

to  create files  that  are incompressible  by nature,  but  still

allows  the  user to  verify  the validity  of  the data  with  an

error-detection  probability  of  near  certainty.  However,  this

method  requires one  multiplication and  two addition  operations

per  32-bit integer during  file creation,  but only one  addition

per double-word during read access.

To  prevent  meaningless  results  obtained  from  slow  processor

machines  with concurrent disk access,  a 32KB block of data  (512

records  of 64  bytes) is  prepared in advance,  and records  from

this  block are written  to the disk at  file creation, with  only

one word  in each 16-word record changed for each  write operation

to account for different file offsets.  This will give even a slow

CPU ample time to complete  data preparation before the next write

operation is due.

Page 8

Theoretically, data  created in the method described above  can be

compressed. However,  there is currently no compression  algorithm

capable of detecting the 256Kbit-long  patterns created using this

method.

A question arises as to the  extent to which this random-data file

serves as a  good test case, since in reality most data  files are

compressible by  30%-60% or, in some homogeneous  data acquisition

applications, up to 80%; however,  using compressible data exposes

us  to the vast range  of compression algorithms, each having  its

own  peculiarities  regarding specific  data  patterns,  sometimes

resulting  in a  twofold factor  in throughput  for some  specific

instance. Hence, the approach taken in  RAIDmark is the worst-case

compression scenario, in which all algorithms  behave in a similar

fashion, namely, no compression occurs.

In  a  future  version  the  user  may  be  given  the  option  of

generating semi-random  ("pink noise") data files, permitting  the

compression mechanism to affect results.

-    Test File Creation

If  any of  the  disk tests  is requested,  a  128MB file  is

created.  This process  is timed as  accurately as  possible,

both  the initial  file open  operation, and  the writing  of

data on  the disk. Measurement is done separately  for chunks

of  data of  varying sizes.  The  first two  chunks are  16KB

long, the third is 32KB long, the fourth  -- 64KB, and so on,

with the  8th chunk being 1MB long. The remaining  1MB chunks

are written to bring the total to the final size.

The  128MB file  size was  chosen  arbitrarily, being  larger

than  today's biggest disk  cache units,  but less than  half

the  size of modern  high capacity disks.  This is a  default

value,  intended  to  become  the  'RAIDmark  standard,'  but

values ranging  from 1MB to 1GB are implemented  and selected

using  a command line  option. Test  files smaller than  16MB

might  produce  inaccurate  results, and  the  user  will  be

warned in  case such a selection was made. Using  the default

file  size,  each  separate  test will  take  between  a  few

seconds  and  two minutes  to complete,  permitting  accurate

measurement  without seizing  the system  for an  intolerable

duration. There  are 14 separate tests in a  full test-suite,

so with all the testing overhead,  running the full benchmark

may take the better part of an hour.

-    Determine deferred-write cache size

During  the  file  creation phase,  effective  transfer  rate

throughput  is   measured  for  increasing  data  chunks   as

described  above.  If the  system contains  a  deferred-write

Page 9

cache, the  first few chunks will  take a very short time  to

be written,  usually exhibiting a throughput in excess  of 10

megabytes per second. At some  point, throughput will decline

sharply.  This point in  the data flow marks  the end of  the

posted write  cache. With systems employing dynamic  caching,

the value  obtained for the cache size may be  arbitrary, but

if  the test is  run on  an otherwise idle  system, the  size

detected  will be  close  to the  maximum possible  for  that

system.

Note  that in  order for  this  test to  run accurately,  the

system should be given enough  time to "rest" before the test

commences  in order to  enable previously  cached data to  be

flushed  to disk  by the  cache logic. The  user should  wait

until all previous disk activity  ceases before starting this

test.

-    Determine raw transfer rate for write

After  obtaining an  upper bound for  the posted-write  cache

size,   the  actual  non-cached   transfer  rate  for   write

operations is  measured by first writing data to  a different

location  on the  drive. The  amount of  data written  should

exceed  the size  of the computed  deferred-write cache.  The

benchmark then compares total write times  for data chunks of

different  sizes,  specifically  8KB  and   32KB  blocks.  By

subtracting the  two, all fixed-overhead delays are  canceled

out, and  the net result is the  time it takes to write  24KB

of data.

This figure for transfer rate  will usually be lower than the

nominal  figure quoted by the  drive manufacturer, since  the

measurement  involves drive head  movements (seek and  search

operations),   operating  system  and   BIOS  overhead,   and

cache-miss delays.  However, the figure obtained this  way is

a  more meaningful  measure of  subsystem performance,  since

this  is   the  actual  throughput  as  experienced   by  the

application  and   the  end  user.  Delays  created   by  the

benchmarking  process utility itself  are taken into  account

and removed from the delay measurement.

During  the "post-cache" phase  of writing,  each of the  1MB

chunks  of data should  take approximately  the same time  to

complete.  If the  variability is higher  than expected,  the

user  will  be  warned  of  the  system's  instability.  This

situation  may arise when  other processes  in the system  or

the  network are  active, when  the  cache strategy  involves

some  exotic algorithm, when the  disk is highly  fragmented,

when many bad sectors are  scattered on the disk's test area,

or when  a "flaky" drive causes retried attempts for  some of

the  operations. The  user has  the choice  of repeating  the

Page 10

test,   or   else   referring   to   the   user   guide   for

troubleshooting advice.

-    Determine raw transfer rate for read

Transfer  rate  for sequential  read  access is  measured  by

comparing  total  read times  for  data chunks  of  different

sizes, specifically  8KB and 32KB blocks. By  subtracting the

two, all fixed-overhead delays are canceled  out, and the net

result is the time it takes to read 24KB of data.

-    Main test executive

The main test executive procedure is  a generic routine whose

input parameters  define the real-life environment each  test

case is supposed to simulate. Parameters include:

- record size

- distribution of access type

- number of users or processing threads

The  procedure determines the number  of times the tests  are

to  be repeated as  a function  of the duration  of a  single

test.  The rule is  that, by default,  each test case  should

take from  a few seconds to around two minutes. If  the total

desired  testing time of the  complete benchmark is given  on

the  command  line,  the  duration of  each  single  test  is

adjusted  accordingly. This time  does not include  benchmark

overhead, such as file creation  and cache overflowing, hence

total benchmark duration may be  considerably longer than the

command line parameter.

-    Build random block

This routine creates a 32KB  block in memory for writing onto

the  disk during the file  creation phase. The format of  the

block is  described above. The algorithm used  for generating

random numbers  is the Linear-Congruential method  (described

in  D.E. Knuth, "The  Art of  Computer Programming" Vol.  2).

The  location used for inserting  the checksum word for  each

record is the location (first  four bytes) of each record. In

order  to account for different  file offsets, this  location

is updated for each instance of writing the 32KB block.

Page 11

Conclusion

In  order to help  the user select a  storage system and  configuration

that will  match their computing needs, a standard  reference benchmark

for  measuring storage  system performance has  been defined.  Existing

disk  benchmarking programs  fall short when  measuring modern  'smart'

storage systems, or when unusual  disk configurations are used. Many of

the most important performance factors  are not taken into account, and

the  characteristics of the user's  application don't play any role  in

the  measurement when these  utilities are  used. The DynaTek  RAIDmark

has  been  created  to  overcome these  limitations,  operating  as  an

application-level benchmark  program. RAIDmark measures storage  system

performance  in various  typical environment  conditions, and  provides

the  user with a  good indication of the  expected usefulness of  their

system.

*    Technical reference

-    Patterson, David A.; Gibson, Garth; Katz, Randy H.:

"A  Case for Redundant Arrays  of Inexpensive Disks  (RAID)";

ACM SIGMOD (Chicago, 1988).

-    Patterson, David A.; Chen, Peter;  Gibson, Garth; Katz, Randy

H.:

"An  Introduction to  Redundant Arrays  of Inexpensive  Disks

(RAID)"; IEEE 1989.

-    Integra Technologies, Inc. - Graham, Richard A.:

"Redundant Arrays of Inexpensive Disks" (1991)

-    Finney, Kenneth C.; Graham, Richard A.:

"A Discussion of RAID Technology"; DynaTek/Integra, 1992.

DynaTek and RAIDmark are trademarks of Dynatek Automation Systems Inc.

HPFS,  IBM, Integra, MS-DOS, MS-Windows,  OS/2 and PS/2 are  registered

trademarks of their respective holders.

Page 12