.TH  Viewimage "Display Utils"

.SH

Threshold/Windowing

.LP

This subform allows the user to perform pixel thresholding and/or

pixel windowing operations.

.SH

1)  Pixel Windowing

.PP

Pixel windowing is used to mask out certain parts of the image so that

only a desired range of pixels is observed.  Pixel windowing works by

allowing a windowed region of the original pixel values to be displayed.

The user selects the pixel value of the masked region; all values outside

the windowed region will be set to this masked region pixel value.

If the masked region pixel value does not exist in the colormap,

the corresponding grey level value will be used.

.PP

In order to make sure that none if the other \fBviewimage\fP features

(specifically, pseudocolor or LUT) over-ride this threshold, we set the

pixel's histogram to zero.  None of the other \fBviewimage\fP features will

be able to update these pixels with new values, until the histogram is restored.

.sp

.IP "a)  Use of the Scroll Bars with Pixel Windowing"

The desired range of pixels to be observed is set using the scroll bars that

appear at the bottom of the Threshold/Windowing subform. The top scroll bar

sets the lower end of the desired pixel range;  the bottom scroll bar sets

the upper end of the desired pixel range.  The value boxes to the right of each

scroll bar displays the values to which the lower and upper pixels of

the range are set.

.sp

.IP "b)  The Masked Region Pixel Value"

This is the pixel value used in the masked regions.

.sp

.IP "c)  The Windowed Region - accept/reject"

If the windowed region is set to "accept", the original pixel values between

the values set by the lower scroll bar and the upper scroll bar will be

displayed;  this can be thought of as a bandpass filter.  If the windowed

region is set to "reject",  the original pixel values \fIoutside\fP the

region defined by the lower and upper scroll bars will be displayed; this

can be thought of as a bandreject filter.

.sp

.IP "d)  The Color Strip"

The color strip above the scroll bars displays the region windowed and

masked, based on the lower and upper scroll bar positions.

.sp

.SH

2)  Pixel Thresholding

.PP

Pixel thresholding is used to perform real-time image thresholding.

It generates a binary (or 2 pixel value) image.  Pixel thresholding works by

assigning a non-zero pixel value to the selected region; the rest of the image

will be turned black.

.sp

.IP "a)  Use of the Scroll Bars with Pixel Thresholding"

The desired range of pixels to be thresholded is set using the scroll bars that

appear at the bottom of the Threshold/Windowing subform. The top scroll bar

sets the lower end of the desired pixel range;  the bottom scroll bar sets

the upper end of the desired pixel range.  The value boxes to the right of each

scroll bar displays the values to which the lower and upper pixels of

the range are set.

.sp

.IP "b)  The Non-Zero Pixel Value"

This is the pixel value assigned to the all the pixels within the range defined

by the scroll bars.  Note that pixels outside this range will appear black.

.sp

.IP "c)  The Threshold Region - non-inverted/inverted"

If the threshold region is set to "non-inverted", the non-zero pixel value

will be displayed between the limits set by the lower scroll bar and the

upper scroll bar;  the rest of the image will be black.  If the threshold region

is set to "inverted",  the non-zero pixel value will be displayed outside the

limits set by the lower scroll bar and the upper scroll bar.

.sp

.IP "d)  The Color Strip"

The color strip above the scroll bars displays the thresholded region

based on the lower and upper scroll bar positions.

.sp

.SH

Reset

.PP

Clicking on the "RESET" button will re-set the currently displayed image

to it's original state when the Windowing/Thresholding subform was entered

for the first time.