NOISE.SYS (Version 0.5.5-Beta) - Technical Info

Warning: This file is still under construction. A detailed explanation of

the driver's innards is forthcoming...  If you have any questions

please E-mail me at <wlkngowl@unix.asb.com>

NOISE.SYS API (Version 0.5.3 and later)

--------------------------------------------------------------------------

Note: A proprosed API for NOISE.SYS v0.5.2 was distributed to a few people,

however it differs from the actual implementation.

For now NOISE.SYS uses Interrupt 32h for the API.  If no conflicts

with other programs arise, this will not change.  The AMIS specif-

ication was chosen so that a move to Int 2Dh is easier, if it seems

necessary... but it also allows developers of other TSRs or drivers

to use Int 32h if they adhere to the AMIS spec.

An IOCTL (IO-Control) read from the /dev/random device will return the

following record:

Offset        Size    Description

0            2       szControlOut    Size of control record

2            4       IdTag == 'noiz' Identifier Tag

6            2       Version

8            1       Interrupt       (00h if none)

9            1       Function        (in AH register)

10            4       Far call entry  (00000000h if none)

14            ?                       Reserved

Other fields may be defined in the source code, but they are subject

to changes in future versions.

Note that the Id Tag differs from earlier versions (pre-0.5.3)

because the API is not backward compatible.  Earlier versions

of the API (using IOCTL writes) will not be supported.

Default values for Interrupt and Function are 32h and 6Eh.

----------326E-------------------------------

INT 32 - NOISE.SYS Version 0.5.3+ API

AH = 6Eh ('n') (function id)

AL = subfunction (See INT 32/AX=6E00h)

Return: CF set on error

AL = error codes returned by the API

00h = subfunction not supported

FBh = random pool is empty

FCh = quality of sample to too low

FDh = too many processes using the API or driver

FEh = subfunction is disabled in the current build

FFh = ok

CF clear if successful

Notes: INT 32 is only a proposed interface for NOISE.SYS. Use the controller

read (IOCTL READ) from the RANDOM device to determine the interrupt

and function Id used by the driver, since future versions may use the

Alternate Multiplex Interrupt Service (AMIS) at Int 2Dh.

Assume that unspecified registers (other than DS:SI) may be trashed.

SeeAlso: INT 2D/AMIS

----------326E00-----------------------------

INT 32 - NOISE.SYS - INSTALLATION CHECK

AH = 6Eh

AL = 00h

Return:

AL = installation status

00h = not installed

FFh = installed

CX = Version (ie, 0123h = Version 1.2.3)

DX:DI -> AMIS signature string

----------326E01-----------------------------

INT 32 - NOISE.SYS - GET PRIVATE API ENTRY POINT

AH = 6Eh

AL = 01h

Return:

AL = entry point status

00h = no entry point

FFh = entry point present in DX:BX

DX:BX -> far call entry point to API

----------326E02-----------------------------

INT 32 - NOISE.SYS - Unused but reserved for AMIS

AH = 6Eh

AL = 02h

Return:

AL = 00h

----------326E03-----------------------------

INT 32 - NOISE.SYS - Unused but reserved for AMIS

AH = 6Eh

AL = 03h

Return:

AL = 00h

----------326E04-----------------------------

INT 32 - NOISE.SYS - GET INTERRUPT HOOK LIST

AH = 6Eh

AL = 04h

BL = ignored

Return:

AL = status

00h = unimplemented

04h = DX:BX -> interrupt hook list

FEh = subfunction disabled

Note: the hook list array ends with API interrupt (usually 32h) although

it will differ if the API is installed at another interrupt

----------326E05-----------------------------

INT 32 - NOISE.SYS - Unused but reserved for AMIS

AH = 6Eh

AL = 05h

Return:

AL = 00h

----------326E06-----------------------------

INT 32 - NOISE.SYS - GET DEVICE DRIVER HEADER

AH = 6Eh

AL = 06h

Return:

AL = number of device drivers in NOISE.SYS chain

02h = default (for RANDOM and URANDOM devices)

AH = AMIS device driver flags (set to 00h for now)

DX:BX -> first device in chain

Note: this function is a proposed addition to the AMIS standard

and is subject to change

----------326E07-----------------------------

INT 32 - NOISE.SYS - Unused but reserved for AMIS

AH = 6Eh

AL = 07h through 0Fh

Return:

AL = 00h

Note: These functions are reserved for future use in the AMIS standard

----------326E10-----------------------------

INT 32 - NOISE.SYS - STATUS CHECK

AH = 6Eh (function id)

AL = 10h

Return: CF set on error

AL = error codes returned by the API

FDh = too many processes using the API

CF clear if successful

AL = status

FFh = ok

BH = number of processes using the API

CX = number of random bytes waiting

DX = maximum possible bytes waiting

if CX=DX, the pool is full

Notes: this subfunction is a convenient way to check the driver if any

fresh bytes are waiting in the output pool.

SeeAlso: INT 32,AH=6Eh

----------326E11-----------------------------

INT 32 - NOISE.SYS - GET ENTROPY ESTIMATE

AH = 6Eh (function id)

AL = 11h

Return: CF set on error

AL = error codes returned by the API

00h = subfunction not supported

FDh = too many processes using the API

FEh = subfunction is disabled

CF clear if successful

EBX = estimated bit count (See notes)

CL = FRACBITS (number of fractional bits)

EDX = low 32-bits of total number of samples added

Notes: the estimated number of fresh random bits is equal to

(EBX >> FRACBITS) + ((EBX & ((1 << FRACBITS)-1) / (1 << FRACBITS))

----------326E12-----------------------------

INT 32 - NOISE.SYS - ADD SAMPLE FROM FAST TIMER

AH = 6Eh  (function id)

AL = 12h

Return: CF set on error

AL = error code

FCh = quality of sample is too low

FDh = too many processes using the API

FEh = subfunction is disabled

CF clear if successful

CX = number of random bytes waiting

----------326E13-----------------------------

INT 32 - NOISE.SYS - ADD 16-BIT SAMPLE TO RANDOM POOL

AH = 6Eh (function id)

AL = 13h

DX = sample

Return: CF set on error

AL = error code

FCh = quality of sample is too low

FEh = subfunction is disabled

CF clear if successful

CX = number of random bytes waiting

----------326E14-----------------------------

INT 32 - NOISE.SYS - GET FLAGS

AH = 6Eh  (function id)

AL = 14h

Return:

BX = flags

CX = settable flags mask

Note: later versions may use EBX and ECX if needed.

Defined flags:

0001h = Windows is active

0002h-0020h = reserved

0040h = clock drift sampling

0080h = video retrace drift sampling

0100h = network access sampling (reserved)

0200h = CD-ROM access sampling (reserved)

0400h = DOS spinner

0800h = DOS exec() and exit() sampling

1000h = mouse sampling

2000h = disk sampling (Int 13h)

4000h = keystroke timings

8000h = hardware RNG (reserved)

----------326E15-----------------------------

INT 32 - NOISE.SYS - SET FLAGS

AH = 6Eh  (function id)

AL = 15h

BX = flags

Return:

BX = flags

Note: invalid flags are ignored, however it's best to mask out unsettable

flags from the value of CX returned by subfunction 14h

----------326E16-----------------------------

INT 32 - NOISE.SYS - READ URANDOM BYTES

AH = 6Eh  (function id)

AL = 16h

CX = number of bytes

ES:DI -> buffer

Return: CF set on error

AL = error code

FDh = too many processes using the API

FEh = subfunction is disabled

CF clear if successful

CX = number of random bytes read

----------326E17-----------------------------

INT 32 - NOISE.SYS - READ RANDOM BYTES

AH = 6Eh  (function id)

AL = 17h

CX = number of bytes

ES:DI -> buffer

Return: CF set on error

AL = error code

FBh = random pool is empty

FDh = too many processes using the API

FEh = subfunction is disabled

CF clear if successful

CX = number of random bytes read

Unsorted References

--------------------------------------------------------------------------

Baresel, Andre and Craig Jackson. 1995. "Soundblaster Programming

Information."  Version 0.90.  Online.  Internet.

<URL ftp://oak.oakland.edu/SimTel/msdos/sound/sblast09.zip>

Brown, Ralf. 1995. "MS-DOS Interrupt List".  Online (many sites).

-----. nd. "Alternate Multiplex Interrupt Specification (AMIS)."

v3.5. np. and v3.5.9 (Pers. corr.).

"DOS Programmer's Reference" 2nd Edition.  Terry Dettmann and Jim

Kyle, Eds.  Que Books.  1989.

Dunford, Chris. 1991. "IBM's Interrupt-Sharing Protocol." np.

Kyle, Jim. 1990. "Loading Device Drivers from the DOS Command

Line." Dr. Dobb's Journal.  November 1991. pp. 30-42, 90-98.

(See also the "Undocumented DOS" by the same author.)

Petzold, Charles. "Programming Windows". Microsoft Press.

Macmillan/Penguin. 1990.

Eastlake, D, S. Crocker and J. Schiller. 1994.  RFC-1750.

"Randomness Recommendations for Security." December 1994.

Elliott, Paul. "rng_810.zip" OS/2 driver for CALNET/Newbridge

RNG-810 card.

Ellison, Carl <cme@tis.com> and Burt Kaliski <burt@rsa.com>. 1995.

"P1363: Appendix E -- Cryptographic Random Numbers."  Draft V1.0.

November 11, 1995. Online.  Internet.

<URL http://www.clark.net/pub/cme/P1363/ranno.html>

FIPS PUB 180-1. 31 May 1994. "The Secure Hash Standard."

Gutmann, Peter. 1992.  Source code for "shs.c" and "shsdrvr.c".

Jenkins, Bob. 1994. "A tester of random number generators." from

a post to sci.crypt, July 13, 1995.

Lendl, Otmar. 1996. "Pseudorandom number generation package" v1.3.

Online. Internet. <URL ftp://random.mat.sbg.ac.at/pub/software/>.

Levin, Michael, Marvin Minsky, and Roland Silver.  1965.  "One the

Effective Definition of 'Random Sequence'."  Online.  Internet.

<URL http://www.swcp.com/artvark/random/random.html>.

Marsaglia, George. 1996. "DIEHARD: a battery of tests of randomness.

(Source C files, Jan 3 1996)"

See also "DIEHARD". Online. Internet.

<URL http://stat.fsu.edu/~geo/diehard.html>

Maurer, Ueli M. 1990. "A Universal Statistical Test for Random Bit

Generators."  Advances in Cryptology - CRYPTO '90, Proceedings.

Springer-Verlag, Berlin, New York.

Mitchell, D.P., Jack Lacy, and Matt Blaze. 1995.  Source code for

"truerand".

There is a similar truerand() implementation by Phil Karn as well.

"truerand" uses CPU clock-drift sampling as a source of entropy.

Pickover, Clifford A. 1995. "Random number generators: pretty good

ones are easy to find."  The Visual Computer (1995) 11:369-377.

This article discusses some interesting visual methods for testing

pseudo-RNGs such as "noise spheres".  A utility that plots a noise

sphere written in Turbo Pascal is available at ftp.funet.fi in the

/pub/crypt/random directory.  Implementations in C, X-Windows and

and Java exist.

See also Pickover, 1995. "Keys to Infinity."

Ritter, T. 1991. "The Efficient Generation of Cryptographic

Confusion Sequences." Cryptologia. 15(2): 81-139. Online. Internet.

<URL http://www.sys.uea.ac.uk/~rs/ritter.html>

Rivest, Ron. 1992. RFC-1321.  "The MD5 Message Digest Algorithm."

Schneier, Burce. 1994.  "Applied Cryptography."  New York: John

Wiley and Sons, Inc. (Second edition, 1995.)

"Selected Bibliography of Random Number Generation, Analysis, and

Use."  Online.  Internet.

<URL http://rainbow.rmii.com/~comscire/QWBIB.html> (21 Mar 1996)

Theodore T'so. 1995. Source code to /dev/random ("random.c") for

Linux, Version 0.95 (4-Nov-95).

Wagner, David. "Randomness resources for Dr. Dobb's Journal readers."

Online.  Internet.

<URL http://http.cs.berkeley.edu/~daw/netscape-randomness.html>.