X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Variac core cutting?

Date: Mon, 02 Nov 1998 07:46:19 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello Coilers,

Why are we supposed to cut a slice out of an old variac core when

making a ballast for a pole pig?

I understand that making the cut will increase the current required to

saturate the core (for a fixed number of winding turns).

However, the cut also decreases the inductance of the core, thus

requiring the addition of more turns to bring the inductance back up!

Remember - it is the non-saturated inductance which limits the pig's

current!

Are these effects contradictory?

Thanks!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: "'brianb@mail.hughes.com'" <brianb@mail.hughes.com>

Subject: Tuning and Performance Questions (6.5" coil)

Date: Mon, 2 Nov 1998 08:01:50 -0800

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

I've been tuning my 6.5" coil and have reached a new personal best of

consistent 60" strikes to a grounded rod with 1.6kva input.

I have two questions/concerns:

1) Toroid height affects my coil operation dramatically.  Not just spark

length but consistency and smoothness of operation. With the toroid set at

it's current height above the secondary, I get a faint purple corona from

the top of the secondary down to the strike rail.  Is this OK? If I raise

the toroid 1/2" I get breakout from the top turns of the secondary.

Lowering the toroid 1/2" destroys performance.

2) My capacitor bank consists of four rolled poly caps in a parallel/series

arrangement for a total of .042mfd with a total dielectric thickness of

.072" (spread over two capacitors).  I also have a safety gap across the

capacitor bank.  It's made of two 1/4" stove bolts with their heads facing

each other and a gap of .3"  When the system is in tune my safety gap starts

to fire whenever my variac voltage gets to 100v (when out of tune it fires

much sooner).  Am I being too conservative with this gap?

3) Performance was in the 34-36" range until the last marathon tuning

session where I did a lot of work on toroid height and fine tuning of the

tap.  I now achieve consistent 60" strikes.  Is this good performance for

1.6kva into the system or should I look for more?

Settings:

Current limited to 17amps using three MOT's (with shorted secondaries) and

6.2 ohms (all in parallel).

Maximum Variac setting is 95volts

Primary Tap is at 7-5/8 turns

Spark Gap is RQ type (with a blower) using five gaps for .150"

Secondary windings start .5" above the primary

Toroid is 4.5" above the top secondary winding

Coil specifications are as follows:

Primary Transformer

Input Voltage		95 VAC

Input Current		17 Amps

Output Voltage		12,000 VAC (@120v in)

Input frequency		60 HZ

Primary Design

Inside Diameter		9.5 inches

Average Radius		8.88 inches

Wire Diameter		.025 inches

Total Diameter		26.0 inches

Wire Spacing		0.25 inches

Wire Length	      	79.0 feet

Total Turns	      	17.0

Secondary Res. Freq.	124.2 khz

Inductance	      	0.1407 mh

Primary Cap to Res.	0.0117 uF

Angle of Windings 	0.0 degrees

Secondary Design

Diameter	      	6.50 inches

Radius			3.25 inches

Winding Length		27.5 inches

Circumference		20.42 inches

Wire Gauge	      	22 awg

Wire Length	      	1698 feet

Turns per Inch		37.4

H/D Aspect	      	4.23

Inductance	      	34.56 mh

Resonate Frequency	248.1 khz

Medhurst_K	      	0.72

Res. Freq. w/Toroid	130 khz

Self_C			11.91 pf

Top Load

Type			Dryer Duct Toroid

Size			6" chord x 30" diameter

Top Load C		32.1pf

Tank Capacitors

Number of Capacitors	4

Capacitor Configuration	Series/Parallel

Type		      	Rolled LDPE

Overall C	      	.042 uf

C of each unit		.042 uf

Dielectric Constant	2.0

Plate Length		120 inches

Plate Width	      	14 inches

Dielectric	      	.036 inches (6@.006")

Pictures available at the following:

http://www2.hughes.net/~brianb/teslahome.html

Thanks in advance (hope this isn't too much information)

Brian D. Basura

X-Mailer: IPAD 2.0

Date: Mon, 02 Nov 1998 12:03:17 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Variac core cutting?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:46 AM 11/2/98 PST, you wrote:

>Hello Coilers,

> Why are we supposed to cut a slice out of an old variac core when

>making a ballast for a pole pig?

>

> I understand that making the cut will increase the current required to

>saturate the core (for a fixed number of winding turns).

>

> However, the cut also decreases the inductance of the core, thus

>requiring the addition of more turns to bring the inductance back up!

>Remember - it is the non-saturated inductance which limits the pig's

>current!

>

> Are these effects contradictory?

>

>Thanks!

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH

>http://www.geocities.com/CapeCanaveral/Hangar/6160

Yes those effects are somewhat contradictory. However the most

importance aspect is the prevention of saturation. Because a

saturated ballast is no ballast at all.

I know of at least one person using a variac as a current limiter

without gapping the core. As long as you are not in danger of

saturation I would not gap the core. I just cant see sawing

into an expensive variac.

The other problem with variacs as current limiters is poor range

of adjustment. The setting between just enough current and way

too much current might amount to just a couple degrees of rotation.

Considering the cost of a large variac I would be more inclined

to purchase an electric welder to use as ballast. You can always

use it as a welder when not coiling.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 02 Nov 1998 12:28:27 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning and Performance Questions (6.5" coil)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:01 AM 11/2/98 -0800, you wrote:

>Hello,

>

>I've been tuning my 6.5" coil and have reached a new personal best of

>consistent 60" strikes to a grounded rod with 1.6kva input.

Not too bad at all. Consistant 60 inches at 1.6 kva is doing

quite well.

>

>I have two questions/concerns:

>

>1) Toroid height affects my coil operation dramatically.  Not just spark

>length but consistency and smoothness of operation. With the toroid set at

>it's current height above the secondary, I get a faint purple corona from

>the top of the secondary down to the strike rail.  Is this OK? If I raise

>the toroid 1/2" I get breakout from the top turns of the secondary.

>Lowering the toroid 1/2" destroys performance.

Sounds like you might be slightly overcoupled. Try raising up the

secondary just a little. Sometimes just a 1/4 inch here can make

a difference. In general try to raise the toroid as much as you

can without losing the corona suppression it provides.

>

>2) My capacitor bank consists of four rolled poly caps in a parallel/series

>arrangement for a total of .042mfd with a total dielectric thickness of

>.072" (spread over two capacitors).  I also have a safety gap across the

>capacitor bank.  It's made of two 1/4" stove bolts with their heads facing

>each other and a gap of .3"  When the system is in tune my safety gap starts

>to fire whenever my variac voltage gets to 100v (when out of tune it fires

>much sooner).  Am I being too conservative with this gap?

Yes I think that is quite conservative for 12kv operation.

I run my safety gap at .375 inch (3/8) for use at 12720 V.

You can afford to open it slightly. Only open it as much as

necessary.

>

>3) Performance was in the 34-36" range until the last marathon tuning

>session where I did a lot of work on toroid height and fine tuning of the

>tap.  I now achieve consistent 60" strikes.  Is this good performance for

>1.6kva into the system or should I look for more?

Yes its very good. Im getting consistant 50 inches at 1.2 kva

on a 4 inch diameter secondary. So you can probably squeeze a little

more out of yours.

With the level of capacitance you have and your toroid size

I would investigate a larger toroid. Especially a larger

cross section. If your current toroid is displaying several

simultaneous streamers rather than a single longer one thats

a sign you need a larger toroid.

A larger toroid will lower the operating frequency lowering losses

somewhat and allow you to tap in more primary turns which should

help to improve gap operation. Especially since you are only tapping

in 7 5/8 turns out of 17. My coil has always worked better and better

the more primary inductance I had tapped in.

I also see your gap consists of 5 series gaps with a total of

.150 inch. Thats pretty narrow for 12kv. I would add a couple more

electrodes and run a little wider. I normally run around .200-.225

inch for 12kv.

<SNIP SPECS>

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Tuning and Performance Questions (6.5" coil)

Date: Mon, 2 Nov 1998 12:00:26 -0800

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I'll raise the secondary a 1/4" at a time and see how it works.  In the past

I raised it in 1/2" steps which may have been too great a change to find the

sweet spot.

The reason I limited my SG to .150" was to keep the capacitor safety gap

from firing. I'll try increasing the safety gap to 3/8" and adding more main

gaps in .030" increments.

I'll also look into increasing my topload C so I can add more primary turns.

Thanks for your input.

Brian D. Basura

-----Original Message-----

From: Mike Hammer [mailto:mhammer@misslink.net]

Sent: Monday, November 02, 1998 10:28 AM

To: Tesla-2 Mailing List

Subject: Re: Tuning and Performance Questions (6.5" coil)

At 08:01 AM 11/2/98 -0800, you wrote:

>Hello,

>

>I've been tuning my 6.5" coil and have reached a new personal best of

>consistent 60" strikes to a grounded rod with 1.6kva input.

Not too bad at all. Consistant 60 inches at 1.6 kva is doing

quite well.

>

>I have two questions/concerns:

>

>1) Toroid height affects my coil operation dramatically.  Not just spark

>length but consistency and smoothness of operation. With the toroid set at

>it's current height above the secondary, I get a faint purple corona from

>the top of the secondary down to the strike rail.  Is this OK? If I raise

>the toroid 1/2" I get breakout from the top turns of the secondary.

>Lowering the toroid 1/2" destroys performance.

Sounds like you might be slightly overcoupled. Try raising up the

secondary just a little. Sometimes just a 1/4 inch here can make

a difference. In general try to raise the toroid as much as you

can without losing the corona suppression it provides.

>

>2) My capacitor bank consists of four rolled poly caps in a parallel/series

>arrangement for a total of .042mfd with a total dielectric thickness of

>.072" (spread over two capacitors).  I also have a safety gap across the

>capacitor bank.  It's made of two 1/4" stove bolts with their heads facing

>each other and a gap of .3"  When the system is in tune my safety gap

starts

>to fire whenever my variac voltage gets to 100v (when out of tune it fires

>much sooner).  Am I being too conservative with this gap?

Yes I think that is quite conservative for 12kv operation.

I run my safety gap at .375 inch (3/8) for use at 12720 V.

You can afford to open it slightly. Only open it as much as

necessary.

>

>3) Performance was in the 34-36" range until the last marathon tuning

>session where I did a lot of work on toroid height and fine tuning of the

>tap.  I now achieve consistent 60" strikes.  Is this good performance for

>1.6kva into the system or should I look for more?

Yes its very good. Im getting consistant 50 inches at 1.2 kva

on a 4 inch diameter secondary. So you can probably squeeze a little

more out of yours.

With the level of capacitance you have and your toroid size

I would investigate a larger toroid. Especially a larger

cross section. If your current toroid is displaying several

simultaneous streamers rather than a single longer one thats

a sign you need a larger toroid.

A larger toroid will lower the operating frequency lowering losses

somewhat and allow you to tap in more primary turns which should

help to improve gap operation. Especially since you are only tapping

in 7 5/8 turns out of 17. My coil has always worked better and better

the more primary inductance I had tapped in.

I also see your gap consists of 5 series gaps with a total of

.150 inch. Thats pretty narrow for 12kv. I would add a couple more

electrodes and run a little wider. I normally run around .200-.225

inch for 12kv.

<SNIP SPECS>

From: DavidAllan@aol.com

Date: Mon, 2 Nov 1998 15:17:15 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: First light, sort of (need tuning help?)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well, I had the chance to energize my coil last Friday night. I had hoped to

be able to tune it up and get it working well enough to add to my Halloween

display. But, I wasn't able to tune it satisfactorily. I have read Mike

Hammer's Tuning FAQ (thanks, Mike), but I still have a few questions. First,

the particulars on my coil. It is built around a 10 kV OBIT. Here are the

specs:

Primary Transformer Output Voltage (VAC): 10 kV

Primary Transformer Rated Current (AMP): 23 mA

Primary Coil Inside Diameter (IN): 5.5

Primary Coil Wire Diameter (IN):  0.25 Cu tubing

Primary Coil Wire Spacing (IN):  0.25

Primary Coil Number of Turns: 14.8

Primary Coil Angle From Horizontal Plane (DEGREES): 30

Roll Capacitance (uF): Two series 0.012 caps for 0.0061 uF total (nominal)

Secondary Wire Diameter (IN): 0.0285 (22 ga)

Secondary Coil Total Outside Diameter (IN): 4.25

Secondary Coil Total Number of Turns: 700

Length of Secondary Wire (IN):19.04

Top Load Capacitance (PF):19.6

Toroid Tube Diameter (IN): 4

Toroid Diameter Tube Center-to-Center (IN): 14

Cylindrical Spark gap for 0.25-0.28 total

Following the recommendations, I started at low power (20 V on variac) and

reduced gap, probably 0.06 inches or so. (I had some difficulty getting

accurate placement of the gap elements in my cylindrical gap. Still, I guess

it should work.) I was able to tune it somewhat using a quarter inch stove

bolt as the top electrode of the secondary. I could see small (less than one

inch) discharges as I ramped the power up. Still, I didn't know what to expect

and don't know how I was doing.

Here are some questions:

1.  Am I just underpowered for the size secondary and top load that I have? I

have another OBIT that I can stick in, but I'd like to get it going with what

I've got right now.

2. What is the advantage to tuning without the top load (as suggested in the

FAQ)? Don't I have to repeat the whole process (close the gap, go to low

power, etc.) after I add the top load (with a point discharge, e.g.

thumbtack)?

3. I'm going to move from my front porch into my garage. I guess I need to

build another RF ground. How long can my wire run to the ground be? What kind

of wire? Does it need HV insulation? How do I protect my GDO (garage door

opener--don't you love TLAs :-)?

Thanks in advance for help.

David Allan

X-Mailer: IPAD 2.0

Date: Mon, 02 Nov 1998 14:45:10 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Tuning and Performance Questions (6.5" coil)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:00 PM 11/2/98 -0800, you wrote:

>Mike,

>

>I'll raise the secondary a 1/4" at a time and see how it works.  In the past

>I raised it in 1/2" steps which may have been too great a change to find the

>sweet spot.

>

>The reason I limited my SG to .150" was to keep the capacitor safety gap

>from firing. I'll try increasing the safety gap to 3/8" and adding more main

>gaps in .030" increments.

>

>I'll also look into increasing my topload C so I can add more primary turns.

>

>Thanks for your input.

>

>Brian D. Basura

>

Brian,

Change only one thing at a time and test the coil

for any improvement. Since your coil is running well

you wont need any dramatic changes.

You do have a safety gap across your transformer as

well dont you? I would set this gap to .375 and use

it as an indicator of when you have enough main gap.

Increase your main gap one step at a time and note

how smoothly the coil runs. If it starts running

roughly or sputters then back off the main gap somewhat.

I think your coil is pretty well set up and only some minor

tweaking would be required.

Good luck.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 3 Nov 1998 09:06:16 +1200

Subject:       Re: Variac core cutting?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Variac core cutting?

> Date sent:      Mon, 02 Nov 1998 07:46:19 PST

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hello Coilers,

>  Why are we supposed to cut a slice out of an old variac core when

> making a ballast for a pole pig?

>

>  I understand that making the cut will increase the current required to

> saturate the core (for a fixed number of winding turns).

>

>  However, the cut also decreases the inductance of the core, thus

> requiring the addition of more turns to bring the inductance back up!

> Remember - it is the non-saturated inductance which limits the pig's

> current!

>

>  Are these effects contradictory?

No. Let's say you require xH of inductance. The core is currently

ungapped and has a permeability of y. It will require n turns to hit

the required inductance. Ampere-turns applied to the core is z say.

Suppose z is too high. That implies that core flux density generated

by z is too high. Insert a gap in the core. This has the effect of

adding reluctance to the magnetic path. Now the flux is thinned out

along this path and hence the flux/unit area of the core has gone

down. Required current and inductance remain the same and since core

permeability has now dropped because of the gap, more turns are

needed to hit the required inductance and (note well), the wire

diameter should be increased to keep copper losses the same. The

effect of increasing n is to increase z but you will find that the

flux generated by the increased z does not reach the previous level

*in the core itself* because in adding the gap, you have multiplied

the path length of the core greatly.

This probably doesn't seem to wash if you read it in words like

this but if you look at the math it certainly does. Flux = applied

magnetizing force divided by the reluctance of the magnetic path and

you'll find that the permeability doesn't drop as rapidly as the

reluctance increases because the core material is still present. The

effect of this is to slant and stretch the B-H curve for the core so

you can now apply a much higher magnetizing force to it + gap before

you hit the same flux density.

Pros and Cons: coupling between turns and windings is slightly

decreased as gapping increases. More turns are required as gapping

increases for a given target inductance and wire size must also

increase to maintain a constant copper loss.

In practice, you find that as you gap a core up, the core itself

can decrease in volume but the copper keeps on increasing so you

eventually hit a point where there is not enough room for the

required windings. Again, this might seem a bit nebulous but it

certainly washes both mathematically and in practice.

Malcolm

X-Mailer: IPAD 2.0

Date: Mon, 02 Nov 1998 17:26:03 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First light, sort of (need tuning help?)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:17 PM 11/2/98 EST, you wrote:

>Well, I had the chance to energize my coil last Friday night. I had hoped to

>be able to tune it up and get it working well enough to add to my Halloween

>display. But, I wasn't able to tune it satisfactorily. I have read Mike

>Hammer's Tuning FAQ (thanks, Mike), but I still have a few questions. First,

>the particulars on my coil. It is built around a 10 kV OBIT. Here are the

>specs:

>

>    Primary Transformer Output Voltage (VAC): 10 kV

>    Primary Transformer Rated Current (AMP): 23 mA

>    Primary Coil Inside Diameter (IN): 5.5

>    Primary Coil Wire Diameter (IN):  0.25 Cu tubing

>    Primary Coil Wire Spacing (IN):  0.25

>    Primary Coil Number of Turns: 14.8

>    Primary Coil Angle From Horizontal Plane (DEGREES): 30

>    Roll Capacitance (uF): Two series 0.012 caps for 0.0061 uF total (nominal)

>    Secondary Wire Diameter (IN): 0.0285 (22 ga)

>    Secondary Coil Total Outside Diameter (IN): 4.25

>    Secondary Coil Total Number of Turns: 700

>    Length of Secondary Wire (IN):19.04

>    Top Load Capacitance (PF):19.6

>    Toroid Tube Diameter (IN): 4

>    Toroid Diameter Tube Center-to-Center (IN): 14

>    Cylindrical Spark gap for 0.25-0.28 total

>

>Following the recommendations, I started at low power (20 V on variac) and

>reduced gap, probably 0.06 inches or so. (I had some difficulty getting

>accurate placement of the gap elements in my cylindrical gap. Still, I guess

>it should work.) I was able to tune it somewhat using a quarter inch stove

>bolt as the top electrode of the secondary. I could see small (less than one

>inch) discharges as I ramped the power up. Still, I didn't know what to expect

>and don't know how I was doing.

>

>Here are some questions:

>

>1.  Am I just underpowered for the size secondary and top load that I have? I

>have another OBIT that I can stick in, but I'd like to get it going with what

>I've got right now.

No you are not really underpowered. You have available 230 Va and

Ive used a 7500V/30ma (225 Va) neon quite successfully (14 inch streamers)

with this size coil.

>

>2. What is the advantage to tuning without the top load (as suggested

>in the FAQ)?

One of the biggest stumbling blocks to beginners is getting their primary

circuit tuned to the same frequency as the secondary. Unless these

two resonant frequencies are very close you will get little or no

output. When you are far off of the proper tune as your new coil will

be its hard to tell how to procede without some visual indication.

With a large toroid attached you will probably get no output

sparks at all until the tuning gets pretty close. With a small

terminal like a copper toilet tank float or other small sphere

you will be able to get some spark even when far out of tune.

How does the amount of output you see vary with different tap points

on the primary coil? Do you get the most output with all of your primary

tapped in? If so the problem is insufficient primary inductance or capacitance

or both.

> Don't I have to repeat the whole process (close the gap, go to low

>power, etc.) after I add the top load (with a point discharge, e.g.

>thumbtack)?

Yes. But it becomes easier with experience and once you begin

to know how your coil performs.

>

>3. I'm going to move from my front porch into my garage. I guess I need to

>build another RF ground. How long can my wire run to the ground be?

Only as long as necessary. Run it in as straight and as short a line

as possible.

>What kind of wire? Does it need HV insulation?

For your power level almost any sizable wire will do. If the run

is short then some AWG 14 will probably do. As you increase power level

and/or ground lead length you will need heavier conductors.

It will not need HV insulation as it should be (hopefully) at ground

potential.

> How do I protect my GDO (garage door opener--don't you love TLAs :-)?

Operate your coil in the next county. Failing that unplug all wiring

going to the opener including the antenna. Place a grounded shield

around the opener and hope for the best.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 02 Nov 1998 19:51:50 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tuning and Performance Questions (6.5" coil)

Sender: tesl-err@emachine.com

Brian,

Good performing coil! My comments are interspersed below...

Basura, Brian wrote:

>

> Hello,

>

> I've been tuning my 6.5" coil and have reached a new personal best of

> consistent 60" strikes to a grounded rod with 1.6kva input.

>

> I have two questions/concerns:

>

> 1) Toroid height affects my coil operation dramatically.  Not just spark

> length but consistency and smoothness of operation. With the toroid set at

> it's current height above the secondary, I get a faint purple corona from

> the top of the secondary down to the strike rail.  Is this OK? If I raise

> the toroid 1/2" I get breakout from the top turns of the secondary.

> Lowering the toroid 1/2" destroys performance.

As you indicated below, raising or lowering the toroid will change the

secondary's resonant frequency somewhat - you may need to retune a bit

after each adjustment for best results. As long as the faint corona

doesn't result in actual secondary:primary or secondary inter-turn

flashovers, this should be OK. You might try increasing the secondary

heignt just a bit to reduce coupling and increase the toroid:strikerail

clearance.

>

> 2) My capacitor bank consists of four rolled poly caps in a parallel/series

> arrangement for a total of .042mfd with a total dielectric thickness of

> .072" (spread over two capacitors).  I also have a safety gap across the

> capacitor bank.  It's made of two 1/4" stove bolts with their heads facing

> each other and a gap of .3"  When the system is in tune my safety gap starts

> to fire whenever my variac voltage gets to 100v (when out of tune it fires

> much sooner).  Am I being too conservative with this gap?

Yes. The RF voltage developed across the tank cap ionizes the air more

easily - once you're sure you're in tune, try increasing the gap to 0.4"

or 0.5". Note: this larger distance ONLY applies to the safety gap

across the tank cap.

>

> 3) Performance was in the 34-36" range until the last marathon tuning

> session where I did a lot of work on toroid height and fine tuning of the

> tap.  I now achieve consistent 60" strikes.  Is this good performance for

> 1.6kva into the system or should I look for more?

This is very good performance for this power level! You are getting over

streamers that are almost 2.2X secondary coil length, which is also

quite good.

>

> Settings:

>

<SNIP>

>

> Pictures available at the following:

> http://www2.hughes.net/~brianb/teslahome.html

>

> Thanks in advance (hope this isn't too much information)

> Brian D. Basura

Excellent information re: your system!

Safe coilin' to you!

-- Bert --

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: First light, sort of (need tuning help?)

Date: Mon, 2 Nov 1998 20:25:31 -0800

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

>

> > How do I protect my GDO (garage door opener--don't you love TLAs :-)?

>

> Operate your coil in the next county. Failing that unplug all wiring

> going to the opener including the antenna. Place a grounded shield

> around the opener and hope for the best.

Pray. Alot. :)

However, these are the steps I took to protect my opener (and my

coiling rights.. the wife threatened to throw my coil out the window

if I killed it. When I mentioned that we didn't have a window in the

garage, I got a rather chilly "Yet.". Anyway, I digress.)

I built a full faraday cage of aluminum screen around the unit, and then

installed quick disconnects at the obstruction beam sensor and generator.

When I run the coil, I am truely paranoid. I unplug the GDO, and

place the coiled up power cable in the cage. I unplug the Obstruction system

and wrap the generator and receiver in aluminum foil, attached to ground.

I then (even though I should be protected) remove the two circut boards

from the GDO, and place them as far from where I am coiling as possible.

It survived halloween just fine. This was with my coil throwing 48"

streamers to the door, and associated hardware. [not bad for a guesstimated 1.2KVA]

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

Date: Mon, 02 Nov 1998 23:55:10 -0500

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: The start of my first coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

This is the start of my first coil. There is not much there yet, but

more will be coming soon (I hope) depending on how busy I am at work. I

welcome everyones creative criticism, although there is not to much to

show yet.

http://www.geocities.com/CapeCanaveral/Launchpad/9408/tesla/tesla.html

Regards,

Bert Wikkerink

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: tuning tips

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 03 Nov 1998 00:53:41 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've got a few tips for anyone to try from my limited experience.

I'm really a cut and try type.  I always start with the secondary all the way

down for over coupling.  For me, corona is good tuning indicator.  Start without

a toroid. Wire on top of secondary sticking strait up.  Start with a tap in the

middle of the primary turns.  Start with a decent size gap.  With a good size gap

you can often see some kind of corona even when way out of tune.  Start out

just looking for corona, move the tap around until you start getting some lighting

from the top.

Then start jacking up the secondary. Now add toroid and move tap out

three or four turns.  Once you are tuned in, raise toroid approximately

six inches above top turn and move tap out one turn.  I raise the

secondary until no more corona is visible except maybe a little on the top turn or

the wire leading to the toroid.

Now open your gap as wide as you are comfortable.

Important things to note:

I don't use a variac so it's always full power and only limited by

the size of the gap.

My coils are always taller than the norm. My six incher is 31

inches tall.  So I've never had any secondary to primary flash overs.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Tue, 03 Nov 1998 23:05:08 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 6" coil

Sender: tesl-err@emachine.com

I'm finally getting some respectable output from my new six inch coil.

After modififying my H&R transformer for higher voltage discharge length

has increased to at least 36 inches. One problem I had tonight was with

damping resistors. I have (had!) 3 kohm, 50 watt aluminum housed, wire

wound resistors on each high voltage output of the two transformers.

After running the coil for a few seconds one of them actually exploded!

I immediately killed the power and checked the transformers over. They

were undamaged. I am learning quickly that these transformers can sink

an enormous amount of current and not even get warm. After a few more

short runs the transformers were still ice cold!

As I mentioned in my earlier posts I removed 50 turns from the primary

of each antiparrallel transformer to increase the turns ratio. I also

knocked all of the shunts out of both transformers and added a choke in

series to control the current.

After my third run minus the destroyed resistors I tried cranking up

the variac a little more and the circuit breaker kicked out. It was only

rated at 20amps so my next stop will be to the hardware store to buy a

30amp. Also since I can't use damping resistors unless they are at

1000watts, or at low resistance. I'll have to rely on my LC filter and

safety gaps for protection. Now all I have do is tune it up for a larger

topload.

Frankensteins Helper

Max

Date: Wed, 4 Nov 1998 07:36:09 +0000

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Re: 6" coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In message <363FE083.5B81@magnolia.net>, RODERICK MAXWELL

<tank@magnolia.net> writes

>I'm finally getting some respectable output from my new six inch coil.

>After modififying my H&R transformer for higher voltage discharge length

>has increased to at least 36 inches. One problem I had tonight was with

>damping resistors. I have (had!) 3 kohm, 50 watt aluminum housed, wire

>wound resistors on each high voltage output of the two transformers.

>After running the coil for a few seconds one of them actually exploded!

>I immediately killed the power and checked the transformers over. They

>were undamaged. I am learning quickly that these transformers can sink

>an enormous amount of current and not even get warm. After a few more

>short runs the transformers were still ice cold!

>As I mentioned in my earlier posts I removed 50 turns from the primary

>of each antiparrallel transformer to increase the turns ratio. I also

>knocked all of the shunts out of both transformers and added a choke in

>series to control the current.

>  After my third run minus the destroyed resistors I tried cranking up

>the variac a little more and the circuit breaker kicked out. It was only

>rated at 20amps so my next stop will be to the hardware store to buy a

>30amp. Also since I can't use damping resistors unless they are at

>1000watts, or at low resistance. I'll have to rely on my LC filter and

>safety gaps for protection. Now all I have do is tune it up for a larger

>topload.

>

>

>                               Frankensteins Helper

>                                       Max

Hi Max,

Sound like you are having fun! :-)

A quick question, are your transformers submergered in oil?

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Bringing Tesla Excitement to Swindon, England

the land of 240VAC @ 50Hz

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

Date: Wed, 04 Nov 1998 12:59:02 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

> Hi Max,

>         Sound like you are having fun! :-)

> A quick question, are your transformers submergered in oil?

>

> ------------------------------------------------

> Stephen Bell        (steve@breakfix.demon.co.uk)

No they are dry as a bone!

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Wed, 04 Nov 1998 13:25:38 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:05 PM 11/3/98 -0600, you wrote:

>I'm finally getting some respectable output from my new six inch coil.

>After modififying my H&R transformer for higher voltage discharge length

>has increased to at least 36 inches. One problem I had tonight was with

>damping resistors. I have (had!) 3 kohm, 50 watt aluminum housed, wire

>wound resistors on each high voltage output of the two transformers.

>After running the coil for a few seconds one of them actually exploded!

>I immediately killed the power and checked the transformers over. They

>were undamaged. I am learning quickly that these transformers can sink

>an enormous amount of current and not even get warm. After a few more

>short runs the transformers were still ice cold!

>As I mentioned in my earlier posts I removed 50 turns from the primary

>of each antiparrallel transformer to increase the turns ratio. I also

>knocked all of the shunts out of both transformers and added a choke in

>series to control the current.

>  After my third run minus the destroyed resistors I tried cranking up

>the variac a little more and the circuit breaker kicked out. It was only

>rated at 20amps so my next stop will be to the hardware store to buy a

>30amp. Also since I can't use damping resistors unless they are at

>1000watts, or at low resistance. I'll have to rely on my LC filter and

>safety gaps for protection. Now all I have do is tune it up for a larger

>topload.

>

>

>                               Frankensteins Helper

>                                       Max

Roderick,

While removing primary turns will indeed increase the turns

ratio Im not sure it was wise to do so. As you have already

seen this has dramatically increased primary current. In fact

your transformers are probably saturating. Increasing your breaker

size will probably not help.

Have you tried replacing the magnetic shunts? You will probably

need to add additional ballasting as well.

I would of gone to the extra trouble to wind new secondary

windings for those transformers and left the primaries alone.

It probably would not have been too difficult to wind a 5kv

winding on each transformer and use them in series for 10kv.

Keep us posted on your results.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 5 Nov 1998 09:03:18 +1200

Subject:       Re: 6" coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Roderick,

I'd be most interested to hear what the magnetizing

current of the transformer is when full mains voltage is applied to

the primary. This would be a nice little follow up test to complement

my transformer post from several weeks ago.

Thanks,

Malcolm

> Date sent:      Tue, 03 Nov 1998 23:05:08 -0600

> From:           RODERICK MAXWELL <tank@magnolia.net>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        6" coil

> I'm finally getting some respectable output from my new six inch coil.

> After modififying my H&R transformer for higher voltage discharge length

> has increased to at least 36 inches. One problem I had tonight was with

> damping resistors. I have (had!) 3 kohm, 50 watt aluminum housed, wire

> wound resistors on each high voltage output of the two transformers.

> After running the coil for a few seconds one of them actually exploded!

> I immediately killed the power and checked the transformers over. They

> were undamaged. I am learning quickly that these transformers can sink

> an enormous amount of current and not even get warm. After a few more

> short runs the transformers were still ice cold!

> As I mentioned in my earlier posts I removed 50 turns from the primary

> of each antiparrallel transformer to increase the turns ratio. I also

> knocked all of the shunts out of both transformers and added a choke in

> series to control the current.

>   After my third run minus the destroyed resistors I tried cranking up

> the variac a little more and the circuit breaker kicked out. It was only

> rated at 20amps so my next stop will be to the hardware store to buy a

> 30amp. Also since I can't use damping resistors unless they are at

> 1000watts, or at low resistance. I'll have to rely on my LC filter and

> safety gaps for protection. Now all I have do is tune it up for a larger

> topload.

>

>

>                                Frankensteins Helper

>                                        Max

>

Date: Wed, 04 Nov 1998 18:05:10 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New site!

Sender: tesl-err@emachine.com

Eriksson wrote:

>

> Hello dear coilers! Ive almost managed to put up my tesla site, its not

> finished but there is lots of stuff coming up in the weekend so please be

> patient, but in the meantime you could check it out on

> www.zavior.mindflow.org and follow the tesla link..=)

> Safe coilin!

> Christopher Eriksson

> Please send suggestions/complains to eriksson@mindflow.org

Hint for sending URLs: if you want recipients to be able to click on

your link directly from their browser, preceed www with http://

Date: Wed, 04 Nov 1998 18:08:19 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New site!

Sender: tesl-err@emachine.com

Eriksson wrote:

>

> Hello dear coilers! Ive almost managed to put up my tesla site, its not

> finished but there is lots of stuff coming up in the weekend so please be

> patient, but in the meantime you could check it out on

> www.zavior.mindflow.org and follow the tesla link..=)

> Safe coilin!

> Christopher Eriksson

> Please send suggestions/complains to eriksson@mindflow.org

By the way, nice site!

Date: Wed, 04 Nov 1998 21:06:08 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> Hi Roderick,

>               I'd be most interested to hear what the magnetizing

> cyrrent of the transformer is when full mains voltage is applied to

> the primary. This would be a nice little follow up test to complement

> my transformer post from several weeks ago.

>

> Thanks,

> Malcolm

>

The full magnetizing current seems to hold at 3amps unloaded. In an

earlier post I mistakenly read the amp meter at six amps. Of course if

you use 50 cycle current where you are located this won't mean a thing!

Frankensteins Helper

Max

Date: Wed, 04 Nov 1998 21:32:40 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

> Roderick,

> While removing primary turns will indeed increase the turns

> ratio Im not sure it was wise to do so. As you have already

> seen this has dramatically increased primary current.

It was the secondary current that I saw a great increase in. The

primary current only increased a couple of amps.

> In fact your transformers are probably saturating. Increasing your breaker

> size will probably not help.

I neglected to tell you that the shop vac for the vaccuum gap is also

on the same circuit. This accounts for some of the current draw. I don't

think they are saturating. As I have stated before they stay ice cold after a

run. That includes primary, secondary, and the core. Also they do not

have that loud hum you associate with a transformer under excessive load.

>

> Have you tried replacing the magnetic shunts? You will probably

> need to add additional ballasting as well.

No I have not replaced the magnetic shunts. I want it to run full bore

to charge this huge 56nf cap I have in the tank circuit! I only have a

neon core as ballast in the primary. It seems to do a pretty good job.

>

> I would of gone to the extra trouble to wind new secondary

> windings for those transformers and left the primaries alone.

> It probably would not have been too difficult to wind a 5kv

> winding on each transformer and use them in series for 10kv.

To tell you the truth I'm really to lazy to wind all that wire. If

they should happen to blow though I'll rewind them with heavier gauge

wire. Maybe 26 gauge. But I'll still leave them unshunted and maybe sink

them in an oil tank.

Besides I intend to step up to the big leauges around Christmas and

see if a pig will fit under the tree!

Frankensteins Helper

Max

Date: Thu, 05 Nov 1998 10:38:52 -0800

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tesla Coil Stuff

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have some FREE tesla coil stuff available if anyone near me is

interested. I'm running out of space!

6  15kv/30ma transformers (working)

2  12kv/30ma transformers (working)

6  misc neon transformers (15's and 12's) that need to be rebuilt

1  4.5" x 21" secondary

A couple of toroids

some beer bottle caps

a decent flat primary

I don't really want to ship any of this stuff, so if you live anywhere

near me, let me know when you can swing by, and it's yours.

-tom (Tampa, FL)

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New site!

Date: Thu, 5 Nov 1998 23:29:43 +0100

X-Mailer: IPAD 2.0

X-MIME-Autoconverted: from 8bit to quoted-printable by mb07.swip.net id XAA06790

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello dear coilers! Ive almost managed to put up my tesla site, its not

finished but there is lots of stuff coming up in the weekend so please be

patient, but in the meantime you could check it out on

www.zavior.mindflow.org and follow the tesla link..=)

Safe coilin!

Christopher Eriksson

Please send suggestions/complains to eriksson@mindflow.org

Date: Thu, 05 Nov 1998 16:11:38 -0800

From: Adam Pasternak <pastea2@rpi.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla Coil Stuff

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I know you probably won't do this, but if you'd decide to ship a

transformer to me, I'd reimburse the shipping cost + 20% for your

trouble.....

Adam Pasterank (Albany, NY)

X-Originating-IP: [198.242.63.216]

From: "David Solina" <david_solina@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla Coil Stuff

Date: Thu, 05 Nov 1998 16:13:20 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey Tom

I am in Orlando and would love to pick this stuff up at your earliest convenience.

I'll even throw in some computer equipment in trade if you like.

I am just starting to build a coil and any pieces would be great!!!!

let me know how to contact you more directly

David Solina

solina@dpt.com

david_solina@hotmail.com

407-830-5522 X440 in the day

Date: Fri, 06 Nov 1998 10:06:08 -0800

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla Coil Stuff (gone)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Ok everyone, I received a huge number of responses for this stuff. Sorry

to everyone that missed it. I ended up giving it to a friend who lives

near me.  If I come across a few more transformers, I'll let all of the

people who responded know. I had no idea it was so difficult to find these

transformers.

cya

-tom

tomstock wrote:

X-Mailer: IPAD 2.0

Date: Fri, 06 Nov 1998 13:01:39 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla Coil Stuff (gone)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:06 AM 11/6/98 -0800, you wrote:

>

>Ok everyone, I received a huge number of responses for this stuff. Sorry

>to everyone that missed it. I ended up giving it to a friend who lives

>near me.  If I come across a few more transformers, I'll let all of the

>people who responded know. I had no idea it was so difficult to find these

>transformers.

>

>cya

>-tom

>

Tom,

I imagine the response was more like a stampede.

Neons are somewhat scarce in some parts of the country

but the major problem is most people just wont go out

and look. They call a couple nearby places and give up.

I scoured every sign place within a 75 mile radius of

me to collect the ones that I have.

I find that most sign places that do neon work are more

than happy to supply used and/or dead neons once they

see what you are going to use them for. Take a long a

couple pictures to generate interest.

Our European friends have a more difficult time. Due

to strict regulations appropriate neon transformers

just do not exist over there.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [207.100.20.26]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tesla Coil Stuff (gone)

Date: Fri, 06 Nov 1998 14:43:10 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Tom,

>I imagine the response was more like a stampede.

>Neons are somewhat scarce in some parts of the country

>but the major problem is most people just wont go out

>and look. They call a couple nearby places and give up.

>

>I scoured every sign place within a 75 mile radius of

>me to collect the ones that I have.

>

>I find that most sign places that do neon work are more

>than happy to supply used and/or dead neons once they

>see what you are going to use them for. Take a long a

>couple pictures to generate interest.

>

>Our European friends have a more difficult time. Due

>to strict regulations appropriate neon transformers

>just do not exist over there.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

Well it is true NST's do seem hard to find, and as you may know if you

have talked to me in IRC. Ive had a hellova' time geting one myself, but

saying that the NST-less dont look is wrong. I call my local shops every

Monday and still have not found a single one. All they have is the

switching type that are useless...

Philip Mac Duffie

From: Dsurfr@aol.com

Date: Sat, 7 Nov 1998 13:17:40 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: "pressed" caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: I'm in the process of building stacked plate caps using multiple

sheets of .006" poly & wonder if there would be any advantage to trying to put

them in a press while soaking them in mineral oil. The local cider mills are

closing down for the season & I have one that would be willing to let me try

to come up with some sort of device that would let me try this with their

apple pressing equipment.

Would this improve the "quality" of the cap by removing air trapped between

the layers of poly (in lieu of vacuuming) or would the pressure create even

more trapped air pockets? It seems like the closer the layers of dielectric &

foil are sandwiched the better the cap value would be but I don't have any

real advice on this idea. Thanks,  Jim

Date: Sat, 07 Nov 1998 12:34:03 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: "pressed" caps

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Hi All: I'm in the process of building stacked plate caps using multiple

> sheets of .006" poly & wonder if there would be any advantage to trying to put

> them in a press while soaking them in mineral oil. The local cider mills are

> closing down for the season & I have one that would be willing to let me try

> to come up with some sort of device that would let me try this with their

> apple pressing equipment.

> Would this improve the "quality" of the cap by removing air trapped between

> the layers of poly (in lieu of vacuuming) or would the pressure create even

> more trapped air pockets? It seems like the closer the layers of dielectric &

> foil are sandwiched the better the cap value would be but I don't have any

> real advice on this idea. Thanks,  Jim

Jim,

This is kind of a poser, Jim! If you could squeeze the stack while it's

immersed in oil, much of the air should be squeezed out, so that when

you then released the pressure, the oil should wick back into the cap.

If you then could keep the stack vertical while running it, any left

over bubbles will hopefully work their way up and out.

I wonder what mineral oil-laced cider tastes like? Hmm... think I'll

make my hot cider with vodka instead!  :^)

-- Bert --

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 8 Nov 1998 01:16:38 -0600

X-Mailer: IPAD 2.0

X-Mimeole: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

I was wondering if anyone on the list could cross a H439.

This is a bridge rectifier. It's diamentions are 2" square and

1.25" tall including the connectors. Please email me at

choad69@geocities.com if you find this.

Thanks,

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

Date: Sun, 08 Nov 1998 12:48:59 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Transformer cores

Sender: tesl-err@emachine.com

I have been reading some postings from one of the list members in

Australia who has built his own home built pig. I would like to know if

anyone knows were to find large laminated iron cores? This would give

you the flexibility to wind a transformer to whatever voltage that you

like and would be a great learning experiance as well.

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Sun, 08 Nov 1998 13:23:03 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Transformer cores

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:48 PM 11/8/98 -0600, you wrote:

>I have been reading some postings from one of the list members in

>Australia who has built his own home built pig. I would like to know if

>anyone knows were to find large laminated iron cores? This would give

>you the flexibility to wind a transformer to whatever voltage that you

>like and would be a great learning experiance as well.

>

>

>                                  Frankensteins Helper

>                                          Max

Roderick,

The best source of suitable transformer cores is

from... old transformers. Ask around. Large control

or isolation transformers show up in surplus rather often

and you can use the existing primary and wind a new secondary.

I acquired a PT core from a friend that looks to be good

for at least 3 KVA. I plan to wind two series connected

7200 V secondary windings. Like a big neon. The design Ive

worked out comes to 200ma at 14400 volts. This is quite

conservative.

Erik Schulz also built his own transformer and helped me

with my design. Check out his page at: http://members.aol.com/tesla00/o_index.htm

Erik has pictures and specs and example math on his

transformer project.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 08 Nov 1998 17:20:24 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Transformer cores

Sender: tesl-err@emachine.com

> Roderick,

>

> The best source of suitable transformer cores is

> from... old transformers. Ask around. Large control

> or isolation transformers show up in surplus rather often

> and you can use the existing primary and wind a new secondary.

>

> I acquired a PT core from a friend that looks to be good

> for at least 3 KVA. I plan to wind two series connected

> 7200 V secondary windings. Like a big neon. The design Ive

> worked out comes to 200ma at 14400 volts. This is quite

> conservative.

>

> Erik Schulz also built his own transformer and helped me

> with my design. Check out his page at: http://members.aol.com/tesla00/o_index.htm

>

> Erik has pictures and specs and example math on his

> transformer project.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Yes I have seen Erik's web page. I'm always looking around for someone

elses ideas to steal! But unfortunately in the area were I live surplus

equipment is rather hard to find. The only option I have is to find a

site on the web that sells surplus electrical equipment. This is the

website that got my motors running in this direction.

http://www.cobweb.com.au/~dkfinnis/tf_rebuild.htm

Frankensteins Helper

Max

X-Originating-IP: [216.68.39.84]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Transformer cores

Date: Sun, 08 Nov 1998 18:40:02 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>  Yes I have seen Erik's web page. I'm always looking around for someone

>elses ideas to steal! But unfortunately in the area were I live surplus

>equipment is rather hard to find. The only option I have is to find a

>site on the web that sells surplus electrical eqiupment. This is the

>website that got my motors running in this direction.

Well if you are really desperate - I'm told you can use old (dead)

motors.  The outer housing is usually made of laminations bolted

together.  Rip out the armature and any field windings, and you are left

with an unusually-shaped laminated transformer core!  One buddy make a

welding smoothing inductor this way...

-Bill

Date: Sun, 08 Nov 1998 21:52:45 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Transformer cores

Sender: tesl-err@emachine.com

> Well if you are really desperate - I'm told you can use old (dead)

> motors.  The outer housing is usually made of laminations bolted

> together.  Rip out the armature and any field windings, and you are left

> with an unusually-shaped laminated transformer core!  One buddy make a

> welding smoothing inductor this way...

>

> -Bill

>

That sounds interesting. Kind of hard to wind though.

Frankensteins Helper

Max

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: UK : potential transformers etc. in auction

Date: Mon, 09 Nov 1998 09:48:55 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Extract from advertisment in industrial Exchange & Mart - transformer

manufacturer in liquidation :

TEST EQUIPMENT:1MVA 11KV Step Down Transformer 3x Main Test panels,

Current a Voltage

Test Transformers, Resistance Load banks, Variable & Measuring

Transformers, 300 & 400 Cycle Generators, 240 & 415V Flashing

Transformers, SMITH HOBSON Current & potential Transformers,TINSLEY

'4167D' Digital Transformer Ratiometer (L'5892' Digital Micro-ohmeter,

CROPICO'KBS' Resistance Bridge, Oscilloscopes, Insulation Testers,

Resistance Boxes, Clamp Meters Etc., Etc.

<snipped - coil winding equipment, general machine tools, office,

canteen & computer equipment)>

Viewing Mon 16th Nov 930-430 & morning of sale from 9AM

Sale starts 10:30 Tues 17th Nov.

at : Allenwest Brentford Ltd. Unit A Site C Manor Royal Ind.Est.

Crawley, W.Sussex.

Catalogues/enquiries: Wyles Hardy &co.

Ley Hill Rd, Bovingdon, Hemel Hempstead, Herts HP30 0NW

tel 01442 832234 fax 01442 834342

I'll request a catalogue &  may go if I have time - email me if interested

Date: Mon, 9 Nov 1998 22:16:00 +0000

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Glues

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

It's been kinda quiet on the list lately so I thought I'd post a

little bit about some glue testing I did. I just got hold of some 160mm

PVC tubing and spent half an hour in front of the TV, cutting the ends

square at the required length. I didn't know what glue to use for the

acrylic end discs so I did some tests. I glued sample pieces of PVC

tubing and acrylic sheet (all cleaned with alcohol) together, using the

following:-

1)  General purpose 5 minute Epoxy.

2)  RTV Silicone Rubber sealant.

3)  PVC Pipe Weld (a tube of the stuff used for PVC plumbing).

Results:

1)  The Epoxy was quick but did not hold onto the acrylic very well.

It would seperate from the parts if the joint was flexed. It is

good when used to seal a chamber as there are no volatile solvent

fumes to worry about.

2)  The RTV held on OK but was not very strong. Not sure whether the

acetic acid fumes would be a problem.

3)  The PVC glue dissolved the surface of the PVC tube and bonded

surprisingly well to the acrylic. Once set the bond was very strong,

allowing the joint to be flexed. Solvent based so it produces highly

flammable fumes (OK for secondary forms open at bottom end).

I know which one I'll be using,  number 3!

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Bringing Tesla Excitement to Swindon, England

the land of 240VAC @ 50Hz

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

Sender: tesl-err@emachine.com

Date: Mon, 09 Nov 1998 11:05:02 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Glues

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> 3)  The PVC glue dissolved the surface of the PVC tube and smelled

>     surprisingly good. Once the buzz was strong enough,

>     the joints to be over flexed. Solvent based so it produces a

> I know which one I'll be using,  number 3!

Hey bud,  you're having too much fun with this stuff. ;)

-tom

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Transformer cores

Date: Mon, 09 Nov 1998 06:21:04 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> Rip out the armature and any field windings, and you are left

>> with an unusually-shaped laminated transformer core!  One buddy make a

>> welding smoothing inductor this way...

>>

>> -Bill

>>

Then Max wrote:

>  That sounds interesting. Kind of hard to wind though.

True.  Note that I never said it was a *good* idea. :)

-Bill

X-Originating-IP: [216.68.40.80]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Glues

Date: Mon, 09 Nov 1998 18:49:05 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Stephen wrote:

>    allowing the joint to be flexed. Solvent based so it produces highly

>    flammable fumes (OK for secondary forms open at bottom end).

>

>I know which one I'll be using,  number 3!

Because of the flammable fumes, I assume.  :)

-Bill

From: Hollmike@aol.com

Date: Tue, 10 Nov 1998 07:55:05 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Glues

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Stephen,

I like using hot melt glue.  No solvent vapors to deal with.  It works well

for pvc, but I don't know about pvc to acrylic, but I don't see why it

wouldn't hold. I will have to try that.

Mike

Date: Tue, 10 Nov 1998 20:47:33 +0000

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Re: Glues

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In message <45a1c4a7.364837a9@aol.com>, Hollmike@aol.com writes

>Stephen,

>   I like using hot melt glue.  No solvent vapors to deal with.  It works well

>for pvc, but I don't know about pvc to acrylic, but I don't see why it

>wouldn't hold. I will have to try that.

>Mike

Mike,

Thanks for the reminder.  I just remembered I've got a hot glue

gun too!......Somewhere!?   }:-<

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Bringing Tesla Excitement to Swindon, England

the land of 240VAC @ 50Hz

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

Date: Tue, 10 Nov 1998 16:14:17 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: PT transformer

Sender: tesl-err@emachine.com

Here is a picture of 3 PT transformers I found while I was cruising

around on the web looking for surplus!! The voltage is too high and the

price of them were way out of my league!

Frankensteins Helper

Max

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\60kvpt~1.jpg"

(editors note. as usual no binary attachments in the archive...sorry)

From: Hollmike@aol.com

Date: Wed, 11 Nov 1998 08:31:33 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Colorado coilers get-together

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

This weekend, several coilers in the  Denver area are getting together to

plan the construction of a  magnifier.   If you are interested in joining the

fun, let me know.

Mike Hollingsworth

X-Mailer: IPAD 2.0

Date: Wed, 11 Nov 1998 12:08:37 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Glues

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:55 AM 11/10/98 EST, you wrote:

>Stephen,

>   I like using hot melt glue.  No solvent vapors to deal with.  It works well

>for pvc, but I don't know about pvc to acrylic, but I don't see why it

>wouldn't hold. I will have to try that.

>Mike

The only problem I had using hot glue was that it tended to debond

from whatever it was stuck to when placed in mineral oil.

I used hot glue to seal up some seams in a cap tank I once built from

plexiglass. Once I filled it with mineral oil all the hot glue came loose

and floated to the top.

TTYL!

Mike Hammer

mhammer@misslink.net

From: EdwMullin@aol.com

Date: Wed, 11 Nov 1998 17:33:28 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: PT transformer

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

just what is a PT transformer?  They look really cool though, too bad you

can't use them!!

From: EdwMullin@aol.com

Date: Wed, 11 Nov 1998 17:36:34 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Magnifiers?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've seen magnifiers mentioned on the list a few times, but I don't know

exactly what the are or how they work, anyone care to go into a not too

technical explanation?

X-Mailer: IPAD 2.0

Date: Wed, 11 Nov 1998 16:48:44 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: PT transformer

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:33 PM 11/11/98 EST, you wrote:

>just what is a PT transformer?  They look really cool though, too bad you

>can't use them!!

>

PT stands for potential transformer. These are shell

wound normally dry transformers similar to pole pigs

but much smaller. They are normally rated at 1.5 KVA

and are used by the power companies to step down some

power for special uses like running cooling fans or

metering purposes.

They are not current limited and require extensive

ballasting and control circuitry just as pole pigs do.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Hollmike@aol.com

Date: Wed, 11 Nov 1998 20:50:47 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Colorado coilers get-together

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/11/98 4:21:19 PM Mountain Standard Time, molly@cmn.net

writes:

>

>  Oh! How 'bout that! I would like to make it, but there's no way. I haven't

>  had enough time to coil for a while now. :-(

>

>  Is there anybody else from Colorado here on this list? I live in Steamboat

>  Springs.

>

>  Daniel Lockhart

>

Daniel,

We will be getting together about once a month.  I will let you know of

future meetings, if you would like, in case you can manage to get away.

Mike

From: Hollmike@aol.com

Date: Wed, 11 Nov 1998 21:14:39 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Glues

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/11/98 11:11:35 AM Mountain Standard Time,

mhammer@misslink.net writes:

>

>  The only problem I had using hot glue was that

>  it tended to debond from whatever it was stuck to

>  when placed in mineral oil.

>

>  I used hot glue to seal up some seams in a cap

>  tank I once built from plexiglass. Once I filled

>  it with mineral oil all the hot glue came loose

>  and floated to the top.

>

Good advice.  I never tried it for that application.

Mike Hollingsworth.

From: Hollmike@aol.com

Date: Wed, 11 Nov 1998 21:34:40 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: PT transformer

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/11/98 3:50:58 PM Mountain Standard Time,

mhammer@misslink.net writes:

>  They are normally rated at 1.5 KVA

>  and are used by the power companies to step down some

>  power for special uses like running cooling fans or

>  metering purposes.

I think that they are actually used to meter the power used on very high power

systems.  At least that is what the chief electrician told me when he gave me

a couple of them.

Mike

Date: Wed, 11 Nov 1998 20:54:43 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: PT transformer

Sender: tesl-err@emachine.com

Hollmike@aol.com wrote:

>

> In a message dated 11/11/98 3:50:58 PM Mountain Standard Time,

> mhammer@misslink.net writes:

>

> >  They are normally rated at 1.5 KVA

> >  and are used by the power companies to step down some

> >  power for special uses like running cooling fans or

> >  metering purposes.

> I think that they are actually used to meter the power used on very high power

> systems.  At least that is what the chief electrician told me when he gave me

> a couple of them.

> Mike

These three are rated at 4Kva. Voltage is 64Kv, and the price for them

is 2,500 smackeroos each!

Frankensteins Helper

Max

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: PT transformer

Date: Thu, 12 Nov 1998 08:22:33 -0800

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A while back I had a chance to pick up some of these for 10 cents a pound

but 64KV is way too high to be of any use to me.

Brian D. Basura

-----Original Message-----

From: RODERICK MAXWELL [mailto:tank@magnolia.net]

Sent: Wednesday, November 11, 1998 6:55 PM

To: Tesla-2 Mailing List

Subject: Re: PT transformer

Hollmike@aol.com wrote:

>

> In a message dated 11/11/98 3:50:58 PM Mountain Standard Time,

> mhammer@misslink.net writes:

>

> >  They are normally rated at 1.5 KVA

> >  and are used by the power companies to step down some

> >  power for special uses like running cooling fans or

> >  metering purposes.

> I think that they are actually used to meter the power used on very high power

> systems.  At least that is what the chief electrician told me when he gave me

> a couple of them.

> Mike

Date: Thu, 12 Nov 1998 20:53:53 +0000

From: Ross Ogden <Rossoggy@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: UK : potential transformers etc. in auction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello Mike

Im in the uk too could you tell me where i could get a catalogue from.

Mike Harrison wrote:

> Extract from advertisment in industrial Exchange & Mart - transformer

> manufacturer in liquidation :

>

> TEST EQUIPMENT:1MVA 11KV Step Down Transformer 3x Main Test panels,

> Current a Voltage

> Test Transformers, Resistance Load banks, Variable & Measuring

> Transformers, 300 & 400 Cycle Generators, 240 & 415V Flashing

> Transformers, SMITH HOBSON Current & potential Transformers,TINSLEY

> '4167D' Digital Transformer Ratiometer (L'5892' Digital Micro-ohmeter,

> CROPICO'KBS' Resistance Bridge, Oscilloscopes, Insulation Testers,

> Resistance Boxes, Clamp Meters Etc., Etc.

> <snipped - coil winding equipment, general machine tools, office,

> canteen & computer equipment)>

>

> Viewing Mon 16th Nov 930-430 & morning of sale from 9AM

> Sale starts 10:30 Tues 17th Nov.

> at : Allenwest Brentford Ltd. Unit A Site C Manor Royal Ind.Est.

> Crawley, W.Sussex.

> Catalogues/enquiries: Wyles Hardy &co.

> Ley Hill Rd, Bovingdon, Hemel Hempstead, Herts HP30 0NW

> tel 01442 832234 fax 01442 834342

>

> I'll request a catalogue &  may go if I have time - email me if

> interested

X-Originating-IP: [207.100.20.22]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tube Coils

Date: Thu, 12 Nov 1998 19:47:01 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello everyone, I am looking for as much info and pictures as i can dig

up on Tube coils.. Ive recently become intresterd in tubes themselves

and since my high school doesnt teach tube use ive decided to try to

teach myself.

Thank You,

Philip Mac Duffie

P.S looks like coil might be ready to light soon, thanks.

http://hv.hypermart.net

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: UK : potential transformers etc. in auction

Date: Fri, 13 Nov 1998 10:48:19 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Thu, 12 Nov 1998 20:53:53 +0000, you wrote:

>Hello Mike

>

>Im in the uk too could you tell me where i could get a catalogue from.

Try reading the message - phone no. for catalogue details was included!

I've  got a catalogue - not really much of interest (except a 1MVA

11KV unit - perhaps a little on the large side!) - just a few variacs,

but plenty of stuff for building your own pigs! The catalogue mentions

a stock of transformers not included in the auction, to be sold by

private treaty (no list in catalogue) - it could be worth turning up

and making an offer if there's anything suitable. At factory clearance

type auctions, you sometimes find interesting small odds & ends that

they couldn't be bothered to catalogue, that can be had for the asking

>

>

>Mike Harrison wrote:

>

>> Extract from advertisment in industrial Exchange & Mart - transformer

>> manufacturer in liquidation :

>>

>> TEST EQUIPMENT:1MVA 11KV Step Down Transformer 3x Main Test panels,

>> Current a Voltage

>> Test Transformers, Resistance Load banks, Variable & Measuring

>> Transformers, 300 & 400 Cycle Generators, 240 & 415V Flashing

>> Transformers, SMITH HOBSON Current & potential Transformers,TINSLEY

>> '4167D' Digital Transformer Ratiometer (L'5892' Digital Micro-ohmeter,

>> CROPICO'KBS' Resistance Bridge, Oscilloscopes, Insulation Testers,

>> Resistance Boxes, Clamp Meters Etc., Etc.

>> <snipped - coil winding equipment, general machine tools, office,

>> canteen & computer equipment)>

>>

>> Viewing Mon 16th Nov 930-430 & morning of sale from 9AM

>> Sale starts 10:30 Tues 17th Nov.

>> at : Allenwest Brentford Ltd. Unit A Site C Manor Royal Ind.Est.

>> Crawley, W.Sussex.

>> Catalogues/enquiries: Wyles Hardy &co.

>> Ley Hill Rd, Bovingdon, Hemel Hempstead, Herts HP30 0NW

>> tel 01442 832234 fax 01442 834342

>>

>> I'll request a catalogue &  may go if I have time - email me if

>> interested

>

>

>

X-Originating-IP: [207.100.20.19]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: FIRST LIGHT SORTA

Date: Fri, 13 Nov 1998 19:25:03 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I am so close, yet so far, in the reach of the heavens, but still in the

hole, I canot see, But the light is blinding.

Well all if you heve been reading my progress you will know that tonight

was THE night, but the only problem was that it didnt happen. I was

finishing the NST tank i had it all together and it looked good, I

filled the tank and was dreaming of first light, but again desaster

struck. I hooked up the nst to test out its new home I also added

monster ammounts of insulation to my NST (everywhere except the shunts)

then i wired it up and fired it. I mounted the HV bolts 4" appart and i

set op the jacobs ladder test configuration. When i did it ran about 10

arcs up my 1' ladder then stopped made a odd buzz and some came bilowing

out from under the lid. There was a awfull smell of burt wood products.

So in my histeria I ripped the lid off only to find smoke filled oil! I

pulled the nst out and started feeling for warmth in the coils and

core... Ice cold. I then brushed my forearm across a neuclear hot HV

terminal. After the appropriate screm from burining flesh. I looked

around at what the hell was the matter. I plugend my seemingly doomed

NST in to 110 and listened. There was the humm the nst makes in short

circuit mode. I looked at all concections on my NST and the were all

insulated and connected well. So i foucused my attention on the source

of heat in my HV bolts. Come to find out that my little nst (9kv 30ma)

burned a hole through the plywood lid to the other hv terminal. It read

110ohms on the resistance meter. So? where do i go from here? will a

lexan ,2 peices 1/4", lid work?

Thanks,

Philip Mac Duffie

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ok.. where am I going wrong

Date: Fri, 13 Nov 1998 22:25:52 -0800

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

I just finished wiring up my pig-training facility. This means the

welder, 0.4 ohm resistor and 240V feed. I had thought to calibrate the

switch settings on the welder to KVA numbers by taking voltage readings

across the resistor. The infamous V=IR business. Only.. it didn't work.

Or I don't think it did. At one point, I measured a drop of 17V. This works

out to roughly 42 amps. This cannot be. I have 30A fast blow fuses in the line

and I suspect that they would fry in short order at almost 50% overload.

Therefore, there must be something I am missing. I assume it is related to the

fact that this is AC.

Could someone help me out here? Tell me where I goofed.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

From: Hollmike@aol.com

Date: Sat, 14 Nov 1998 06:00:25 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: FIRST LIGHT SORTA

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/13/98 8:26:35 PM Mountain Standard Time,

punk_42@hotmail.com writes:

>  So? where do i go from here? will a lexan ,2 pieces 1/4", lid work?

>   Thanks,

>  Philip Mac Duffie

That should do much better than wood.  Wood is not particularly good for HV

insulation.

Mike

From: ELECTRONGENIE@prodigy.net

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ok.. where am I going wrong

Date: Sat, 14 Nov 1998 04:38:55 -0700

X-Mailer: IPAD 2.0

X-Mimeole: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

240 VOLTS AC DIVIDED BY .4 OHMS = 600 AMPS  YOU HAVE A DIRECT SHORT!   If

this is a welder you are hooking up it is just a big transformer your

primary should be drawing about 40 amps, your secondary should be

adjustable.  Feel free to contact me.  I'AM AN ELECTRICAN.

-----Original Message-----

From: Coiler <mycroft@access1.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Friday, November 13, 1998 11:33 PM

Subject: Ok.. where am I going wrong

>I just finished wiring up my pig-training facility. This means the

>welder, 0.4 ohm resistor and 240V feed. I had thought to calibrate the

>switch settings on the welder to KVA numbers by taking voltage readings

>across the resistor. The infamous V=IR business. Only.. it didn't work.

>Or I don't think it did. At one point, I measured a drop of 17V. This works

>out to roughly 42 amps. This cannot be. I have 30A fast blow fuses in the line

>and I suspect that they would fry in short order at almost 50% overload.

>Therefor, there must be someting I am missing. I assume it is related to the

>fact that this is AC.

>Could someone help me out here? Tell me where I goofed.

>

>Michael Baumann

>Coiler, Homebrewer, Nerd. mycroft@access1.net

>

>

X-Mailer: IPAD 2.0

Date: Sat, 14 Nov 1998 12:00:38 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: FIRST LIGHT SORTA

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:25 PM 11/13/98 PST, you wrote:

>I am so close, yet so far, in the reach of the heavens, but still in the

>hole, I canot see, But the light is blinding.

>

>Come to find out that my little nst (9kv 30ma) burned a hole

>through the plywood lid to the other hv terminal. It read

>110ohms on the resistance meter. So? where do i go from here?

>Will a lexan ,2 pieces 1/4", lid work?

> Thanks,

>Philip Mac Duffie

>

Philip,

I made the lid for my rebuilt neon as well as the tanks for

my plate caps from plywood and ran the HV terminals through

the wood.

What I did was drill the hole larger than the bolt you are using

and fill the space with RTV silicone. I cut a disk of plexiglass

and attached these to both sides of the hole with RTV.

So the bolt goes through a plexi disk then through the plywood and

then through the second plexi disk and it spaced from the wood

by RTV. Works well and is leakproof once the RTV cures.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 14 Nov 1998 15:17:45 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Magnifiers?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:36 PM 11/11/98 EST, you wrote:

>I've seen magnifiers mentioned on the list a few times, but I don't know

>exactly what the are or how they work, anyone care to go into a not too

>technical explanation?

>

Ed,

A magnifier is a 3 coil variation of the classic two coil

Tesla coil. It has a closely coupled primary/secondary

and the output of this is coupled to the tertiary or

output resonator via a transmission line.

In the standard two coil Tesla coil the energy is coupled

magnetically to the secondary. In the magnifier there is no

magnetic coupling, the energy being base fed into the resonator

via the transmission line.

This means the pri/sec and control equipment can be placed

some distance away from the output and there will be no magnetic

interaction between the pri/sec and the output coil.

There doesnt seem to be much performance advantage in building

a magnifier. They are considerably more difficult than a standard

coil requiring the best of components. You would need a high performance

gap system and the very best caps and a very hardy transformer.

Very few coilers have attempted to build one and those that do are

pretty much on their own.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 14 Nov 1998 17:59:32 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ok.. where am I going wrong

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:25 PM 11/13/98 -0800, you wrote:

>I just finished wiring up my pig-training facility.

Pig training facility. I like that.

>This means the welder, 0.4 ohm resistor and 240V feed. I had thought to

>calibrate the switch settings on the welder to KVA numbers by taking

>voltage readings across the resistor. The infamous V=IR business. Only..

>it didn't work. Or I don't think it did. At one point, I measured a drop of

>17V. This works out to roughly 42 amps. This cannot be. I have 30A fast blow

>fuses in the line and I suspect that they would fry in short order at almost

>50% overload.

>Therefor, there must be something I am missing. I assume it is related to the

>fact that this is AC.

>Could someone help me out here? Tell me where I goofed.

>

>Michael Baumann

>Coiler, Homebrewer, Nerd. mycroft@access1.net

Michael,

Im not sure you have goofed. Your .4 ohm resistor is in series

with the welder is it not? If so the voltage drop across it

will be proportional to the current flowing through it.

Are you ramping up the voltage slowly with a variac? If so that

may be why the fuses are not blowing. Fuses work very fast when

subjected to an instantaneous overload but may pass considerably

more than their rating if you sneak up on it. Especially so with

circuit breakers.

What are you using for a load on the pig? A gap or jacobs ladder

maybe? Or your coil? If its your coil then the pig primary

voltage and current will be quite non sinusoidal. That rather

complicates matters.

TTYL!

Mike Hammer

mhammer@misslink.net

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Ok.. where am I going wrong

Date: Sat, 14 Nov 1998 17:40:36 -0800

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

> Im not sure you have goofed. Your .4 ohm resistor is in series

> with the welder is it not? If so the voltage drop across it

> will be proportional the current flowing through it.

Exactly, this is what I was hoping for. But the numbers don't look right.

>

> Are you ramping up the voltage slowly with a variac? If so that

> may be why the fuses are not blowing. Fuses work very fast when

> subjected to an instantaneous overload but may pass considerably

> more than their rating if you sneak up on it. Especially so with

> circuit breakers.

No variac, I only wish. It'll be a while before the wife lets me drop

any more money into this.. between the pig, and the support stuff, I've

dropped about $600 in the last month. Pushing the limit as it were.

And I am using fuses.

>

> What are you using for a load on the pig? A gap or jacobs ladder

> maybe? Or your coil? If its your coil then the pig primary

> voltage and current will be quite non sinusoidal. That rather

> complicates matters.

I actually dead shorted the pig. As in wire across the output, so I would

not have to deal with the nasty sine stuff. (also using a ladder which does

indeed have a non-sinusodial output)

Thanks for the info tho.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

http://www.access1.net/mycroft

Date: Sat, 14 Nov 1998 21:57:33 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 6" coil

Sender: tesl-err@emachine.com

Would it be okay if I posted a few pics of my six inch coils? One is

built ground up, and the other is retrofit of my first coil which was a 4".

Frankensteins Helper

Max

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Newbie

Date: Sat, 14 Nov 1998 20:44:02 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

Allow me to introduce myself: My name is Jon Rosenstiel and I presently live

in Anaheim, CA. I have had the high voltage bug since the mid 50's,(eons ago),

when I was a young lad. I got started with a model T coil hooked to a train

transformer.  Then I had built a very small Van de Graff generator using a

phonograph motor and a rubber band. The tube was a discarded pill container,

and the hv terminal was a ball of aluminum foil that I had salvaged

from old capacitors.  Since then I have dreamed about building a larger Van de Graff machine, even acquired some plans and the motor, but still no finished product!

For Halloween we always set up a Jacobs ladder,(using a 15kv nst), in the garage

so the trick or treaters can see it through the window on their way to our front door. After this past Halloween the Jacobs ladder got me thinking about Tesla coils and I

decided to see if I could find any information on the web. Boy, was I suprised!

Now I'm gathering all of the information that I can on TC's, and am planning to get

my feet wet. I have already experimented with making flat plate capacitors, and am

going to wind a coil on a wrapping paper cardboard tube which has a diameter of 1.75".

I eventually want to build a Tesla with a secondary coil of around 4 inches and a 60ma

15kv nst.

Jon Rosenstiel  jonr@pacbell.net

Date: Sat, 14 Nov 1998 21:26:34 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

RODERICK MAXWELL wrote:

>

> Would it be okay if I posted a few pics of my six inch coils? One is

> built ground up, and the other is retrofit of my first coil which was a

> 4".

>

>                                Frankensteins Helper

>                                       Max

POST! We need some excitement!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sat, 14 Nov 1998 23:03:39 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> RODERICK MAXWELL wrote:

> >

> > Would it be okay if I posted a few pics of my six inch coils? One is

> > built ground up, and the other is retrofit of my first coil which was a

> > 4".

> >

> >                                Frankensteins Helper

> >                                       Max

> POST! We need some excitement!

>

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\6inch.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\6inch2.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\6inch3.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\6inch4.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\6inch5.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\6inch6.jpg"

(editors note. binary attachments not in archive. sorry)

X-Mailer: IPAD 2.0

Date: Sun, 15 Nov 1998 00:56:08 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:03 PM 11/14/98 -0600, you wrote:

>Gene Waldo wrote:

>>

>> RODERICK MAXWELL wrote:

>> >

>> > Would it be okay if I posted a few pics of my six inch coils? One is

>> > built ground up, and the other is retrofit of my first coil which was a

>> > 4".

>> >

>> >                                Frankensteins Helper

>> >                                       Max

Roderick,

Nice looking coil and nice streamers. Seems to be working

quite well. Is this being powered by your modified H&R

transformers?

Do my eyes deceive me or is your secondary wound with two different

sizes of wire? Or is it just two sections?

TTYL!

Mike Hammer

mhammer@misslink.net

From: Hollmike@aol.com

Date: Sun, 15 Nov 1998 02:36:39 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnifiers?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/14/98 2:20:06 PM Mountain Standard Time,

mhammer@misslink.net writes:

> There doesnt seem to be much performance advantage in building

>  a magnifier. They are considerably more difficult than a standard

>  coil requiring the best of components. You would need a high performance

>  gap system and the very best caps and a very hardy transformer.

>  Very few coilers have attempted to build one and those that do are

>  pretty much on their own.

>

>  TTYL!

>

>  Mike Hammer

>

It is funny that this came up just now.  Several coilers in the Denver

area got together yesterday to try putting together a magnifier.  It was

assembled from components brought in by several in the group.  We did put it

together with a hardy power supply and caps.  Since we also had a signal

generator and o-scope, it was able to be tuned very quickly.  We fired it up

in about an hour after starting.  We got about 3 foot sparks on the first

firing and after a few adjustments, were getting sparks in excess of four

feet.

Mike is right that there is probably no performance advantage to

building a magnifier.  A two coil system could do just as well with the power

input that we had.  There does seem to be one advantage to a magnifier though.

You don't have to worry about overcoupling the resonator and causing arcing

down the windings.   There is also no worry about the discharge striking the

primary coil.  We did have one strike to the transmission line that caused a

surge back through the gap, causing it to short circuit most of the gaps with

a loud pop(no damage though).  Once that problem was corrected, we had a good

time making sparks :)

Mike Hollingsworth

Date: Sun, 15 Nov 1998 06:52:07 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

>

> Roderick,

>

> Nice looking coil and nice streamers. Seems to be working

> quite well. Is this being powered by your modified H&R

> transformers?

>

> Do my eyes deceive me or is your secondary wound with two different

> sizes of wire? Or is it just two sections?

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

The first coil is being driven by the H&R transformers. It is wound

with 21 guage mag wire and has a flat primary.

The next coil is being driven by my 12Kv,180ma. supply. It is wound

at the bottom with 16 guage mag wire, then the rest of the distance to

the top of the form with 21 guage mag wire. I did this to obtain higher

Q.

Frankensteins Helper

Max

Date: Sun, 15 Nov 1998 08:42:14 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

RODERICK MAXWELL wrote:

<SNIP>

>

>    The first coil is being driven by the H&R transformers. It is wound

> with 21 guage mag wire and has a flat primary.

>    The next coil is being driven by my 12Kv,180ma. supply. It is wound

> at the bottom with 16 guage mag wire, then the rest of the distance to

> the top of the form with 21 guage mag wire. I did this to obtain higher Q.

>

>                                Frankensteins Helper

>                                       Max

Roderick,

Nice looking systems! Did you make any comparison measurements of the

Q's between the two secondaries? Using two wire gauges with the larger

wire at the bottom is an interesting approach to reducing secondary

losses, since the RF current (and I^2R/skin effect losses) are highest

at the base of the secondary.

-- Bert --

Date: Sun, 15 Nov 1998 14:25:23 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6" coil

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> RODERICK MAXWELL wrote:

> <SNIP>

> >

> >    The first coil is being driven by the H&R transformers. It is wound

> > with 21 guage mag wire and has a flat primary.

> >    The next coil is being driven by my 12Kv,180ma. supply. It is wound

> > at the bottom with 16 guage mag wire, then the rest of the distance to

> > the top of the form with 21 guage mag wire. I did this to obtain higher

> > Q.

> >

> >                                Frankensteins Helper

> >                                       Max

>

> Roderick,

>

> Nice looking systems! Did you make any comparison measurements of the

> Q's between the two secondaries? Using two wire gauges with the larger

> wire at the bottom is an interesting approach to reducing secondary

> losses, since the RF current (and I^2R/skin effect losses) are highest

> at the base of the secondary.

>

> -- Bert --

No I have not done any comparison tests. With the first system (the

one with the flat pancake primary) I'm using H&R transformers to drive

it. I have not run it at full power yet for three reasons.

1. The primary tank cap has not been fully broken in yet.

2. I'm still feeling my way around with the H&Rs as to the

modifications that I have made.

3. The vaccuum gap is not quenching well. I am trying to finish the

rotary gap I started on last summer. I plan to put this gap in series

with the vaccuum gap, this will allow me to quench the high currents in

the gap created by the unshunted supply.

Frankensteins Helper

Max

From: Dsurfr@aol.com

Date: Sun, 15 Nov 1998 19:06:47 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Fried Neon???

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All. I fear I may have fried my 15kV 30mA Franceformer. I was getting about

18" to 20" arcs & then added another, smaller toroid on top of the first. The

coil ran great then quit abruptly. I can only get an arc from one of the

terminals to the casing but connecting it to a Jacob's ladder produces a

healthy arc between the rods. The coil will run (not as well as before) for

brief spans & then cut on & off. Before I start calling all the neon shops &

maybe make an attempt at repotting this one I need some advice. Both terminals

should arc to case - correct? I'm probably running off 7.5kV now?  Thanks, Jim

X-Mailer: IPAD 2.0

Date: Sun, 15 Nov 1998 18:43:55 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Fried Neon???

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:06 PM 11/15/98 EST, you wrote:

>Hi All. I fear I may have fried my 15kV 30mA Franceformer. I was getting about

>18" to 20" arcs & then added another, smaller toroid on top of the first. The

>coil ran great then quit abruptly. I can only get an arc from one of the

>terminals to the casing but connecting it to a Jacob's ladder produces a

>healthy arc between the rods. The coil will run (not as well as before) for

>brief spans & then cut on & off. Before I start calling all the neon shops &

>maybe make an attempt at repotting this one I need some advice. Both terminals

>should arc to case - correct? I'm probably running off 7.5kV now?  Thanks, Jim

Jim,

Doesnt sound too encouraging. Both terminals should offer

a nice healthy arc to the case. One side sounds dead.

Take a resistance reading and the dead side will probably

show much lower resistance than the other. This is a result

of a carbon track short.

Now there are two things you can do. First you can bake

the neon until the tar liquifies. This will many times

cure a carbon track short and restore operation.

Second is you can unpot and rebuild the neon. Franceformers

are the best for rebuilding. You can also remove a couple shunts

and boost output to 40-45 ma. This is what I would do. You will

end up with a transformer that is better than new.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 15 Nov 1998 19:22:35 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Fried Neon???

Sender: tesl-err@emachine.com

Mike Hammer wrote:

> Franceformers are the best for rebuilding. You can also remove a couple shunts

> and boost output to 40-45 ma.

Does a couple shunts mean a couple of those thin pieces of metal? How

thick should the stack be that is left in the transformer? What effect

is there if you remove all the shunts?

X-Mailer: IPAD 2.0

Date: Sun, 15 Nov 1998 19:36:42 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Fried Neon???

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:22 PM 11/15/98 -0600, you wrote:

>Mike Hammer wrote:

>> Franceformers are the best for rebuilding. You can also remove a couple shunts

>> and boost output to 40-45 ma.

>

>Does a couple shunts mean a couple of those thin pieces of metal?

Yes. The shunts are metal blocks placed in between the primary

and secondaries to divert the magnetic flux. Only enough gets

through to cause the rated current to flow.

The blocks are made up of a stack of thin plates. Removing a few

plates allows more of the magnetic flux to reach the secondaries

allowing greater current flow.

>How thick should the stack be that is left in the transformer?

This varies widely from unit to unit and from manufacturer to

manufacturer.

>What effect is there if you remove all the shunts?

The transformer will likely go into saturation and

the windings will overheat. Smoke will follow soon after.

For 30ma neons you can remove up to 4 or 5 plates from each

shunt block and boost output to 40-45 ma. This is about all

that they can handle. A 60 ma neon is a little more robust

and can be pushed up to 90-100ma.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr@aol.com

Date: Sun, 15 Nov 1998 20:43:27 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Baking Neons

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: thanks as always for the reply. I was pretty sure I had shorted out one

side of the neon but wanted to check. You & Malcolm warned me that this was

likely with my coil a while back, but I guess this is just another phase of

coiling I'll have to adjust to. I'd like to try your first suggestion before I

go through the hassle of repotting:

"Now there are two things you can do. First you can bake

the neon until the tar liquifies. This will many times

cure a carbon track short and restore operation."

Would you elaborate a bit on this? What temperature, how long etc.

In the meantime I'm going to reread your post on repotting your neon & hit the

local sign shops. Apart from the obvious hazards coiling is also very

addictive. I won't last long without the sparks.

Thanks,  Jim

X-Mailer: IPAD 2.0

Date: Sun, 15 Nov 1998 22:05:56 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:43 PM 11/15/98 EST, you wrote:

>Mike: thanks as always for the reply. I was pretty sure I had shorted out one

>side of the neon but wanted to check. You & Malcolm warned me that this was

>likely with my coil a while back, but I guess this is just another phase of

>coiling I'll have to adjust to. I'd like to try your first suggestion before I

>go through the hassle of repotting:

>

>"Now there are two things you can do. First you can bake

>the neon until the tar liquifies. This will many times

>cure a carbon track short and restore operation."

>

>Would you elaborate a bit on this? What temperature, how long etc.

>In the meantime I'm going to reread your post on repotting your neon & hit the

>local sign shops. Apart from the obvious hazards coiling is also very

>addictive. I won't last long without the sparks.

>            Thanks,  Jim

I dont really recommend the procedure but some have used

it with success. The temperature depends on the potting

compound and this varies widely from unit to unit.

You probably need close to 300 degrees to melt the tar

completely but use only as much heat as necessary.

If you plan on unpotting and rebuilding I would not

bake it. That will make it harder to unpot.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 15 Nov 1998 21:00:41 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: winter conditions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I guess I havent been posting lately because of my pyrotechnics hobby.

Would there be any difference in performance if I ran my Tcoil in sub

zero temperatures? What should I use to coat a cardboard coil form?

Anyone interested in my crazy carpet capacitor plans?

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Sun, 15 Nov 1998 22:38:35 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: winter conditions

Sender: tesl-err@emachine.com

Mike N. wrote:

>

> I guess I havent been posting lately because of my pyrotechnics hobby.

>

> Would there be any difference in performance if I ran my Tcoil in sub

> zero temperatures? What should I use to coat a cardboard coil form?

>

> Anyone interested in my crazy carpet capacitor plans?

>

> --

> Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

>

> Sonic Boom

Mike,

I keep my coil on an unheated closed-in patio. It seems to run much

better in the cold and dry conditions of winter. HJowever, watch out for

sudden temperature changes where you may get condensation on the coil or

spark gap, or other parts of your system...

-- Bert --

Date: Sun, 15 Nov 1998 22:57:07 -0700

From: Dan Olsen <dan@olsen.org>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

<snip>

> >"Now there are two things you can do. First you can bake

> >the neon until the tar liquifies. This will many times

> >cure a carbon track short and restore operation."

<snip>

> You probably need close to 300 degrees to melt the tar

> completely but use only as much heat as necessary.

For my Franceformer 30mA I found that about 240 degrees was hot enough to fix my

carbon track and much hotter than that starts the bubbling action which makes a big

mess, they key is to give it a couple of hours to heat through at a low-enough

temperature that none of it bubbles..

> If you plan on unpotting and rebuilding I would not

> bake it. That will make it harder to unpot.

I really wish somebody had told me that earlier!!  I figured I would probably

un-pot mine eventually, but thought a quick bake would get me back in action

quickly (which it did, but it made un-potting much more difficult than it might

have been)  I hear the cold-cracking method works pretty well, and I can testify

that "melting-off" the tar is really messy and smelly.

Good Luck.

Date: Sun, 15 Nov 1998 23:04:19 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: winter conditions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Mike N. wrote:

>>

>> I guess I havent been posting lately because of my pyrotechnics hobby.

>>

>> Would there be any difference in performance if I ran my Tcoil in sub

>> zero temperatures? What should I use to coat a cardboard coil form?

>>

>> Anyone interested in my crazy carpet capacitor plans?

...and Burt wrote:

>I keep my coil on an unheated closed-in patio. It seems to run much

>better in the cold and dry conditions of winter. HJowever, watch out for

>sudden temperature changes where you may get condensation on the coil or

>spark gap, or other parts of your system...

When I ran my coil last it was in an unheated garage. The temperature

wasn't that cold -- maybe forty degrees F at the least. There was a

noticeable difference, although not huge, as the temperature change was

maybe only twenty-or-so-degrees. I'm looking forward to testing the coil

outside some cold January night where it will (hopefully) be below zero.

I'll report my results then.

Tell me more about your crazy carpet capacitor plans. They use rollable

plastic sleds, right? Do the plans cost anything?

Daniel Lockhart

Date: Mon, 16 Nov 1998 09:43:26 -0800

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >Would you elaborate a bit on this? What temperature, how long etc.

> >In the meantime I'm going to reread your post on repotting your neon & hit the

> >local sign shops. Apart from the obvious hazards coiling is also very

> >addictive. I won't last long without the sparks.

> >            Thanks,  Jim

Hi, I'll save you some time here.  I tried several times to bake a neon, and ended

up with a messy non-working neon.  I left it in the oven for hours, gradually

increasing the temerature, and had no success actually getting the tar to melt.

When I finally did get it to melt, it melted at the bottom first, and ended up

getting a little too hot and oozing out all over the tray.  Even after this, it

didn't work.

I've decided that my next attempt at unpotting a neon will be to pull the top off

of the neon, and set it upside down on a metal rack which is submerged in

turpentine.  My thinking here is that gravity will cause all of the disolved tar to

fall to the bottom of the bucket, and eventually I'll have a nice shiny transformer

sitting on the rack.  Sure it'll take a couple of weeks, but this thing has just

been sitting in the garage anyway waiting for the day I get around to trying to

chip the tar out.......If I'd soaked it a month ago, I'd be done by now.

cya!

-tom

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 16 Nov 1998 04:00:50 EST

Subject: Re: Baking Neons

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Mon, 16 Nov 1998 09:43:26 -0800 Tomstock

<tomstock@geocities.com> wrote:

>Hi, I'll save you some time here.  I tried several times to bake a

>neon, and ended up with a messy non-working neon.  I left it in the

>oven for hours, gradually increasing the temerature, and had no

>success actually getting the tar to melt. When I finally did get it

>to melt, it melted at the bottom first, and ended up getting a little

>too hot and oozing out all over the tray.  Even after this, it didn't

>work.

I'll agree, cooking neon sign transformers seems to do more harm than

good, besides it really smells bad!

>I've decided that my next attempt at unpotting a neon will be to pull

>the top off of the neon, and set it upside down on a metal rack which

>is submerged in turpentine.

WOW, what a waste of good solvent! For most of us it is getting good and

cold outside, why not use it to ones advantage. Put the transformer

outside where it is protected from rain and snow and allow it to

literally chill. When you are sure the transformer is cold all the way

to the core, unscrew the top and using vice grips in a rolling motion peel

off the metal casing. BE CAREFUL NOT TO DAMAGE THE HIGH VOLTAGE INSULATORS!

They are held in by clips. Once the transformer and it's pitch coating

are free of the metal shell give it a few wacks with a wooden mallet,

this will remove the vast majority of the pitch and THEN put the

remaining core and exposed windings in the solvent. Not only will you

use much less solvent but you'll get the whole deal cleaner. To prove

that I practice what I preach, right now I have 6 - 15 KV 30 ma. neon

sign transformers and another 6 - 9 KV 30 ma neon sign transformers

chilling on my side porch, soon to be all unpotted.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

X-Mailer: IPAD 2.0

Date: Mon, 16 Nov 1998 10:46:58 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:57 PM 11/15/98 -0700, you wrote:

<SNIP

>> If you plan on unpotting and rebuilding I would not

>> bake it. That will make it harder to unpot.

>

>I really wish somebody had told me that earlier!!  I figured I would probably

>un-pot mine eventually, but thought a quick bake would get me back in action

>quickly (which it did, but it made un-potting much more difficult than It might

>have been)  I hear the cold-cracking method works pretty well, and I can testify

>that "melting-off" the tar is really messy and smelly.

>

The cold cook method does work well with some neons.

Depends on the consistancy of the potting compound.

Some are pretty hard and brittle while others remain

pretty sticky even when frozen.

Since most of my unpotting jobs have been done during

warm months I usually dont freeze the neons first.

I simply split open the steel case with a chisel

and discard it. I then chip away the tar from the transformer

using a screwdriver and small hammer. You can remove 95 percent

of the tar this way. I clean it off until I can get the

core separated and the windings off. Then everything goes

into kerosene to dissolve away the remaining tar.

The most important thing is to go slowly and carefully.

Its very easy to run a screwdriver into a secondary and damage it.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 16 Nov 1998 10:59:11 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:43 AM 11/16/98 -0800, you wrote:

>> >Would you elaborate a bit on this? What temperature, how long etc.

>> >In the meantime I'm going to reread your post on repotting your neon & hit the

>> >local sign shops. Apart from the obvious hazards coiling is also very

>> >addictive. I won't last long without the sparks.

>> >            Thanks,  Jim

>

>Hi, I'll save you some time here.  I tried several times to bake a neon, and ended

>up with a messy non-working neon.  I left it in the oven for hours, gradually

>increasing the temerature, and had no success actually getting the tar to melt.

>When I finally did get it to melt, it melted at the bottom first, and ended up

>getting a little too hot and oozing out all over the tray.  Even after this, it

>didn't work.

Baking the neon will only repair one kind of fault. And that is a carbon

track short through the potting compound. And it will not even fix all of those.

If your neon has a different kind of problem then baking wont help.

Thats why I dont normally recommend it. However if you have no intention

of trying to unpot your neon its worth a shot before you toss it out.

>

>I've decided that my next attempt at unpotting a neon will be to pull the top off

>of the neon, and set it upside down on a metal rack which is submerged in

>turpentine.  My thinking here is that gravity will cause all of the disolved tar to

>fall to the bottom of the bucket, and eventually I'll have a nice shiny transformer

>sitting on the rack.  Sure it'll take a couple of weeks, but this thing has just

>been sitting in the garage anyway waiting for the day I get around to trying to

>chip the tar out.......If I'd soaked it a month ago, I'd be done by now.

>

I doubt it would be done. You would probably have a big gooey blob

of tar. And a nasty mess. I did this just once on my first rebuild.

I soaked the tar block in kerosene for a month and had a real mess.

I had to scrape the gooey tar off and underneath that the remaining

tar had hardly been touched.

What you need to do is chisel away all the large areas of tar. Some of

the big chunks will come away in one piece. The more you remove before

soaking the faster the soaking job will go.

With Franceformers the core comes apart rather easily once you remove

the bolts and retaining strips. It splits apart in the middle with the

primary coil straddling the joint.

Once you have the core apart and the windings off dump the parts into

separate containers of kerosene to remove the remaining tar. If there

is still quite a bit of tar on the parts you may need several changes

of fresh kerosene.

I've been slowly repaving the road in front of my house with all the tar. :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: DavidAllan@aol.com

Date: Mon, 16 Nov 1998 12:28:31 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Garage coiling, and tuning (again)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have a small-medium (?) coil (four-inch secondary, powered by a 10kV/23 mA

OBIT) that I'm working on. It is my preference to work in my garage, but my

wife is concerned about flammables and so forth. (I've already asked the list

about protecting the garage door opener.) I know that many coilers work in

their garages.

What precautions do you take to avoid fires? Do you store gasoline or other

solvents in your garage? Has anyone ever lit his garage on fire?

Also, another tuning question. I have been able to tune my coil to get 7-10 cm

streamers from a bolt as the top electrode, but I'm *still* having trouble

with the toroid. I taped a screw to the top of the toroid (point facing up)

and could tune only to streamers of 1-2 centimeters. Moving my primary tap in

either direction diminished even this discharge. Still appreciate any advice

here. With the toroid in place and a properly tuned coil, should I expect

streamers to the air, or do I need some kind of grounded target? I noticed in

someone's post that the Colorado coilers tuned their magnifier with a signal

generator/scope. If I could gain access to test equipment, how would I use

that to tune my coil? (Pointers to the archive on this point will also be

welcome.)

Thanks.

David Allan

Date: Mon, 16 Nov 1998 10:48:49 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: winter conditions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> When I ran my coil last it was in an unheated garage. The temperature

> wasn't that cold -- maybe forty degrees F at the least. There was a

> noticeable difference, although not huge, as the temperature change was

> maybe only twenty-or-so-degrees. I'm looking forward to testing the coil

> outside some cold January night where it will (hopefully) be below zero.

> I'll report my results then.

>

> Tell me more about your crazy carpet capacitor plans. They use rollable

> plastic sleds, right? Do the plans cost anything?

>

> Daniel Lockhart

I want mine in the shed cause I the downstairs is just too risky(and my

coil is just too big)

It's the plastic sleds, all rolled up, the plans are free, its 55k.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Mon, 16 Nov 1998 17:17:51 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

Alfred A Skrocki wrote:

> They are held in by clips. Once the transformer and it's pitch coating

> are free of the metal shell give it a few wacks with a wooden mallet,

> this will remove the vast majority of the pitch and THEN put the

> remaining core and exposed windings in the solvent. Not only will you

> use much less solvent but you'll get the whole deal cleaner.

One trick to keep from damaging the fine wires is to warm the insides up

a little after it has completely chilled. Assuming the primary is good,

just plug it in for 15-30 min. This softens the pitch immediately around

the transformer guts.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Baking Neons

Date: Mon, 16 Nov 1998 18:28:00 CST

From: Kent Vander Velden <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>> They are held in by clips. Once the transformer and it's pitch coating

>> are free of the metal shell give it a few wacks with a wooden mallet,

>> this will remove the vast majority of the pitch and THEN put the

>> remaining core and exposed windings in the solvent. Not only will you

>> use much less solvent but you'll get the whole deal cleaner.

Recently I unpotted three neon transformers and an OBIT using solvent.

Two of the neons I chipped away some of the tar but the others I just

dumped into the solvent to see what would happen.  The transformers

went one at a time into a three gallon bucket of diesel fuel.

Having seen how diesel fuel affects asphalt my hope was that the fuel would

have no trouble with the tar.  After being in the solvent for a week I

would take the transformer out and wash the loose tar off.  Then back into

the bucket it would go until done.  About a month was needed for each

transformer.  In the end, nearly all the tar was removed

from the transformers.  There were patches of tar that seemed to be dissolving

much slower than the majority.  I have no explanation for this other than

the patches were composed of a different material.  These patches were

always on the ends of the core and were easily snapped off without any

risk of damage to the coils.  The diesel fuel (a little at a time :)

to help start a wood burning furnace.  Very economical and more

environmently friendly than flushing with clean solvent several times.

In summary, I found the use of diesel fuel a highly effective way of

unpotting a neon without risk of damage.  Three gallons of diesel fuel

would appear to be enough to remove the tar from two or three neons with

little need to chisel tar away.  Probably requires more time than renewing

the solvent but worth consideration.

---

Kent Vander Velden

kent@iastate.edu

X-Mailer: IPAD 2.0

Date: Mon, 16 Nov 1998 19:46:19 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Garage coiling, and tuning (again)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:28 PM 11/16/98 EST, you wrote:

>I have a small-medium (?) coil (four-inch secondary, powered by a 10kV/23 mA

>OBIT) that I'm working on. It is my preference to work in my garage, but my

>wife is concerned about flammables and so forth. (I've already asked the list

>about protecting the garage door opener.) I know that many coilers work in

>their garages.

>

>What precautions do you take to avoid fires? Do you store gasoline or other

>solvents in your garage? Has anyone ever lit his garage on fire?

I would remove any flammables from the area. At higher power levels

there are many unseen sparks jumping around nearby objects.

A fire extinguisher rated for electrical equipment is a very good

idea to have nearby.

>

>Also, another tuning question. I have been able to tune my coil to get 7-10 cm

>streamers from a bolt as the top electrode, but I'm *still* having trouble

>with the toroid. I taped a screw to the top of the toroid (point facing up)

>and could tune only to streamers of 1-2 centimeters. Moving my primary tap in

>either direction diminished even this discharge. Still appreciate any advice here.

Adding a toroid will dramatically lower your secondary frequency.

If your coil tunes well with NO discharge terminal then it most

likely will not be able to tune low enough to match the secondary + toroid.

You most likely will need additional primary turns or a larger main

cap or both.

> With the toroid in place and a properly tuned coil, should I expect

>streamers to the air, or do I need some kind of grounded target?

Air streamers once the tuning is correct.

>I noticed in someone's post that the Colorado coilers tuned their magnifier

>with a signal generator/scope. If I could gain access to test equipment,

>how would I use that to tune my coil? (Pointers to the archive on this point

>will also be welcome.)

The use of test equipment to measure coil parameters could take

an entire book. And its not really that necessary. If you have

this equipment then it can save you some hit and miss experiments.

You can calculate the inductance of your coils pretty accurately

and if you know your cap value you can determine your resonant

frequencies. As long as your primary and secondary frequencies match

you are all set.

If you have a signal generator you can base feed your secondary

through a couple of back to back LEDs. When the generator is set

to the coils resonant frequency the LEDs light the brightest.

A cap meter or digital LCR meter would be a good investment.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 16 Nov 1998 21:15:36 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Baking Neons

Sender: tesl-err@emachine.com

> >Hi, I'll save you some time here.  I tried several times to bake a neon, and ended

> >up with a messy non-working neon.  I left it in the oven for hours, gradually

> >increasing the temerature, and had no success actually getting the tar to melt.

> >When I finally did get it to melt, it melted at the bottom first, and ended up

> >getting a little too hot and oozing out all over the tray.  Even after this, it

> >didn't work.

>

> Baking the neon will only repair one kind of fault. And that is a carbon

> track short through the potting compound. And it will not even fix all of those.

> If your neon has a different kind of problem then baking wont help.

> Thats why I dont normally recommend it. However if you have no intention

> of trying to unpot your neon its worth a shot before you toss it out.

>

> >

> >I've decided that my next attempt at unpotting a neon will be to pull the top off

> >of the neon, and set it upside down on a metal rack which is submerged in

> >turpentine.  My thinking here is that gravity will cause all of the disolved tar to

> >fall to the bottom of the bucket, and eventually I'll have a nice shiny transformer

> >sitting on the rack.  Sure it'll take a couple of weeks, but this thing has just

> >been sitting in the garage anyway waiting for the day I get around to trying to

> >chip the tar out.......If I'd soaked it a month ago, I'd be done by now.

Gasoline works much better than kerosene. It will eat the tar almost

completely away. But no matter what type of solvent you use it works

better, and FASTER in the summer when the tempertures are warm. After

soaking mine in gasoline in July all I had to do was finish the cleanup

with a little fresh gasoline, and a paint brush. Then set the

transformer outdoors in a sunny location for a couple of days until the

odor died down.

Danger!!! don't put the container you soak the transfomer in inside of

any structure you don't want burned down! ALLWAYS perform any cleaning

with gasoline, or any flammable solvent OUTDOORS!

Frankensteins Helper

Max

From: McFalc0n@aol.com

Date: Mon, 16 Nov 1998 23:17:34 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: First Light

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got my coil up and working 2 sundays ago.  I have not finished tuning, but

I am getting 10" to groung running 450 watts.  So far it's been fun amazing my

friends(i'm only a fish in high school).

-I am using multiple strand RF ground wire.  I read from some RQ file that

flat strapping works the best.  should I switch?

-Where is a good file with instr.'s for tuning???

-How much ozone is too much???  I've been working in my garage, and i make

frequent trips between 5-second runs to discharge the cap. and "try" to tune

it (I never have time to finish the job).  I think I get a lot of ozone while

working near the coil.

-Plus, I have some weird corona coming from one lead on my cap.  I soldered

the wire to a ring connector, so there may be a sharp point.  The corona never

looks the same.  One time it looked sort of like a fog.  Could perfect tuning

help fix this?

-thanx for the help and answering the questions.

Date: Mon, 16 Nov 1998 23:40:55 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First Light

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

McFalc0n@aol.com wrote:

>

>         I got my coil up and working 2 sundays ago.  I have not finished tuning, but

> I am getting 10" to groung running 450 watts.  So far it's been fun amazing my

> friends(i'm only a fish in high school).

>         -I am using multiple strand RF ground wire.  I read from some RQ file that

> flat strapping works the best.  should I switch?

Solid wire is always better than standed, round is best to avoid corona loss.

-How much ozone is too much???

Ozone is a lot like carbon monoxide and will build up in the bottom of

your lungs causing headaches, flu symptoms and finally uncontiousness.

Its like running your car in a sealed garage.

> I've been working in my garage, and I make

> frequent trips between 5-second runs to discharge the cap. and "try" to tune

> it (I never have time to finish the job).  I think I get a lot of ozone while

> working near the coil.

>         -Plus, I have some weird corona coming from one lead on my cap.  I soldered

> the wire to a ring connector, so there may be a sharp point.  The corona never

> looks the same.  One time it looked sort of like a fog.  Could perfect tuning

> help fix this?

Try putting a dab of clear silicone on that terminal, it's a power loss.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Mon, 16 Nov 1998 23:48:42 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: side effects

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I ran a 4" coil a little while ago, the entire coil was glowing

bluish/purple but no sparks( except the odd break out to primary).

it was pvc with thick coating of varnish

I tried another smaller coil with the same setup and it worked great,any ideas?

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

X-Originating-IP: [195.121.16.245]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First Light

Date: Tue, 17 Nov 1998 06:34:51 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>-How much ozone is too much??

You know youve had too much when you cough blood and collapse on the

floor. Until than its alright :-)

>Ozone is a lot like carbon monoxide and will build up in the bottom of

>your lungs causing headaches, flu symptoms and finally uncontiousness.

>Its like running your car in a sealed garage.

Small correction here:

Carbon Monoxide is a poison. Our respiratory system can absorb it 600%

more efficiently than it can absorb oxygen, therefore concentrations of

just 3% are enough to kill.

Ozone, in the other hand, is an enriched Oxygen molecule. It is

absorbed by our bodies just as well as oxygen, and it is eliminated just

as easy. Although not a poison, ozone is a powerful oxidising agent, and

exposure to large amounts for extended periods of time can be harmful.

The effects are different from those of CO2 and it is not dense enough

to stay at the bottom of your lungs. Irritation to the eyes and lungs

are common symptoms.

Safe Coiling to you!

Sam Barros.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 18 Nov 1998 08:43:02 +1200

Subject:       Re: First Light

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Sam,

I'd definitely count ozone as a poison. It burns living

tissue without a flame.

Malcolm

> From:           "Sam Barros" <sambarros@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: First Light

> Date sent:      Tue, 17 Nov 1998 06:34:51 PST

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >-How much ozone is too much??

>

>  You know youAEve had too much when you caught blood and collapse on the

> floor. Until than itAEs alright :-)

>

> >Ozone is a lot like carbon monoxide and will build up in the bottom of

> >your lungs causing headaches, flu symptoms and finally uncontiousness.

> >Its like running your car in a sealed garage.

>

>

>  Small correction here:

>  Carbon Monoxide is a poison. Our respiratory system can absorb it 600%

> more efficiently than it can absorb oxygen, therefore concentrations of

> just 3% are enough to kill.

>  Ozone, in the other hand, is an enriched Oxygen molecule. It is

> absorbed by our bodies just as well as oxygen, and it is eliminated just

> as easy. Although not a poison, ozone is a powerful oxidising agent, and

> exposure to large amounts for extended periods of time can be harmful.

> The effects are different from those of CO2 and it is not dense enough

> to stay at the bottom of your lungs. Irritation to the eyes and lungs

> are common symptoms.

>

>

>  Safe Coiling to you!

>

>  Sam Barros.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 18 Nov 1998 08:43:53 +1200

Subject:       Re: side effects

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Was the tuning correct in the first case?

Malcolm

> Date sent:      Mon, 16 Nov 1998 23:48:42 -0800

> From:           " Mike N." <705728@ican.net>

> Organization:   Canadian Pyrotechnics Association

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: side effects

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I ran a 4" coil a little while ago, the entire coil was glowing

> bluish/purple but no sparks( except the odd break out to primary).

>

> it was pvc with thick coating of varnish

> I tried another smaller coil with the same setup and it worked great,

> any ideas?

> --

> Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

>

> Sonic Boom

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 18 Nov 1998 10:18:10 +1200

Subject:       Re: side effects

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

> Date sent:      Tue, 17 Nov 1998 16:05:36 -0800

> From:           " Mike N." <705728@ican.net>

> Organization:   Canadian Pyrotechnics Association

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: side effects

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm Watts wrote:

> >

> > Was the tuning correct in the first case?

> >

> > Malcolm

> Tuning was fine,I set it to the point where I got the most glowing,

> changing the primary tuning would just reduce it, I had a nice raspy

> spark at the stationary gap, I attempted different outputs, point,

> copper ball and still the same effect after retuning, could it be my

> wire was bad?

What wire did you use? In fact, what are the details? If you've

posted them already, don't bother - I'll search back to your original

post. Didn't you get some kind of a discharge from the top? With a

grounded wire held near it to tempt it? Sounds like an interesting

coil.

> > > I ran a 4" coil a little while ago, the entire coil was glowing

> > > bluish/purple but no sparks( except the odd break out to primary).

> > >

> > > it was pvc with thick coating of varnish

> > > I tried another smaller coil with the same setup and it worked great,

> > > any ideas?

> > > --

> > > Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Good question :)

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 17 Nov 1998 17:59:22 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First Light

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:17 PM 11/16/98 EST, you wrote:

>	I got my coil up and working 2 sundays ago.  I have not finished tuning, but

>I am getting 10" to ground running 450 watts.  So far it's been fun amazing my

>friends(i'm only a fish in high school).

10 inches for first sparks is doing very well. Much better than

my first coil. (and my second)

>	-I am using multiple strand RF ground wire.  I read from some RQ file that

>flat strapping works the best.  Should I switch?

No. You are not likely to notice any difference. I use AWG 8 stranded wire

for my ground lead quite successfully. Stranded wire is OK but braided cable is not.

>	-Where is a good file with instr.'s for tuning???

Well take a look on Dave Bronakowski's archive page at:

http://www.velocity.net/~djb/files.shtml

and take a look at the file named tuning.txt

I happen to think its a great file. And I happen to know

the author rather well.  :)

>	-How much ozone is too much???  I've been working in my garage, and i make

>frequent trips between 5-second runs to discharge the cap. and "try" to tune

>it (I never have time to finish the job).  I think I get a lot of ozone while

>working near the coil.

As long as you get some ventilation and go out for fresh air

you wont have a problem. When the nose starts to burn and the eyes

begin to itch you've had too much.

>	-Plus, I have some weird corona coming from one lead on my cap.  I soldered

>the wire to a ring connector, so there may be a sharp point.  The corona never

>looks the same.  One time it looked sort of like a fog.  Could perfect tuning

>help fix this?

Corona is always going to be around whenever you have high voltages.

Try to minimize it by eliminating sharp points and edges. A big glob

of RTV over the terminal should help.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 17 Nov 1998 16:05:36 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: side effects

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm Watts wrote:

>

> Was the tuning correct in the first case?

>

> Malcolm

Tuning was fine,I set it to the point where I got the most glowing,

changing the primary tuning would just reduce it, I had a nice raspy

spark at the stationary gap, I attempted different outputs, point,

copper ball and still the same effect after retuning, could it be my

wire was bad?

>

> > I ran a 4" coil a little while ago, the entire coil was glowing

> > bluish/purple but no sparks( except the odd break out to primary).

> >

> > it was pvc with thick coating of varnish

> > I tried another smaller coil with the same setup and it worked great,

> > any ideas?

> > --

> >

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 18 Nov 1998 15:28:32 +1200

Subject:       Re: side effects

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

What is the inductance of the primary coil (inductance you

have tapped in) and what kind of a topload are you using on the

secondary? Size/shape?

> Primary: archemedes spiral 15 degree incline

> glass plate cap: .005 Mfd(iirc)

> winding length: 32" #22 magnetron wire(from an old microwave)

> power:15Kv at 60ma

> static gap

> grounding: cold water pipe

> discharge terminal: variety

>

> I did no tests with a grounded rod

Malcolm

> > > > > I ran a 4" coil a little while ago, the entire coil was glowing

> > > > > bluish/purple but no sparks( except the odd break out to primary).

> > > > >

> > > > > it was pvc with thick coating of varnish

> > > > > I tried another smaller coil with the same setup and it worked great,

> > > > > any ideas?

> > > > > --

> > Good question :)

> > Malcolm

>

>

>

Date: Tue, 17 Nov 1998 20:52:12 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: side effects

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > >

> > > Was the tuning correct in the first case?

> > >

> > > Malcolm

> > Tuning was fine,I set it to the point where I got the most glowing,

> > changing the primary tuning would just reduce it, I had a nice raspy

> > spark at the stationary gap, I attempted different outputs, point,

> > copper ball and still the same effect after retuning, could it be my

> > wire was bad?

>

> What wire did you use? In fact, what are the details? If you've

> posted them already, don't bother - I'll search back to your original

> post. Didn't you get some kind of a discharge from the top? With a

> grounded wire held near it to tempt it? Sounds like an interesting

> coil.

Primary: archemedes spiral 15 degree incline

glass plate cap: .005 Mfd(iirc)

winding length: 32" #22 magnetron wire(from an old microwave)

power:15Kv at 60ma

static gap

grounding: cold water pipe

discharge terminal: variety

I did no tests with a grounded rod

> > > > I ran a 4" coil a little while ago, the entire coil was glowing

> > > > bluish/purple but no sparks( except the odd break out to primary).

> > > >

> > > > it was pvc with thick coating of varnish

> > > > I tried another smaller coil with the same setup and it worked great,

> > > > any ideas?

> > > > --

> > > > Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

>

> Good question :)

> Malcolm

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Tue, 17 Nov 1998 23:21:31 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ozone (was First Light)

Sender: tesl-err@emachine.com

Sam Barros wrote:

<SNIP>

>  Ozone, in the other hand, is an enriched Oxygen molecule. It is

> absorbed by our bodies just as well as oxygen, and it is eliminated just

> as easy. Although not a poison, ozone is a powerful oxidising agent, and

> exposure to large amounts for extended periods of time can be harmful.

> The effects are different from those of CO2 and it is not dense enough

> to stay at the bottom of your lungs. Irritation to the eyes and lungs

> are common symptoms.

>

>  Safe Coiling to you!

>

>  Sam Barros.

Sam and all,

Apparently ozone is toxic at any levels above about 1 part per million.

I've included some pertinent excerpts from a site which tells us MUCH

more about ozone than we probably ever wanted to know:

http://ntp-db.niehs.nih.gov/NTP_Reports/NTP_Chem_H&S/NTP_Chem1/Radian10028-15-6.txt

Ozone is not only a powerful oxidizer, it's also an irritant, mutagen,

carcinogen, may cause birth defects, and it can kill lab animals at

comparatively low concentrations (10-50 ppm) over a period of 1/2 - 3

hours(!)...

------------------------------------------------------------------------

Excerpts:

========

*ACUTE/CHRONIC HAZARDS:

This compound is irritating and injurious at concentrations >1 ppm

[102].  It may cause severe irritation of the respiratory tract and eyes

[031,042,102].

SYMPTOMS:

Exposure to a 1 ppm concentration of this compound may result in

headache and irritation of the eyes and upper respiratory tract.

Symptoms disappear after leaving the exposure [042].  Other symptoms

include dryness of the nose, upset stomach or vomiting, pain in the

chest, tiredness and lung damage [102]. This compound is a lung irritant

capable of causing death from pulmonary edema (gross pulmonary edema is

evident in mice exposed to concentrations >2 ppm). Long-term exposure

may cause thickening of the terminal respiratory bronchioles.  Chronic

bronchitis, fibrosis and emphysematous changes are observed at

concentrations slightly above 1 ppm.  At concentrations of 0.25-0.75

ppm, exposure may cause shallow, rapid breathing, decrease in pulmonary

compliance, cough, tightness of the chest and dryness of the throat.  It

also increases sensitivity of the lung to bronchoconstrictors such as

histamine, acetylcholine and allergens [406]. People with obstructive

lung diseases showed increased breathing resistance, increased oxygen

consumption and decreased arterial oxygen concentration when exposed to

this compound.  Other symptoms include reduced visual acuity, increased

peripheral vision, decreased night vision and altered balance of muscles

controlling the position of the eye [301].  Exposure has also caused

dyspnea, dermatitis, lacrimation and somnolence [015].

------------------------------------------------------------------------

Small coils seem to generate larger amounts of ozone, while larger coils

seem to generate more nitrogen oxides. Anyone who's done any extended

coiling has undoubtedly felt some of the symtoms described above, and

inhaling either byproduct can cause some real problems!

Play safe - ventillate or evacuate the area until the levels subside!

-- Bert --

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 18 Nov 1998 17:23:55 +1200

Subject:       Re: side effects

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I cannot troubleshoot your system without the requested

information:

> Date sent:      Tue, 17 Nov 1998 21:53:48 -0800

> From:           " Mike N." <705728@ican.net>

> Organization:   Canadian Pyrotechnics Association

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: side effects

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm Watts wrote:

> >

> > Mike,

> >        What is the inductance of the primary coil (inductance you

> > have tapped in) and what kind of a topload are you using on the

> > secondary? Size/shape?

>

>     I believe I tapped at the 3rd turn, as for the top load I tried a

> nail and a copper toilet ball, same effect after retuning.

So what was the primary inductance (not the number of turns)? What

size was the copper ball? What changes in tapping/inductance did you

have to make to get into tune?

Malcolm

Date: Tue, 17 Nov 1998 21:53:48 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: side effects

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm Watts wrote:

>

> Mike,

>        What is the inductance of the primary coil (inductance you

> have tapped in) and what kind of a topload are you using on the

> secondary? Size/shape?

I believe I tapped at the 3rd turn, as for the top load I tried a

nail and a copper toilet ball, same effect after retuning.

> > Primary: archemedes spiral 15 degree incline

> > glass plate cap: .005 Mfd(iirc)

> > winding length: 32" #22 magnetron wire(from an old microwave)

> > power:15Kv at 60ma

> > static gap

> > grounding: cold water pipe

> > discharge terminal: variety

> >

> > I did no tests with a grounded rod

>

> > > > > > I ran a 4" coil a little while ago, the entire coil was glowing

> > > > > > bluish/purple but no sparks( except the odd break out to primary).

> > > > > >

> > > > > > it was pvc with thick coating of varnish

> > > > > > I tried another smaller coil with the same setup and it worked great,

> > > > > > any ideas?

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Wed, 18 Nov 1998 00:06:04 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: side effects

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Mike,

>       I cannot troubleshoot your system without the requested

> information:

> > Malcolm Watts wrote:

> > >

> > > Mike,

> > >        What is the inductance of the primary coil (inductance you

> > > have tapped in) and what kind of a topload are you using on the

> > > secondary? Size/shape?

> >

> >     I believe I tapped at the 3rd turn, as for the top load I tried a

> > nail and a copper toilet ball, same effect after retuning.

>

> So what was the primary inductance (not the number of turns)?

I really have no idea as to the inductance,

>What size was the copper ball?

aprox. 3.5" with many bumps

>What changes in tapping/inductance did you have to make to get into tune?

With the copper ball I had to increase the number of turns used in the

primary. Could it have this affect if I didnt treat the abs first.

The only sparks were to the primary a best tunning, and they were bright

white and around 12" long from the center of the coil.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

X-Originating-IP: [195.121.16.105]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: High power HVDC cap design.

Date: Wed, 18 Nov 1998 04:54:48 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello everyone,

I'm trying to design (and hopefully build) a 500KV DC, 1KJ Marx

generator and I was wondering if anyone could help me out with cap

design. I know this isn't really Tesla-Related, but I guess since Tesla

Coils use high performance capacitors, and a lot of research has gone

into building them, this would be the best palace to look for info...

Basically I was thinking of charging it with a MOT connected to a 25

stages cascade, outputting about 50KV (comments here would also be

appreciated). Than I could use a 10 stage Marx arrangement to bring the

output to 500KV. The main problem lies in the capacitor construction:

These capacitors are very high power and buying them is out of question

(100Joule capacitors must cost so MUCH).

But, since they are going to work on DC mode, with no destructive RF or

high BPS (I doubt it could go beyond 10BPS), I figured they could be

made using some cheap, easily obtained dielectric.

Well, I would like to know if anyone has tried a MARX before and if

anyone has ideas for a good cap design.

Please contact me off-list at sambarros@yahoo.com

X-Originating-IP: [199.212.10.74]

From: "James C." <dragonzbreath@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: HV Cap

Date: Wed, 18 Nov 1998 10:04:19 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Have you been able to get actual values worked out for the "Crazy Kap"?

Please do send your plans.

Thanks,

James

Date: Wed, 18 Nov 1998 08:50:15 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: HV Cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Have you been able to get actual values worked out for the "Crazy Kap"?

>Please do send your plans.

>Thanks,

>James

Ditto.

Daniel Lockhart

molly@cmn.net

X-Mailer: IPAD 2.0

Date: Wed, 18 Nov 1998 12:04:30 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: HV Cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:04 AM 11/18/98 EST, you wrote:

>Have you been able to get actual values worked out for the "Crazy Kap"?

>Please do send your plans.

>Thanks,

>James

>

James and others interested in the Crazy carpet cap design

from Mike Nikolas.

Mike has sent me a copy of his file and after some minor

editing and conversion to GIF format from BMP I will

upload it to Dave Bronakowski's archive site. I will

post the info on the file and where to find it once its in place.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 19 Nov 1998 08:56:34 +1200

Subject:       Re: side effects

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

It sounds to me as though your primary is running at twice

the frequency it should. Get hold of a program or spreadsheet or

formulae and calculate your secondary coil inductance and capacitance

and frequency. Do the same for the primary. I think the answer lies

there.

Malcolm

> Date sent:      Wed, 18 Nov 1998 00:06:04 -0800

> From:           " Mike N." <705728@ican.net>

> Organization:   Canadian Pyrotechnics Association

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: side effects

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > Mike,

> >       I cannot troubleshoot your system without the requested

> > information:

>

> > > Malcolm Watts wrote:

> > > >

> > > > Mike,

> > > >        What is the inductance of the primary coil (inductance you

> > > > have tapped in) and what kind of a topload are you using on the

> > > > secondary? Size/shape?

> > >

> > >     I believe I tapped at the 3rd turn, as for the top load I tried a

> > > nail and a copper toilet ball, same effect after retuning.

> >

> > So what was the primary inductance (not the number of turns)?

> I really have no idea as to the inductance,

>

> What

> > size was the copper ball?

> aprox. 3.5" with many bumps

>

>  What changes in tapping/inductance did you

> > have to make to get into tune?

> With the copper ball I had to increase the number of turns used in the

> primary. Could it have this affect if I didnt treat the abs first.

>

> The only sparks were to the primary a best tunning, and they were bright

> white and around 12" long from the center of the coil.

>

Date: Wed, 18 Nov 1998 13:12:38 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HV Cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer wrote:

>

> At 10:04 AM 11/18/98 EST, you wrote:

> >Have you been able to get actual values worked out for the "Crazy Kap"?

> >Please do send your plans.

> >Thanks,

> >James

> >

>

> James and others interested in the Crazy carpet cap design

> from Mike Nikolas.

That is Mike Nicholas.

>

> Mike has sent me a copy of his file and after some minor

> editing and conversion to GIF format from BMP I will

> upload it to Dave Bronakowski's archive site. I will

> post the info on the file and where to find it once its in place.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Wed, 18 Nov 1998 16:11:56 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HV Cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Enjoy

Daniel Lockhart wrote:

>

> >Have you been able to get actual values worked out for the "Crazy Kap"?

> >Please do send your plans.

> >Thanks,

> >James

>

> Ditto.

>

> Daniel Lockhart

> molly@cmn.net

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\Crazycap.gif"

(editors note. binary attachments not in archive. this file can be found

at http://www.velocity.net/~djb/files.shtml. file name is crazycap.gif)

Date: Wed, 18 Nov 1998 16:42:01 -0500

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HV Cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

As far as I am concerned, don't worry about it. They were a good set of plans.

Bert W.

Mike N. wrote:

> Sorry, I didnt mean to post the plans on the group, it was supposed to

> go to an individual.

>

>

> > Enjoy

> >

> > Daniel Lockhart wrote:

> > >

> > > >Have you been able to get actual values worked out for the "Crazy Kap"?

> > > >Please do send your plans.

> > > >Thanks,

> > > >James

> > >

> > > Ditto.

> > >

> --

> Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

>

> Sonic Boom

Date: Wed, 18 Nov 1998 16:27:32 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HV Cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry, I didnt mean to post the plans on the group, it was supposed to

go to an individual.

> Enjoy

>

> Daniel Lockhart wrote:

> >

> > >Have you been able to get actual values worked out for the "Crazy Kap"?

> > >Please do send your plans.

> > >Thanks,

> > >James

> >

> > Ditto.

> >

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Massive Confusion.

Date: Wed, 18 Nov 1998 19:33:05 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Importance: Normal

Sender: tesl-err@emachine.com

Someone that is using a welder as a ballast, I would like to talk to you

about your current measurements. Something is very confusing about my setup.

I am using a lincon 225A welder as ballast for my pig. I shorted the output

of the pig, and took current readings with a borrowed current probe.

I got the following readings :

Welder  | AC Current

40A	  |	20A

60A	  |	28A

75A	  |	33.7A

90A     |   40A

105A	  |   47.4A

120A	  |   52.3A

At this point, I got worried, as I only have a 5KVA pig, and this represents

almost 12KVA input power, if you believe the readings. What bothers me, is

this is 50% of the setting.. and the welder wants a 50amp breaker. Mind you, my

30A fuses were fat-dumb-happy at this setting.

So I cranked it up to 225A setting. At this point, I showed 90A on my meter

and finally my 30A fuse popped. (note that for this, I bypassed the pig)

This makes *no* sense to me. [though the readings are consistant with

what I measured via a resistor in line] its almost like the readings are

100% of the true readings.

This leads to 2 questions:

1) will someone that uses a Lincoln welder and can measure the current please

check their values at the above settings.

2) Just how far can I push a 5KVA pig without it going *POOF*.

I admit I am a little disappointed.. I was hoping to be able to start at

about 2KVA, but if these numbers are correct, the minimum I have is 4.8KVA :/

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

http://www.access1.net/mycroft

X-Mailer: IPAD 2.0

Date: Wed, 18 Nov 1998 23:20:36 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Krazy Carpet cap plans

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To everyone that was interested in the cap plans.

Mike Nicholas's Krazy Carpet cap plans are now

available on Dave Bronakowski's archive page.

The URL is: http://www.velocity.net/~djb/files.shtml

File name is crazycap.gif

I see that Mike posted the GIF image to the list earlier

so I guess no one really needs it now. But its there

for the taking nonetheless.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 18 Nov 1998 23:38:11 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Krazy Carpet cap plans

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> To everyone that was interested in the cap plans.

>

> Mike Nicholas's Krazy Carpet cap plans are now

> available on Dave Bronakowski's archive page.

I saw the plans, missed somehow, what is Krazy Carpet and where does one

get it?

X-Mailer: IPAD 2.0

Date: Wed, 18 Nov 1998 23:48:56 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Krazy Carpet cap plans

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:38 PM 11/18/98 -0600, you wrote:

>Mike Hammer wrote:

>>

>> To everyone that was interested in the cap plans.

>>

>> Mike Nicholas's Krazy Carpet cap plans are now

>> available on Dave Bronakowski's archive page.

>

>I saw the plans, missed somehow, what is Krazy Carpet and where does one

>get it?

Well my impression is that its a sled. A thick piece of

poly you use like a sled. Ive seen something like it before

but dont remember where. Im going to check out WalMart tomorrow.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Krazy Carpet cap plans

Date: Thu, 19 Nov 1998 10:16:46 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I remember the little things.  I actually have one(somewhere) out in my

garage.  They are little sheets of plastic, just barely large enough to fit

a person's rear on, with two little hand-holds on the front.  As I recall,

when I got mine, they were only in the color of blue, but that might have

changed.  It was about $5-10 at the time(late eighties).

I could get a picture taken, scanned, and post it on the list, if

everyone would like that.

-Kent Rankin

-----Original Message-----

From: Mike Hammer <mhammer@misslink.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Thursday, November 19, 1998 12:56 AM

Subject: Re: Krazy Carpet cap plans

>At 11:38 PM 11/18/98 -0600, you wrote:

>>Mike Hammer wrote:

>>>

>>> To everyone that was interested in the cap plans.

>>>

>>> Mike Nicholas's Krazy Carpet cap plans are now

>>> available on Dave Bronakowski's archive page.

>>

>>I saw the plans, missed somehow, what is Krazy Carpet and where does one

>>get it?

>

>Well my impression is that its a sled. A thick piece of

>poly you use like a sled. Ive seen something like it before

>but dont remember where. Im going to check out WalMart tomorrow.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

Date: Thu, 19 Nov 1998 10:18:11 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Krazy Carpet cap plans

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Tom Polk wrote:

>

> Mike Hammer wrote:

> >

> > To everyone that was interested in the cap plans.

> >

> > Mike Nicholas's Krazy Carpet cap plans are now

> > available on Dave Bronakowski's archive page.

>

> I saw the plans, missed somehow, what is Krazy Carpet and where does one

> get it?

Any department store will carry it, it's a cheap plastic sled for

sliding on snow. Unless you dont get snow.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

From: EdwMullin@aol.com

Date: Thu, 19 Nov 1998 16:54:56 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: We have Ignition, sort of...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well, I fired up my latest coil, with the 3.5 inch secondary, tuning was going

nicely, I was getting six inch streamers at first, Then...   I used a material

for my flat plate cap that I got for free, I though it was polypropelene,

later I found out that it was a type of vinyl,  it worked great at low power

settings.  But when I had the tuning going well decided to increase power, I

got a large Primary strike and then Poof the cap quit!   I'll get more

information when I do the autopsy.   Still got the salt water caps though...

Live and Learn!!

X-Mailer: IPAD 2.0

Date: Fri, 20 Nov 1998 12:17:47 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: We have Ignition, sort of...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:54 PM 11/19/98 EST, you wrote:

>Well, I fired up my latest coil, with the 3.5 inch secondary, tuning was going

>nicely, I was geting six inch streamers at first, Then...   I used a material

>for my flat plate cap that I got for free, I though It was polypropelene,

>later I found out that it was a type of vinyl,  it worked great at low power

>settings.  But when I had the tuning going well decided to incease power,   I

>got a large Primary strike and then Poof the cap quit!   I'll get more

>information when I do the autopsy.   Still got the salt water caps though...

>Live and Learn!!

>

Ed,

I once made a flat plate cap from the same type of material.

It was a fairly thick vinyl and seemed at the time to be perfect.

Little did I know that it had terrible RF loss characteristics.

It had a good breakdown voltage and was very cheap.

It worked marginally better than my SW bottle caps for a while

and then died. An autopsy revealed that the vinyl dielectric

had actually melted in several places.

Just one of the many caps I blew up during my learning curve.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 20 Nov 1998 12:37:12 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Massive Confusion.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:33 PM 11/18/98 -0800, you wrote:

>Someone that is using a welder as a ballast, I would like to talk to you

>about your current measurements. Something is very confusing about my setup.

Michael,

Im not using a welder as a ballast but let me share a couple thoughts

for what its worth.

>I am using a lincon 225A welder as ballast for my pig. I shorted the output

>of the pig, and took current readings with a borrowed current probe.

>I got the following readings :

>Welder  | AC Current

>40A	  |	20A

>60A	  |	28A

>75A	  |	33.7A

>90A    |   40A

>105A	  |   47.4A

>120A	  |   52.3A

>At this point, I got worried, as I only have a 5KVA pig, and this represents

>almost 12KVA input power, if you beleive the readings. What bothers me, is

>this is 50% of the setting.. and the welder wants a 50amp breaker.

Is your worry that the actual measured current draw is not

corresponding to the labeled current settings on the welder?

If so consider that those labels most likely represent the

current available on the welder secondary not current draw

from the wall. And you are using it in a manner probably not

considered by the manufacturer.

>Mind you, my 30A fuses were fat-dumb-happy at this setting.

>So I cranked it up to 225A setting. At this point, I showed 90A on my meter

>and finally my 30A fuse popped. (note that for this, I bypassed the pig)

>This makes *no* sense to me. [though the readings are consistant with

>what I measured via a resistor in line] its almost like the readings are

>100% of the true readings.

Since your in line readings match up with the readings taken via the voltage

drop across the ballast resistor they are likely to be accurate.

The fact that your 30 amp fuses didnt blow until the current was higher

isnt really that suprising given the fact that your actual run times were

probably rather short. Those high current fuses take time to heat up enough

to melt the element.

>This leads to 2 questions:

>1) will someone that uses a Lincoln welder and can measure the current please

>   check their values at the above settings.

>2) Just how far can I push a 5KVA pig without it going *POOF*.

Actually quite a bit. 150% continously. 200% for hours. 300% for minutes.

>

>I admit I am a little disappointed.. I was hoping to be able to start at

>about 2KVA, but if these numbers are correct, the minimum I have is 4.8KVA.

You may just need to add a little more ballast. Increase your resistive

ballast a little or add in some additional inductance. Paul Marshall

has successfully used MOTs for current limiting on his pig. Throw a

MOT in the loop and make a few more measurements.

Its good that you are taking the time to make these measurements.

This will give you a baseline that you can use to judge how things are

doing when on the coil and alert you when something is wrong.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 20 Nov 1998 13:37:47 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: pri coil design?

Sender: tesl-err@emachine.com

Mike and anyone,

I'm laying out the Pri coil form: 30 deg 'saucer'; .3125" Cu tube @ .25"

spacing on the 30 deg diagonal. Inner turn: ~5.5" dia., for a 4" 2ndary

coil.

1. Is there a handy-dandy calculation for how much tube 20 turns will

require?   or How many turns can I get out of 50', at that spacing?

2. How much of a detriment to performance will excess outer turns,

beyond the tap be?

3. We calculated 15-17 turns as optimum??  Still open to revision...

trying to optimize Pri outer diameter form design. looking at 26.75 "

roughly, now. If I can't get it any smaler, I'll pick up a 30" round of

'finish' particle board for the coil support base.

4. I recall a posting which seemed to purport that the coupling and

other factors might be enhanced by an almost torroidal Pri winding---  ?

5. How critical is Pri turn spacing ?

6. Any argument with saucer form as 'most' optimum? 30, 40, ?? deg?

Or not worth the effort over a pancake?

7. any design suggestions welcome, as I have not started cutting chips, yet.

Still sputtering along....

-----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Massive Confusion

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Fri, 20 Nov 1998 23:04:48 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Michael I have a few comments for you. If I understand correctly

you are measuring welder output current. Welder "arc volts" are usually

around 20v, so at 100a your only at 2000 watts. 2000 watts/ 240 volts

=8.3 amps primary. Of course it will actually be a little higher due to

effiency, cooling fan I2R...etc   If you are talking the fuse in series

with welder out put, The way the fuse works is the confusion. The fuse

your using is probably rated at 125v or higher. At a higher voltage the

wattage dissapated across the fuse is greater = more heat to melt the

fuse element. ALSO fuses don't blow right on the rating anyway.   As for

how much the trans can handle. If you can open the lid, check the low

voltage winding size. #14 can handle 20 - 25 amps cont. I have a 5kva

pole trans. The low voltage winding is about #10 and I have dumped in as

much as 125 amps for ~ 10 seconds many times.  A note about  ozone .. if

2 ppm is so poisonous we're all dead many times over.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Sat, 21 Nov 1998 00:06:05 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: pri coil design?

Sender: tesl-err@emachine.com

Hi Gene!

My Comments are interspersed below.

Gene Waldo wrote:

>

> Mike and anyone,

>

> I'm laying out the Pri coil form: 30 deg 'saucer'; .3125" Cu tube @ .25"

> spacing on the 30 deg diagonal. Inner turn: ~5.5" dia., for a 4" 2ndary

> coil.

>

> 1. Is there a handy-dandy calculation for how much tube 20 turns will

> require?   or How many turns can I get out of 50', at that spacing?

You might want to set up a little spreadsheet which estimates this by

summing the circumference of a series of circles from turn 1 to 20. The

diameter of each circle will increase by the increase in diameter per

turn (which will be a function of the cosine of the angle of your

primary). For example, your 20-turn primary at 30 degrees will have a

first turn of about 5.5 inches in diameter, or about 17.3" in

circumference. Since your center-to-center turn spacing is 5/8" (3/8"

tubing with 1/4" spacing between turns), the effective diameter of the

NEXT turn will be 5.5" + 2*(0.625*Cosine(30)) or about 6.58" in

diameter, and its circumference will be about 20.7". Turn three's

diamter will be increase by another factor of 2*0.625*Cosine(30) and so

on.

Summing the circumferences for 20 turns with a 30 degree angle and

Rinner = 5.5" results in a total material length of about 82.7 feet (not

including any extra length for end-terminations). Looking at it another

way, a 50 foot coil of material will permit you to have about 14 turns

with material to spare. At turn 14 you'd have about 67 uH of inductance.

However, I'd increase the inner diameter to 6.5" (see answer to question

5 below). This would require about 88 feet of material (N/I extra) for

20 turns, and 50 feet of material would just barely give you 14 turns

and about 75 uH of inductance.

>

> 2. How much of a detriment to performance will excess outer turns,

> beyond the tap be?

>

No real detriment, other than making for a more tempting target for

toroid:primary streamer hits due to the greater vertical height.

Remember that autotransformer action will also occur in the unsued

portion of the primary. If you tap at turn 10, you'll have twice the

primary tank voltage at "open" turn 20. This can be a problem if the

voltage is high enough to arc over to the strikerail above the primary.

> 3. We calculated 15-17 turns as optimum??  Still open to revision...

> trying to optimize Pri outer diameter form design. looking at 26.75 "

> roughly, now. If I can't get it any smaler, I'll pick up a 30" round of

> 'finish' particle board for the coil support base.

Your calculations sound about right.

>

> 4. I recall a posting which seemed to purport that the coupling and

> other factors might be enhanced by an almost torroidal Pri winding---  ?

The coupling coefficient will increase as you make the primary more

solenoidal in shape - a helical primary will have much higher coupling

than a pancake primary. However, you really don't want too high a

coupling coefficient, since this tends to cause poorer gap quenching

performance and may also lead to damaging turn-turn flashovers across

the secondary winding. A flat primary will provide sufficient coupling

as long as you can adjust the bottom of the secondary winding to be

about even with the plane of the primary. This style primary also

provides you with maximum clearance to the toroid for better performance

at high power levels.

>

> 5. How critical is Pri turn spacing ?

Not very. Allow enough so that you can "tap" the primary without causing

a turn-turn flahover due to reduced clearances. A 1/4" gap between turns

should be more than sufficient, and will also be fairly tolerant of

unavoidable variations in the actual turn-turn spacing in the winding

process. It's actually more important that you allow enough spacing

between the secondary and the innermost turn of the primary, since a

flahover here can cause physical damage (welding and turn-to-turn

shorting) on your secondary. I'd recommend you increase this to at least

1" all around (minimum) or 1.25" (better yet).

>

> 6. Any argument with saucer form as 'most' optimum? 30, 40, ?? deg?

>   Or not worth the effort over a pancake?

My personal opinion is that there's no advantage to a saucer type

primary, since most coilers tend to want to increase the power of their

system over time. A saucer primary looks more interesting, and will

permit a slightly smaller overall primary diameter... but it will also

limit the power level that the system can be run at without encountering

excessive toroid-primary hits.

>

> 7. any design suggestions welcome, as I have not started cutting chips,

> yet.

>

Increase your inside primary diameter to at least 6 or 6.5".

> Still sputtering along....

> -----------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Hope this helped, good luck, and keep us posted on your progress!

-- Bert --

Date: Sat, 21 Nov 1998 15:00:58 +0000

From: Ross Ogden <Rossoggy@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Shock Hazard

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

(this is a one off)

(not very tesla related sorry)

Seeing there is some people on this list with quite good medical knowledge i need

to know about shock hazards on particular voltages.

Eg in wrong conditions could 35 vac 100 amps be dangerous. The body resistance

should keep down any current anaway ?

but what happens if say your skin is wet ?

Date: Sat, 21 Nov 1998 15:14:29 +0000

From: Ross Ogden <Rossoggy@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Shock Hazard

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Ps my hand to hand resistance is 1.5 meg ohm

hamd to foot is 750 k ohm

is this about average

Ross Ogden wrote:

> (this is a one off)

>  (not very tesla related sorry)

>

> Seeing there is some people on this list with quite good medical knowledge i need

> to know about shock hazards on particular voltages.

>

> Eg in wrong conditions could 35 vac 100 amps be dangerous. The body resistance

> should keep down any current anaway ?

>

> but what happens if say your skin is wet ?

Date: Sat, 21 Nov 1998 19:55:14 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary Flashover

Sender: tesl-err@emachine.com

Last night I decided to really give my six inch coil a real workout. I

set the system up and cranked the variac up to 100% with one of my

smaller toruses attached. Everything worked out fine, got 36" discharges

easily. Then I decided to change the toroid to a much larger one to see

if I could make it breakout. As soon as the gap fired there was a bright

light at the base of the secondary, and no sparks from the torus. At

first I was not sure were the light was comming from and thought the cap

was blown! After breaking out in a cold sweat I began to search around

for the problem. The first turn of the primary had flashed over to the

base of the secondary leaveing a nice burn on the side of the coil form!

Luckily the damage was only cosmetic. If I had been running at 5Kva it

would have blown a hole in the secondary. I don't want to change the

coupling by removing the first turn of the primary or raising the

secondary. Does anyone have a solution?

Frankensteins Helper

Max

Date: Sat, 21 Nov 1998 21:31:22 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Flashover

Sender: tesl-err@emachine.com

RODERICK MAXWELL wrote:

>

> Last night I decided to really give my six inch coil a real workout. I

> set the system up and cranked the variac up to 100% with one of my

> smaller toruses attached. Everything worked out fine, got 36" discharges

> easily. Then I decided to change the toroid to a much larger one to see

> if I could make it breakout. As soon as the gap fired there was a bright

> light at the base of the secondary, and no sparks from the torus. At

> first I was not sure were the light was comming from and thought the cap

> was blown! After breaking out in a cold sweat I began to search around

> for the problem. The first turn of the primary had flashed over to the

> base of the secondary leaveing a nice burn on the side of the coil form!

>   Luckily the damage was only cosmetic. If I had been running at 5Kva it

> would have blown a hole in the secondary. I don't want to change the

> coupling by removing the first turn of the primary or raising the

> secondary. Does anyone have a solution?

>

>                                 Frankensteins Helper

>                                        Max

Roderick,

You're fortunate - this often results in one or more shorted turns on

the secondary! While the real fix is increasing the primary-to-secondary

separation, you can try putting a batch of LDPE film rolled up between

the two. This should prevent hard flashovers, but will not completely

eliminate corona between the two. Sounds like your system is starting to

cook now! Good luck, and safe coiling to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary Flashover

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Sat, 21 Nov 1998 22:39:52 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

There are some things you just have to do. Remove a turn on the

inside, lower the secondary, retune and crank up the power a notch. Your

coil will reward you.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Sun, 22 Nov 1998 09:07:34 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Flashover

Sender: tesl-err@emachine.com

RODERICK MAXWELL wrote:

>

> Last night I decided to really give my six inch coil a real workout.

>   Luckily the damage was only cosmetic. If I had been running at 5Kva it

> would have blown a hole in the secondary. I don't want to change the

> coupling by removing the first turn of the primary or raising the

> secondary. Does anyone have a solution?

>

>                                 Frankensteins Helper

>                                        Max

Max and all,

I don't actually have a solution---  Only some conjecture and questions.

That kind of problem has been on my mind of late as I begin to layout a

Primary design:

1st assumption: most mtls (wood, plex, PVC, etc) will be totally

transparent to the coupling field emanating from the Pri.

2nd: that locating the Pri UNDER the form support base will not be a

problem; PRO: clean upper surface,  easier tapping of pri, 'some'(?)

mechanical/aerial sheilding between primary & 2ndary.  CON: loss of the

esthetics of a visible primary.

This further leads to the additional placement of a low plex or PVC

cylinder containment surrounding the base of the 2ndary, extending

upwards perhaps 6", which would be a possible advantage, however the

Primary was situated?

Ball in play; anyone can run with it!

----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sun, 22 Nov 1998 11:24:38 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Flashover

Sender: tesl-err@emachine.com

Kyle A. Break wrote:

>

>         There are some things you just have to do. Remove a turn on the

> inside, lower the secondary, retune and crank up the power a notch. Your

> coil will reward you.

Yes you may be right. But I want to explore another solution before I

take those "drastic" measures! The coupling at it's present point seems

to be just about perfect.

Frankensteins Helper

Max

Date: Sun, 22 Nov 1998 11:29:34 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Flashover

Sender: tesl-err@emachine.com

> Roderick,

>

> You're fortunate - this often results in one or more shorted turns on

> the secondary! While the real fix is increasing the primary-to-secondary

> separation, you can try putting a batch of LDPE film rolled up between

> the two. This should prevent hard flashovers, but will not completely

> eliminate corona between the two. Sounds like your system is starting to

> cook now! Good luck, and safe coiling to you!

>

> -- Bert --

I think I'll try this. I have a lot of leftover 30mil poly that I

used to build my caps with. Now I only have to get some plastic ties of

some kind to hold it into place.

Frankensteins Helper

Max

Date: Sun, 22 Nov 1998 10:07:36 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: pri coil design?

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Mike, Bert and anyone,

Bert, thanks for your help!

Revised (current) Primary design:

I'm laying out the Pri coil form: Flat; .3125" Cu tube @ .25"

spacing. Plan to mount it UNDER the base. Inner turn: ~6.5" dia., for a

4" 2ndary  coil.

I have overcome the esthetic loss of a top surface Primary winding, but

Dan is slower than I, in many areas. If we can pony up the bucks we may

do a Plex base, over the pri... Best of both worlds?

I'm tending towards a plex or PVC tube base-shield around the 2ndary to

prevent flashovers? described in prior post.

I may consider an inner-turn splice/join means for removing the inner

turns if we decide on a 6" or 8" dia secondary in the future; this means

would also extend to outer turns additions (excess of 50' winding

length), if that ever becomes necessary.

This will necessarily complicate the initial design, but may provide a

"universally" expandable coil.  Any thoughts?

Does the tap connection get any more complicated than a simple battery

clip?

Will using .25" spacing on a .3125" dia. conductor cause problems with

the bulk of such a clip? Like to keep spacing to the minimum.

Any design suggestions welcome, as I have not started cutting chips,

yet.

Looking for a source on ~3/16" cond., 2 pc. ceramic thru-hull

stand-offs....

APEX is SOL on these!

Still sputtering along....

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

Date: Sun, 22 Nov 1998 12:51:12 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: pri coil design?

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Gene Waldo wrote:

> >

> > Mike, Bert and anyone,

> Bert, thanks for your help!

>

> Revised (current) Primary design:

>

>  I'm laying out the Pri coil form: Flat; .3125" Cu tube @ .25"

>  spacing. Plan to mount it UNDER the base. Inner turn: ~6.5" dia., for a

> 4" 2ndary  coil.

>

> I have overcome the esthetic loss of a top surface Primary winding, but

> Dan is slower than I, in many areas. If we can pony up the bucks we may

> do a Plex base, over the pri... Best of both worlds?

This should work OK as long as the secondary coilform can come through

the base so that the botom of the coilform is below the base. This is

important, since optimal coupling usually is about at the point where

the bottom of the secondary winding is even with the plane of the

primary winding (+/- 1" or so).

>

> I'm tending towards a plex or PVC tube base-shield around the 2ndary to

> prevent flashovers? described in prior post.

This is probably not necessary as long as you have a minimum of 1- 1.5"

all around the secondary for clearance to the innermost turn of the

primary. Adding more insulation won't hurt, but interestingly enough,

adding a relatively thick layer of higher dielectric constant material

(such as PVC - k~5-6) may actually cause corona to form in the remaining

air space between! If you want to add supplementary insulation, use a

low dielectric constant material such as LDPE (k~2.2), and only use

about 100 mils or so.

>

> I may consider an inner-turn splice/join means for removing the inner

> turns if we decide on a 6" or 8" dia secondary in the future; this means

> would also extend to outer turns additions (excess of 50' winding

> length), if that ever becomes necessary.

>

> This will necessarily complicate the initial design, but may provide a

> "universally" expandable coil.  Any thoughts?

If you know that you're going to migrate to a 6" or 8" coils soon, then

the added design complexity is worth it. I reused the primary from my 6"

coil by removing the innermost 3 turns and then adding another 7 or so

when moving to the 10" coil. It required going to a larger base as well.

If you can do some tricks now to avoid some of the re-work later, it

sure sounds like it makes sense!

> Does the tap connection get any more complicated than a simple battery

> clip? Will using .25" spacing on a .3125" dia. conductor cause > > problems with the bulk of such a clip? Like to keep spacing to the > minimum.

A battery clip will work, but a simple clamp made from a strip of copper

wound around the copper tubing and tightnned with a screw will make

better contact. You may want to modify the shape of the clamping end of

the battery clip so that is more closely matches the circular profile of

the copper tubing to make contact over a larger surface area. You may

also need to slip over short pieces of insulated tubing on the turns on

either side of the tap to prevent inter-turn flashovers to the clip.

>

> Any design suggestions welcome, as I have not started cutting chips,

>  yet.

>

> Looking for a source on ~3/16" cond., 2 pc. ceramic thru-hull

> stand-offs....

> APEX is SOL on these!

Check out Surplus Sales of Nebraska, or some of the other surplus

houses. Also, at hamfests.

>

>  Still sputtering along....

>

> --

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Safe coilin' to you, Gene!

-- Bert --

Posted-Date: Sun, 22 Nov 1998 20:26:33 +0100 (MET)

From: "Peter Staal" <drtesla@casema.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: TC's in Holland

Date: Sun, 22 Nov 1998 20:26:12 +0100

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

As the title shows, my question is:

Where can I find Tesla coil(s) in Holland.

Does anybody know?

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 23 Nov 1998 08:58:03 +1200

Subject:       Re: Primary Flashover

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Roderick,

You did remember to retune (tap more turns in) the

primary didn't you?   ?     ?

Malcolm

> Date sent:      Sat, 21 Nov 1998 19:55:14 -0600

> From:           RODERICK MAXWELL <tank@magnolia.net>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Primary Flashover

> Last night I decided to really give my six inch coil a real workout. I

> set the system up and cranked the variac up to 100% with one of my

> smaller toruses attached. Everything worked out fine, got 36" discharges

> easily. Then I decided to change the toroid to a much larger one to see

> if I could make it breakout. As soon as the gap fired there was a bright

> light at the base of the secondary, and no sparks from the torus. At

> first I was not sure were the light was comming from and thought the cap

> was blown! After breaking out in a cold sweat I began to search around

> for the problem. The first turn of the primary had flashed over to the

> base of the secondary leaveing a nice burn on the side of the coil form!

>   Luckily the damage was only cosmetic. If I had been running at 5Kva it

> would have blown a hole in the secondary. I don't want to change the

> coupling by removing the first turn of the primary or raising the

> secondary. Does anyone have a solution?

>

>

>

>                                 Frankensteins Helper

>                                        Max

Date: Sun, 22 Nov 1998 17:59:25 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Flashover

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> Hi Roderick,

>               You did remember to retune (tap more turns in) the

> primary didn't you?   ?     ?

>

> Malcolm

>

Yes I tuned out one turn for starters. But the sucker flashed over as

soon as the spark gap fired so I never did get a chance to see if it was

in tune or not. Correct me if I'm wrong, but my theory is the energy in

the primary built up to too high a level since it could not escape

through the secondary due to the large topload.

Frankensteins Helper

Max

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Which spark gap?

Date: Sun, 22 Nov 1998 17:26:47 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have been pondering which type of spark gap to construct for use with a 15kv/60ma

nst, a single gap air blast type, (I have a compressor), or the multiple gap type.

(I believe it's commonly called a Richard Quick type). I would like to hear the pros

and cons of each type of gap.

Jon Rosenstiel

jonr@pacbell.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 23 Nov 1998 13:33:17 +1200

Subject:       Re: Primary Flashover

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Roderick,

> Date sent:      Sun, 22 Nov 1998 17:59:25 -0600

> From:           RODERICK MAXWELL <tank@magnolia.net>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: Primary Flashover

<snip>

>

>   Yes I tuned out one turn for starters. But the sucker flashed over as

> soon as the spark gap fired so I never did get a chance to see if it was

> in tune or not. Correct me if I'm wrong, but my theory is the energy in

> the primary built up to too high a level since it could not escape

> through the secondary due to the large topload.

You'd have to look on the scope to see exactly what was going on. It

can't build up to more than that allowed by a single cap recharge of

the transformer. It can get trapped in the primary by the out-of-tune

secondary or do a series of quick, shallow weak transfers. The

secondary would have been ringing at the wrong frequency if tuning

was the problem.

It is probably best to provide a discharge point for the

secondary until the system is brought into tune if a scope isn't

available.

Cheers,

Malcolm

Date: Sun, 22 Nov 1998 21:02:36 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Which spark gap?

Sender: tesl-err@emachine.com

Jon Rosenstiel wrote:

>

> I have been pondering which type of spark gap to construct for use

> with a 15kv, 60ma nst, a single gap air blast type, (I have a

> compressor), or the multiple gap type, (I believe it's commonly called

> a Richard Quick type).

>

> I would like to hear the pros and cons of each type of gap.

>

> Thank you

> Jon Rosenstiel   jonr@pacbell.net

Jon,

Actually, either type of gap can work quite well at this power level. An

air-blast gap may be somewhat easier to make, can handle a large amount

of power, but requires a source of compressed air, and it also tends to

be much louder in operation than a multiple-gap system. The so-called

Richard Quick type gap (originally invented by the Tesla Coil Builders

of Richmond- TCBOR), provides good quenching at low to moderate power

levels (up to about 1 -1.5 kVA), and can be cooled sufficiently using a

simple cooling (boxer) fan. However, it tends to overload and lose

quenching capability at higher power levels, causing coil output to drop

substantially. Another multiple-gap style (called a vacuuum gap, where

high velocity air is sucked between the spaces between groups of

parallel copper pipes) can run at higher power levels with good success.

Hope this helps!

-- Bert --

From: "Bill Langston" <blangsto@iwvisp.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: pri coil design?

Date: Sun, 22 Nov 1998 21:08:59 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello everyone, I am Bill Langston in Trona California. Thanks to all of you

who use this list, my son's, and my, first serious size coil, 6.5" powered

by 4 parallel 15Kv, 60 ma. NSTs fired well within 15 minutes of first power.

Again, thanks.

To the point, as I played with the power and tuning, we developed a

flashover problem that actually scared us to death the first time it

happened. We did the polyethylene tube around the secondary business, and it

worked very well.

X-Originating-IP: [195.121.16.8]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC's in Holland

Date: Mon, 23 Nov 1998 02:05:19 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>From: "Peter Staal" <drtesla@casema.net>

>To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Subject: TC's in Holland

>Date: Sun, 22 Nov 1998 20:26:12 +0100

>Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>As the title shows, my question is:   Where can I find  Tesla coil(s)

>in Holland.

>Does anybody know?

I have a Tesla Coil... So I guess you could say there is at least one

in Holland (I live in the north of Holland)...

But I haven't found a power supply yet. If you decide to build one

you'll have the same problem.

From: "Steve B." <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary coils

Date: Mon, 23 Nov 1998 16:30:51 -0000

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

Just finished work on my primary coil, and some of the stand hardware.

I've put some photos on my web site so you can see what it looks like,

rather good if I do say so myself!  :-)

(http://www.breakfix.demon.co.uk)

If I get past making and operating my first coil, I'd quite like to design

a second one using old style materials (wood and ceramics) to look cool but

still perform as well as a modern unit.

Q:

Is there any interest in setting up a database of coilers and their coils,

primarily to show the geographical spread? (Is there already such a thing?)

What about a photo gallery showing the best photos of streamers/corona and sparks,

as art rather than for technical information!

There's plenty on space on my web site

TTYL

Steve Bell.

X-Mailer: IPAD 2.0

Date: Mon, 23 Nov 1998 12:54:47 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:30 PM 11/23/98 -0000, you wrote:

>Hi Guys,

>	Just finished work on my primary coil, and some of the stand hardware.

>I've put some photos on my web site so you can see what it looks like,

>rather good if I do say so myself!  :-)

>

>(http://www.breakfix.demon.co.uk)

Nicely done web page. Primary coil looks quite nicely done as well.

Looks MUCH better than mine.

One tip I can give you on your web page. Schematic diagrams

and any type of line art should be done in GIF format rather

than JPG. The GIF will actually be smaller than the JPG and

much clearer.

>

>If I get past making and operating my first coil, I'd quite like to design

>a second one using old style materials (wood and ceramics) to look cool but

>still perform as well as a modern unit.

>

>Q:

>Is there any interest in setting up a database of coilers and their coils,

>primarily to show the geographical spread? (Is there already such a thing?)

>What about a photo gallery showing the best photos of streamers/corona and >sparks, as art rather than for technical information!

This has been attempted several times. The problem is that while

those involved started the project with enthusiasm it was never

kept up and soon fell by the wayside.

Several people have suggested that I should gather up the various

attempts and bring them up to date but Im not sure that there

is sufficient interest to warrant the time and effort.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 23 Nov 1998 12:58:46 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: pri coil design?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:08 PM 11/22/98 -0800, you wrote:

>Hello everyone, I am Bill Langston in Trona California. Thanks to all of you

>who use this list, my son's, and my, first serious size coil, 6.5" powered

>by 4 parallel 15Kv, 60 ma. NSTs fired well within 15 minutes of first power.

>Again, thanks.

Bill,

Four paralleled 15kv/60s? Thats some serious power.

Even for a 6 inch secondary. What is your cap size

and gap type and spacing and resulting spark output?

What size is your toroid?

I would bet you are not even utilizing 1/2 of your

available power and the rest is just vaporizing your

gap electrodes.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 23 Nov 1998 13:05:12 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Shock Hazard

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:00 PM 11/21/98 +0000, you wrote:

>(this is a one off)

> (not very tesla related sorry)

>

>Seeing there is some people on this list with quite good medical

>knowledge I need to know about shock hazards on particular voltages.

>

>Eg in wrong conditions could 35 vac 100 amps be dangerous.

>The body resistance should keep down any current anyway ?

>

>but what happens if say your skin is wet ?

Ross,

35 volts is not likely to be particulary hazardous unless

you were standing barefoot in water.

It does vary from person to person. I normally cannot feel

any voltages below about 40 volts.

The high current can give you some nasty burns if there is a short

circuit. Ive been burned plenty of times working on 12VDC and 28VDC

high current power supplies.

Keeping my feet out of the water...

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Phasing T-formers

Date: Mon, 23 Nov 1998 13:32:14 -0600

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi,

What was the test method to see if you have the secondaries properly phased

when trying to phase two transformers?

Thanks,

Roy Minnich

X-Mailer: IPAD 2.0

Date: Mon, 23 Nov 1998 17:13:13 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Phasing T-formers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:32 PM 11/23/98 -0600, you wrote:

>Hi,

>

>What was the test method to see if you have the secondaries properly phased when

>trying to phase two transformers?

>

>Thanks,

>

>Roy Minnich

>

Roy,

What I do is wire up the transformers in parallel and then

measure the current flow from the paralled secondaries.

If its not what is expected then the phasing is wrong and

I swap around one set of primary leads.

If you dont have an AC milliameter to make the direct

measurement then parallel up the transformers leaving

the last secondary connection free. Power up the transformers

and make that last connection with an insulated stick.

If you get little or no spark then the phasing is correct.

If you get a big spark then swap one set of primary leads around.

TTYL!

Mike Hammer

mhammer@misslink.net

From: McFalc0n@aol.com

Date: Mon, 23 Nov 1998 21:58:24 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Which spark gap?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Really?  Where are the plans for a vacuum gap?

Date: Mon, 23 Nov 1998 21:00:29 -0600

From: Tedd Payne <teddp@earthlink.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale:  Variacs, NSTs, 155 KV Rectifiers, Spectrum Tubes, HV Caps, Meters

Sender: tesl-err@emachine.com

Hi All,

If interested, please contact me DIRECTLY at teddp@earthlink.net

I have the following items for sale, SEE MY LIST (link below) for

details.

155 KV RECTIFIERS

100 mA continuous, 350 mA pulse

1-3=$12 ea., 4-8=$11 ea., 9+ =$10 ea.

NEON TRANSFORMERS

NT1.0  15 kV 30 mA  $17.00

NT2.0  12 kV 30 mA  $17.00

NT4.3  12 kV 30 mA  $14.00

and others

HV neon transformer wire, flexible *14* gauge ($1.00/ft) & std 16 gauge

($.50/ft)

SPECTRUM TUBES

Used, CENCO (made in USA), choice of 4 colors: helium, neon, mercury,

nitrogen.  Colorful demo with Tesla coil or HV source.  $18.00 ea., or

4/$65.00

VARIACS

(can probably be used at higher current for short periods, such as in a

Tesla coil)

#1  estimated 6A  $22.00

#2  marked 3A  $15.00

#3  marked 6A  $26.00

#4  estimated 15-20A  $70.00

METERS

0-500 mA AC, can measure transformer current directly  $10.00

Scale "0-130 kV", but meter input is 0-130 VAC  $10.00

HV CAPACITORS

Two types, high freq. MICA & oil-filled.  Various values & prices, see

list for details.

Please see the list, tell me what you're interested in, and include your

ZIP

CODE to determine shipping cost.

Also, if you have a web page and would like to "trade links",

please let me know.

Thanks,

Tedd

----------------------------------------------------------

My FOR-SALE list is at:  http://home.earthlink.net/~teddp

----------------------------------------------------------

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: coax for bypass caps?

Date: Mon, 23 Nov 1998 23:14:52 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Was wondering if RG-213/U coax could be used for

bypass capacitors. It has 30.7pfd/foot capacitance and a PE dielectric of about

100mils which at 1000v/mil should be good for around 100kv. If one used 10ft of

this coax you would end up with a 307pfd cap. What have I

overlooked?

Jon Rosenstiel

jonr@pacbell.net

From: "Bill Langston" <blangsto@iwvisp.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: pri coil design?

Date: Tue, 24 Nov 1998 10:11:14 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

-----Original Message-----

From: Mike Hammer <mhammer@misslink.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Monday, November 23, 1998 11:03 AM

Subject: Re: pri coil design?

>At 09:08 PM 11/22/98 -0800, you wrote:

>>Hello everyone, I am Bill Langston in Trona California.

>

>Four paralleled 15kv/60s? Thats some serious power.

>Even for a 6 inch secondary.

We are down to three 15 Kv, 60 ma NSTs for a while, till I can unpot and

repair one that went on the fritz.

>What is your cap size

I have two rolled caps insulated to 20 layers of 6 mil poly and in oil. I

can run either of them singly (and have for hours with no problem). Each

measures at .076 mfd. That capacity is not what I was trying for when I

built them, but once the oil was in and vacuum applied for several days, they

stabilized out at .076 mfd.

>and gap type and spacing and resulting spark output?

Our gap is a adjustable from 1 to 12, .012 in, gaps mounted in two

physically parallel rows, one over the other in 4" black PVC. We have a high

capacity fan mounded to draw air up through the gap. We have a space filler

taking up the open area in the 4" pipe (max airflow over conducting

surfaces). Our electrodes are 2" sections of 3/4" hard copper tubing.

>What size is your toroid?

Now we are using a 4" aluminum duct wrapped in metal tape and smoothed. I am

going to a 7" tube as soon as I get it finished.

We also tried several spheres with varying results. The most interesting was

with an 18" sphere, the entire coil disappeared in fire, blue glows here and

there, and huge streamers that reached to where I was trying to melt into a

crack to get away. I will never, never, again get that machine between me

and a clear way of escape from where ever we are operating.

I haven't tried that again because I do not understand what happened yet.

But whatever it was, it announced itself with much energy being sent

everywhere. I made my adult living with high energy physics and related

fields, but I have never been that close to that much light and noise

expressing itself as it did --- ever before! In the world of tax

dollar-supported design and research, things are far better controlled

[usually, at least the ones that can be talked about without being sent to jail].

For anyone who is just starting out:  Within 20 or 30 seconds of our first

serious, stable operation (about 50" streamers), my wife's brand new (that

very day brand new) microwave was sent to that happy place in the sky, and

every single electronic clock in our house was scrambled or reset.

We had a heavy gauge, totally separate RF ground system to the middle of the

backyard, we had (and still have) 3 sets of RF chokes and a grounded safety

gap. One set of those chokes was on the mains wiring going into my variac.

None of it helped. I had a long night that night.

We now run with a 20 amp EMI filter first on the mains supply. That seems to

work. The RF field this things puts out seems to be almost as amazing as the

amount of noise it makes.

>

>

>I would bet you are not even utilizing 1/2 of your

>available power and the rest is just vaporizing your

>gap electrodes.

>

I did not think of that. Any ideas of how to better utilize the power will

be greatly appreciated. Thanks again.

Bill Langston

blangsto@iwvisp.com

Peter Langston

plangsto@polymail.cpunix.calpoly.edu

X-Mailer: IPAD 2.0

Date: Tue, 24 Nov 1998 12:19:10 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coax for bypass caps?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:14 PM 11/23/98 -0800, you wrote:

>    Was wondering if RG-213/U coax could be used for bypass capacitors. It

>has 30.7pfd/foot capacitance and a PE dielectric of about 100mils which at

>1000v/mil should be good for around 100kv. If one used 10ft of  this coax

>you would end up with a 307pfd cap. What have I  overlooked?

>jonr@pacbell.net

Jon,

Sorry but mil-spec RG-213 coax has a maximum voltage

rating of 5000V RMS. Consumer grade coax is usually

much lower than this.

The commonly available RG-8 cables with the foamed

polyethylene dielectric has a maximum voltage rating

of only 1500V RMS.

I dont really approve of the use of bypass caps for neon

protection circuits anyway. If used they should be quite

small. On the order of 150 pf.

Bypass caps (and series chokes for that matter) are essentially

energy storage devices. When the main gap quits conducting

near the AC zero crossing this stored energy has to go somewhere.

That somewhere is across the neon secondary hopefully limited in

amplitude by your safety gap.

My thoughts on protection most recently have taken two approaches.

On lower power coils (single 30ma neon) mount the main gap right

at the neon with nearly zero lead length. No chokes, caps or safety

gap. The main gap is the safety gap. This has worked well on my small

coils and was first proposed by Malcolm Watts.

On my larger coils Ive been using a safety gap mounted right at

the neon with very short lead length and with the main gap mounted

close by. Small ferrite toroidal chokes are used between the safety

and main gaps to electrically isolate them. Only about 1 or 2 mh is used.

I no longer advocate the use of air core chokes with or without resistors.

Comments, criticism and flying bricks always welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 25 Nov 1998 08:56:52 +1200

Subject:       Re: coax for bypass caps?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Jon,

> From:           "Jon Rosenstiel" <jonr@pacbell.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        coax for bypass caps?

> Date sent:      Tue, 24 Nov 1998 20:14:52 +1300

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Was wondering if RG-213/U coax could be used for bypass capacitors. It =

> has 30.7pfd/foot capacitance and a PE dielectric of about 100mils which =

> at 1000v/mil should be good for around 100kv. If one used 10ft of this =

> coax you would end up with a 307pfd cap. What have I overlooked?

>

> Jon Rosenstiel   jonr@pacbell.net

You might want to rethink using coax as a bypass cap. A piece of coax

terminated at one end only or not terminated by its characteristic

impedance at both ends is a tuned circuit with appreciable Q.

Although the resonant frequency of a short piece is well above your

coil's frequency it can be shock excited by the discharging spark

gap. Just a thought.

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 24 Nov 1998 15:32:31 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: need video card

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I know this is off topic. This gives everyone else

a chance to jump on me for a change about off topic posts.

I need a PCI bus video card for my backup machine.

It has an ISA ATI Mach 32 that has produced the

last crash it will ever perform. I need to get

rid of this piece of garbage.

I plan on using it for a top load on my big coil.

(slightly on topic)

If anyone has a spare or old PCI bus video card

you want to part with cheap let me know. Will consider

a trade for coil and/or electronic parts.

Contact me directly off list.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 24 Nov 1998 15:50:58 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Which spark gap?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:58 PM 11/23/98 EST, you wrote:

>Really?  Where are the plans for a vacuum gap?

I dont recollect seeing any plans for a vacuum gap.

However its not really that complex. Construct a normal

static gap of your choice and suck air through it with a

vacuum cleaner instead of blowing air through with a fan.

Directing the airflow directly between the electrodes

provides best performance.

Ive built two vacuum gaps and they are great performers.

The better one is a pretty much standard cylinder gap

with the top and bottom sealed with disks of plexiglass

and 1/8th inch slots cut into the gap housing to admit

airflow between the gap electrodes.

The second one is patterned after the gap made by Gary Lau.

This was a cheap quick gap to build. $4 worth of parts from

Lowes and a couple hours for construction.

Take a look at Gary's web page for pictures.

http://people.ne.mediaone.net/lau/tesla/onegap.htm

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 24 Nov 1998 20:40:54 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Neons as Current limiting?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To all,

I was wondering if anyone knows if you can use a neon to current limit a

potential transformer? Or maybe an induction motor? If neither of these

will work and I DO NOT have a welder. If I were to wind my own inducter,

what type of wire and how large should the inductor be? By the way I

only have 20A breakers, so I will have to limit it to less than 20A of

input current.

Chris

X-Originating-IP: [216.68.41.136]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Neons as Current limiting?

Date: Tue, 24 Nov 1998 19:01:40 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>I was wondering if anyone knows if you can use a neon to current limit a

>potential transformer? Or maybe an induction motor?

You mean use an induction motor to limit the pig?  Thats a new idea....

Should work OK I guess if you can keep the RF out of it!

>If neither of these will work and I DO NOT have a welder.

>If I were to wind my own inductor, what type of wire and how

>large should the inductor be? By the way I only have 20A breakers,

>so I will have to limit it to less than 20A of input current.

Don't know but you'll need a core - a dead variac core would be perfect!

More than that I can't say.

Is your 20amps at 120 or 240v?

If 240v then you need < 31 mH of inductance.  Exact number of turns

depends on the core at hand...

Sounds like you have a pig and are anxious to use it...?

I'd try using resistive heaters first - just to get the hang of things.

A toaster oven works great at 120 volts...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: HPDE pipe

Date: Tue, 24 Nov 1998 22:35:36 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Another question for everyone. Does anyone have

experience using HPDE drain pipe as a secondary coil form? I bought a length of

this stuff at Home Depot, (was quite cheap). Was wondering if I should try it.

Thanks

jonr@pacbell.net

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flyback Coil system

Date: Tue, 24 Nov 1998 23:00:00 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Importance: Normal

Sender: tesl-err@emachine.com

I admit to my lack of knowledge when it comes to these systems, but I

thought it would be fun to build one, since I found myself with a flyback

from an old CGA monitor that was lying around the shop. (most now on its

way to the landfill) So I build the (in)famous 3055 based flyback driver,

and it works just fine - getting 3/4" solid arc (corona out to about 1").

The problem is that the 3055s are getting *HOT* as is the 3A bridge that

is feeding the circut. I am hoping that there is some error in design.

(if not, I already have good sized heatsinks on the 3055s, I'll just add

a fan to the mix.)

In addition to a request to check the below circut, some questions:

Should this be able to turn a standard bulb into a 'Plasma globe',

or is this too low a power?

Should it be holding a charge after power off (I get little snaps and

sparks for bit.)

I'll have to describe the circut, I hope I get it clear enough - as ascii

art sucks.

Out of the bridge/filter (8000uf filter cap) I have:

27ohm resistor  between (-) and the center tap of the feedback winding.

Continue on to a 270 ohm resistor that goes to the centertap of the

primary, and then back to the (+) side of the supply. Emitters to (-).

Collectors to each side of the primary winding. Bases to each side of

the feedback winding.  Primary winding is 10 turns CT, feedback 4 turns CT.

Sorry to be asking so many questions, but inquiring minds (mine) want to know:)

Thanks!

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: HPDE pipe

Date: Tue, 24 Nov 1998 23:04:10 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Importance: Normal

Sender: tesl-err@emachine.com

Jon:

you can try it.. but I make no promises as to how it will work. I have used that

HDPE pipe (white outside, black inside) with no problems whatsoever. Current arc

record for my coil is 48 inches which I intend to have fall this weekend.

The problem is that sometimes that black is the result of a carbon additive,

which can make the plastic semiconducting. I may have just gotten lucky, but

you never know. You can try it.. the worst case is that you waste wire and time...

the best case is you get good sparks. If I were a betting man, I would bet on the fact

that it works... only because mine did. (now black ABS.. I think that is totally bad

news.)

Michael Baumann

Coiler, Homebrewer, Nerd.

mycroft@access1.net

From: Hollmike@aol.com

Date: Wed, 25 Nov 1998 07:13:25 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HPDE pipe

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/24/98 11:34:15 PM Mountain Standard Time,

jonr@pacbell.net writes:

>  Was wondering if I should try it.

>

>  Thanks

>  Jon Rosenstiel   jonr@pacbell.net

>

it is supposed to be great for a secondary form.

Mike

Date: Wed, 25 Nov 1998 06:23:41 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HPDE pipe

Sender: tesl-err@emachine.com

Jon Rosenstiel wrote:

>

> Another question for everyone. Does anyone have experience using HPDE

> drain pipe as a secondary coil form? I bought a length of this stuff

> at Home Depot, (was quite cheap). Was wondering if I should try it.

>

> Thanks

> Jon Rosenstiel   jonr@pacbell.net

Jon,

HDPE alone is excellent. However, it's often colored with carbon black,

which has been known to degrade its dielectric strength. Try taking a

small piece and see how it withstands the high voltage from a neon sign

transformer versus a piece of clear LDPE. If it's noticeably worse, don't

use it, since the voltage stress on the secondary coil form will

carbon-track it.

-- Bert --

From: EdwMullin@aol.com

Date: Wed, 25 Nov 1998 12:47:38 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Coil system

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I built a 3055 driver to push an automotive ignition coil, it does get hot!

Heat sink, and thermal paste help.  I conect it to a 60w clear light bulb and

get some very nice blue/violet streamers inside.  So I figure that your

flyback transformer should really make a nice show.  Good luck

From: EdwMullin@aol.com

Date: Wed, 25 Nov 1998 12:56:52 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: coax for bypass caps?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 11/24/98 1:23:10 PM Eastern Standard Time,

mhammer@misslink.net writes:

<<

My thoughts on protection most recently have taken two approaches.

On lower power coils (single 30ma neon) mount the main gap right

at the neon with nearly zero lead length. No chokes, caps or safety

gap. The main gap is the safety gap. This has worked well on my small

coils and was first proposed by Malcolm Watts.

>>

Mike, would this be a single gap? or would it work with a series type?

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Flyback Coil system

Date: Wed, 25 Nov 1998 13:10:33 -0600

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Michael,

That is almost exactly the same flyback system I built a while ago.  Works well but the

power transistors and diode bridge will get hot.  I added a fan on the 4 X 6 inch heat

sink I used and that solved my heat problem.

Some light bulbs make excellent plasma globes.  Some don't.  I bought a 4 inch clear

globe bulb and it worked well with purple streamers coming off the filament supports

and dancing around the inside surface of the globe.  You could actually hear the

streamers hitting the inside of the globe.  After I put a hole in that bulb (by holding

a grounded wire to the outside surface of the bulb and the arc melted the glass) I bought another similar bulb but it was by a different manufacturer.  This new bulb wouldn't make

any streamers at all.  Reverting back to the original type and manufacture of bulb solved

the problem.  I've heard that some light bulbs have a small amount of inert gas in them

and that this is what glows and makes the streamers possible.

Yes watch out for the 8000uf cap it will give you a zap after you disconnect the power.

It can hold a charge for some time, even days.  So always discharge the cap by shorting

the + and - terminals when you are done experimenting.

I also built a similar circuit that can vary the frequency of the output voltage.

You can view a drawing and some text about it at http://home.earthlink.net/~gweaver/hv-page.htm on the High Voltage page it's the 8th project down the list.  I built it using

an old HEI coil from a 1978 General Motors 350cid truck engine.  It works well if you

leave the 1N914 diodes out.  Every time I tried to put them in the 555 timer chip would literally explode.  This circuit really screams when it is running.  My dogs hate it!

I made a really neat mini-Jacob's ladder with it.

Happy coiling,

Roy Minnich

----------

From: 	Coiler[SMTP:mycroft@access1.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, November 25, 1998 1:00 AM

To: 	Tesla-2 Mailing List

Subject: 	Flyback Coil system

I admit to my lack of knowledge when it comes to these systems, but I

thought it would be fun to build one, since I found myself with a flyback from

an old CGA monitor that was lying around the shop. (most now on its

way to the landfill)

So I build the (in)famous 3055 based flyback driver, and it works just

fine - getting 3/4" solid arc (corona out to about 1"). The problem

is that the 3055s are getting *HOT* as is the 3A bridge that is feeding

the circut. I am hoping that there is some error in design. (if not,

I already have good sized heatsinks on the 3055s, I'll just add a fan

to the mix.)

In addition to a request to check the below circut, some questions:

Should this be able to turn a standard bulb into a 'Plasma globe',

or is this too low a power?

Should it be holding a charge after power off (I get little snaps and

sparks for bit.)

I'll have to describe the circut, I hope I get it clear enough - as ascii

art sucks.

Out of the bridge/filter (8000uf filter cap) I have:

27ohm resistor  between (-) and the center tap of the feedback winding.

Continue on to a 270 ohm resistor that goes to the centertap of the

primary, and then back to the (+) side of the supply. Emitters to (-).

Collectors to each side of the primary winding. Bases to each side of

the feedback winding.  Primary winding is 10 turns CT, feedback 4 turns CT.

Sorry to be asking so many questions, but inquirings minds (mine) want to know:)

Thanks!

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Flyback Coil system

Date: Wed, 25 Nov 1998 13:21:36 -0600

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I also recommend Mike's Electric Stuff at http://i.am/volts for images and info.

on plasma globes and related tubes etc.

Roy Minnich

X-Mailer: IPAD 2.0

Date: Wed, 25 Nov 1998 13:39:56 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coax for bypass caps?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:56 PM 11/25/98 EST, you wrote:

>In a message dated 11/24/98 1:23:10 PM Eastern Standard Time,

>mhammer@misslink.net writes:

>

><<

> My thoughts on protection most recently have taken two approaches.

> On lower power coils (single 30ma neon) mount the main gap right

> at the neon with nearly zero lead length. No chokes, caps or safety

> gap. The main gap is the safety gap. This has worked well on my small

> coils and was first proposed by Malcolm Watts.

>  >>

>

>Mike, would this be a single gap? or would it work with a series type?

By "main gap" I meant the main gap system whatever

form that might take. In this instance the gap I was

using was a 9 electrode RQ/TCBOR cylinder gap.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 25 Nov 1998 13:57:48 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Coil system

Sender: tesl-err@emachine.com

Coiler wrote:

> So I build the (in)famous 3055 based flyback driver, and it works just

> fine - getting 3/4" solid arc (corona out to about 1"). The problem

> <SNIP>

I built one a few years back, (yes, hot transistors) filled a 10 inch

globe full of helium and partly evacuated it, got pink streamers. Here

are some shots of nitrogen streamers.

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\plasma1.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\plasma2.jpg"

(editors note. no binary attachments in the list archive.)

From: Scott_L_Hanson@notes.seagate.com

X-Lotus-FromDomain: SEAGATE@INTERNET

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 25 Nov 1998 12:28:37 -0800

Subject: Re: HPDE pipe

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Aside from the flimsy, thin-wall co-extruded (white outside, black inside)

HDPE  drain pipe, there are much more heavily constructed HDPE and MDPE

piping systems with many different diameters and wall thicknesses

available. I'm thinking about using some 6" or 8"  "thin-wall" (1/4") stuff

for my first secondary coil form . Has anyone had any difficulty in

bonding/securing the secondary windings to a polyethylene form? There is

almost no adhesive that will create a structural bond to PE.

Also, what is the actual drawback to using "lossy" PVC pipe? I know that in

reference tables of dielectric properties PVC has a much greater

dissipation factor than other polymers, but of what significance is that in

a Tesla coil application? If it is "lossy", what is the magnitude of the

loss, and how is it manifested? The only loss I could think would be of

dielectric heating, yet we don't hear of PCV coilforms deforming or

distorting due to heat build-up, even with prolonged run-times.

Aside from possibly reducing moisture absorption, what possible advantage

is there to painting a PVC coilform with insulating varnish or similar

coatings? What would an additional .005" to .010"  of winding to coil-form

spacing accomplish?

I'm just trying to get some rational answers, rather than the oft-repeated

warnings with no data to back them up.

Thanks for any input,

S. Hanson

Costa Mesa, CA

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 26 Nov 1998 10:12:59 +1200

Subject:       Re: Flyback Coil system

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Michael,

> I admit to my lack of knowledge when it comes to these systems, but I

> thought it would be fun to build one, since I found myself with a flyback from

> an old CGA monitor that was lying around the shop. (most now on its

> way to the landfill)

> So I build the (in)famous 3055 based flyback driver, and it works just

> fine - getting 3/4" solid arc (corona out to about 1"). The problem

> is that the 3055s are getting *HOT* as is the 3A bridge that is feeding

> the circut. I am hoping that there is some error in design. (if not,

> I already have good sized heatsinks on the 3055s, I'll just add a fan

> to the mix.)

Is it really a flyback design?  A true flyback would use a single or

paralleled output transistor, a transformer with a gapped core and

operate the core in one quadrant only. If you have a pushpull

arrangement, well, it is pushpull/forward and not a flyback,

regardless of what the transformer looks like.

> In addition to a request to check the below circut, some questions:

> Should this be able to turn a standard bulb into a 'Plasma globe',

> or is this too low a power?

>

> Should it be holding a charge after power off (I get little snaps and

> sparks for bit.)

>

> I'll have to describe the circut, I hope I get it clear enough - as ascii

> art sucks.

>

> Out of the bridge/filter (8000uf filter cap) I have:

> 27ohm resistor  between (-) and the center tap of the feedback winding.

> Continue on to a 270 ohm resistor that goes to the centertap of the

> primary, and then back to the (+) side of the supply. Emitters to (-).

> Collectors to each side of the primary winding. Bases to each side of

> the feedback winding.  Primary winding is 10 turns CT, feedback 4 turns

> CT.

OK, it is a pushpull design and it is self-oscillating. Such an

inverter mandates that the core have zero gapping in it as snappy

feedback operation relies on the core saturating fairly rapidly.

Is the core at all gapped? Glued together?  In this design, the

higher the primary inductance, the lower the quiescent (no load)

current it will draw. It is also important to use a suitable feedback

winding to primary ratio and a suitable choice of current limiting

base resistor/s.

Malcolm

Date: Wed, 25 Nov 1998 14:59:01 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Coil system

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

EdwMullin@aol.com wrote:

>

> I built a 3055 driver to push an automotive ignition coil, it does get hot!

> Heat sink, and thermal paste help.  I conect it to a 60w clear light bulb and

> get some very nice blue/violet streamers inside.  So I figure that your

> flyback transformer should really make a nice show.  Good luck

I built the one using a flyback and 3055 circuit and also the one

with the car coil and quadrac(triac and diac in one package)

I got a clear bulb and pasted tin foil to half of it and covered that

half with black tape, grounded that half also. Then I connected the base

of the light bulb to the top of the car coil, voila, it produced quick

multiple branching sparks in the bulb.

But with the flyback setup you don't need the foil because it works on a

much higher frequency, it produces nice blue corona in a clear light

bulb, you can even touch it bare handed which should never be attempted

using the car coil setup because the frequency is too low to produce

skin affect.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Wed, 25 Nov 1998 17:24:06 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HPDE pipe

Sender: tesl-err@emachine.com

Scott_L_Hanson@notes.seagate.com wrote:

>

> Aside from the flimsy, thin-wall co-extruded (white outside, black inside)

> HDPE  drain pipe, there are much more heavily constructed HDPE and MDPE

> piping systems with many different diameters and wall thicknesses

> available. I'm thinking about using some 6" or 8"  "thin-wall" (1/4") stuff

> for my first secondary coil form . Has anyone had any difficulty in

> bonding/securing the secondary windings to a polyethylene form? There is

> almost no adhesive that will create a structural bond to PE.

Polyethylene is slippery stuff for sure. I have not personally used this

material for a coilform, since PVC was available and much handier for me

to use.

>

> Also, what is the actual drawback to using "lossy" PVC pipe? I know that in

> reference tables of dielectric properties PVC has a much greater

> dissipation factor than other polymers, but of what significance is that in

> a Tesla coil application? If it is "lossy", what is the magnitude of the

> loss, and how is it manifested? The only loss I could think would be of

> dielectric heating, yet we don't hear of PCV coilforms deforming or

> distorting due to heat build-up, even with prolonged run-times.

Practically speaking, you can wind your secondary on PVC coil form and

get excellent results. While you can instrument a slight decrease in Q

between identical coils wound on PVC versus Polyethylene coilforms, once

the sparks start flying, there's virtually NO performance difference.

PVC also has excellent dielectric strength, and has been succesfully

used on some of the best performing systems.

>

> Aside from possibly reducing moisture absorption, what possible advantage

> is there to painting a PVC coilform with insulating varnish or similar

> coatings? What would an additional .005" to .010"  of winding to coil-form

> spacing accomplish?

Coilers sometimes wind their coils on PVC pipe that's been out in the

sun (UV light) and weather for awhile. Under these adverse conditions,

the outer surface of the PVC can oxidize a bit and this layer can absorb

a bit of moisture, increasing its leakage current and reducing it

voltage standoff capability. Sanding, drying, and then sealing the PVC

form with an insulating varnish removes the problem layer and makes the

form less sensitive to high humidity conditions. You also want to sand

off any lettering on the pipe since black ink (carbon black pigment?)

has been known to cause coilform insulation failures.

>

> I'm just trying to get some rational answers, rather than the oft-repeated

> warnings with no data to back them up.

>

> Thanks for any input,

>

> S. Hanson

> Costa Mesa, CA

You should have excellent luck with either coilform material. Keep us

posted on your progress!

-- Bert --

From: "Stephen Bell" <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HPDE pipe

Date: Wed, 25 Nov 1998 23:57:20 -0000

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Bert Hickman <bert.hickman@aquila.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Re: HPDE pipe

> Date: 25 November 1998 12:23

>

> Jon Rosenstiel wrote:

> >

> > Another question for everyone. Does anyone have experience using HPDE

> > drain pipe as a secondary coil form? I bought a length of this stuff

> > at Home Depot, (was quite cheap). Was wondering if I should try it.

> >

> > Thanks

> > Jon Rosenstiel   jonr@pacbell.net

>

> Jon,

>

> HDPE alone is excellent. However, it's often colored with carbon black,

> which has been known to degrade its dielectric strength. Try taking a

> small piece and see how it withstands the high voltage from a neon sign

> transformer versus a piece of clear LDPE. If it's noricably worse, don't

> use it, since the voltage stress on the secondary coilform will

> carbon-track it.

>

> -- Bert --

>

Hi Guys,

I tested some HDPE tubing with a 10kV NST which did use carbon to colour

it black but it showed no signs of conduction. I think higher voltage or

higher frequency is needed to fully test it, i.e. wind a secondary and see

it pass or fail!

Steve B.

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Flyback Coil system

Date: Wed, 25 Nov 1998 21:04:24 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Importance: Normal

Sender: tesl-err@emachine.com

Stefan: thanks for the tip off here. I was trying to figure out

why I could not get the plasma sphere to ignite except for

a moment. Turns out this coil must have the embedded diode/cap

The corona was only when there were points facing each other, not

to free air. I found that I can charge myself up with the output

[standing on 2ft of plastic] and then discharge myself to ground

with a healthy snap. So....

Is is possible to dissect out the diode, or am I going to have to

switch to a auto coil solution for a small plasma globe?

> Hmmm. Flybacks with internal diodes hold some charge after power of

> because of their built in capacitor, but if it produces corona, there is

> no diode built in.

>

>

> Bye,

>  Stefan

>

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 26 Nov 1998 07:40:18 +0200

Subject: Filled capacitor orientation

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I have seen several pictures of oil-filled rolled capacitors that are used

in horizontal position.

I am near to fill my first capacitor with oil and I wonder this: doesn't

the air that I leave inside the cap over the oil possibly get in contact

with the capacitor layers if I turn it 90 degrees (if I use it horizontal)?

Wouldn't that spoil all the work of getting the air out of the layers?

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 26 Nov 1998 07:40:36 +0200

Subject: IMPORTANT: RESEARCH ON WHAT?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

From: Marco Denicolai@MARTIS on 26.11.98 07:40

To:   Tesla List2 <tesla-2@emachine.com>

cc:

Subject:  IMPORTANT: RESEARCH ON WHAT?

To make a long story brief, I'll just tell you that the High Voltage

Institute of Helsinki (part of the Helsinki University of Technology) is

interested in Tesla Coils.

They are going to support me financially in order to build one (or more)

Tesla Coils and to make extensive research on it. That will constitute my

PhD thesis.

They have got facilities and measurement instruments we have always been

dreaming about and they are researching only about high voltage and related

topics.

Having such a great opportunity I have to lay down a proposal about WHAT I

am going to research about and here I need YOUR help, because, after all, I

am just a beginner who has built one TC only.

My questions are:

1. What is still not understood about Tesla Coils and what would be worth

investigating about? In that laboratory I am confident I could, for

instance, measure all the parameters of a TC you have always been dreaming

about and try to make a model of it.

2. Would it be more challenging insteas of a 2-coil TC to build and measure

a 3-coil TC (Magnifier)?

3. What you would start researchin about, if you were me?

P.S. I promise I'll keep the list all the time constantly informed about

results and findings, but please help me to find a good subject. I don't

want to discover something that has already been known by the majority of

you. Instead I think some of you could have some technical questions

bothering but no possibility to find answers for missing of instrumentation

or facilities.

Marco Denicolai

Tellabs Oy                        tel: +358 9 4131 2769

Access Networks - xDSL Products   mobile: +358 50 353 9468

Sinikalliontie 7                  fax: +358 9 4131 2410

02630 Espoo  FINLAND              email: marco.denicolai@tellabs.fi

Date: Wed, 25 Nov 1998 23:41:56 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: [Fwd: Re: Flyback Coil system]

Sender: tesl-err@emachine.com

Message-ID: <365C613C.D77@prismnet.com>

Date: Wed, 25 Nov 1998 13:57:48 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: tpolk@prismnet.com

X-Mailer: Mozilla 3.0Gold (Win95; I)

MIME-Version: 1.0

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Subject: Re: Flyback Coil system

Coiler wrote:

> So I build the (in)famous 3055 based flyback driver, and it works just

> fine - getting 3/4" solid arc (corona out to about 1"). The problem

> <SNIP>

I built one a few years back, (yes, hot transistors) filled a 10 inch

globe full of helium and partly evacuated it, got pink streamers. Here

are some shots of nitrogen streamers.

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\plasma11.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\plasma21.jpg"

(editors note. binary attachments not in archive.)

Date: Thu, 26 Nov 1998 00:21:42 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: IMPORTANT: RESEARCH ON WHAT?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> 3. What you would start researching about, if you were me?

Magnifiers get such hot press. I would like to know once and for all if

they really generate more output than a two coil system.

Date: Thu, 26 Nov 1998 00:45:32 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New pictures on my site

Sender: tesl-err@emachine.com

I finally got some time to put more spark pictures up on my site.

http://www.prismnet.com/~tpolk/coils/coils.html

The longest arcs are just over four feet.

I should name the coil Bumblebee. According to physicists and

aerodynamicists, bumblebees can't fly.  But they do.

To prove to new coilers anything is possible, my coil was built without

the excellent help this list provides. It still contains lots of second

class engineering:

For example the secondary is 1600 turns of #26 wire, too long and too

skinny a wire. The primary is a 2" wide strip of copper wound in a very

close spiral, about 12" in diameter. Copper strapping is inferior to

copper tubing, I understand.

The transformer is a 30 ma 15kv Franceformer with the shunts COMPLETELY

removed, resulting in saturation. It runs hot, and it is cooled by

immersion in an oil-filled cardfile box. It has a horn type safety gap

that rarely fires. Total gap is 7/16", which I understand is way too

wide.

There is no ballast and no resistors, only hookup wire wound in several

layers around iron pipe and a 500pf safety cap.

The gap is a 21 point 1400 rpm rotary; rotaries are not good matches for

NSTs.

The poly cap uses 1/8" single layers of LDPE, vs. safer

multi-thicknesses.

After all I've learned, this thing shouldn't work as well as it does,

and certainly shouldn't have held up through four Halloweens! Enjoy the pics!

X-Originating-IP: [195.121.16.57]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject:  Plasma Globes

Date: Thu, 26 Nov 1998 08:18:41 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> I have never tried using an ignition coil to power a plasma globe,

> but I don't think it works very well.

I have: I used my double ignition coil driver to power two plasma

globes. I had it set for high frequency (over 100 pulses per second) and

connected each output wire to a different bulb. They worked fine, you

could hear the noise of the streamers dancing aound and they were very

visible when you touched the bulbs. Touching them was very painful

though, and once a friend of mine touched me on the back when I was

holding a bulb in my hand and we were both thrown back.

So, yes it works but it for the brave only. One interesting thing is to

use one of those neon bulbs, like the ones with flamingos, or flames or,

better yet, a sphere on the center. They make excellent plasma globes.

X-Originating-IP: [207.100.20.20]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Flyback Coil system

Date: Thu, 26 Nov 1998 14:59:20 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>

>Hi Michael,

>

>That is almost exactly the same flyback system I built a while ago.

>Works well but the power transistors and diode bridge will get hot.

>I added a fan on the 4 X 6 inch heat sink I used and that solved my

>heat problem.

>

>Some light bulbs make excellent plasma globes.  Some don't.  I bought a

>4 inch clear globe bulb and it worked well with purple streamers coming

>off the filament supports and dancing around the inside surface of the

>globe.  You could actually hear the streamers hitting the inside of the

>globe.  After I put a hole in that bulb (by holding a grounded wire to

>the outside surface of the bulb and the arc melted the glass) I bought

>another similar bulb but it was by a different manufacturer.  This new

>bulb wouldn't make any streamers at all.  Reverting back to the original

>type and manufacture of bulb solved the problem.  I've heard that some

>light bulbs have a small amount of inert gas in them and that this is

>what glows and makes the streamers possible.

>

>Yes watch out for the 8000uf cap it will give you a zap after you

>disconnect the power.  It can hold a charge for some time, even days.

>So always discharge the cap by shorting the + and - terminals when you

>are done experimenting.

>

>I also built a similar circuit that can vary the frequency of the output

>voltage.  You can view a drawing and some text about it at

>http://home.earthlink.net/~gweaver/hv-page.htm on the High Voltage page

>it's the 8th project down the list.  I built it using an old HEI coil

>from a 1978 General Motors 350cid truck engine.  It works well if you

>leave the 1N914 diodes out.  Every time I tried to put them in the 555

>timer chip would literally explode.  This circuit really screams when it

>is running.  My dogs hate it!  I made a really neat mini-Jacob's ladder

>with it.

>

>Happy coiling,

>

>Roy Minnich

I made the same schematic and it runs fine with the diodes in.

Phililp Mac Duffie

Date: Thu, 26 Nov 1998 17:03:08 -0800

From: Cliff Blackburn <repairco@axionet.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi I have built a plasma globe from the plans out of a Electronics now

magazine but it doesn't work. I have posted the plans to my site at

http://members.xoom.com/dlktdr

I would appreciate it if someone could look at these plans and see

it there is an error or something. I would also like to know whether anyone

has built one like this before and if it worked or not. The only thing

I did differently is used a different chip. It supposedly does the same

thing as the one I was supposed to use but maybe not. maybe the flyback

primary is wound wrong. The modifyed flyback pic is on there too.

Thanx for any help.

Date: Thu, 26 Nov 1998 20:13:56 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Cliff Blackburn wrote:

>

> Hi I have built a plasma globe from the plans out of a Electronics now

> magazine but it doesn't work. I have posted the plans to my site at

> http://members.xoom.com/dlktdr and I would appreciate it if someone

> could look at these plans and see it there is an error or something. I

> would also like to know whether anyone has built one like this before

> and if it worked or not. The only thing I did differently is used a

> different chip. It supposedly does the same thing as the one I was

> supposed to use but maybe not. maybe the flyback primary is wound

> wrong. The modifyed flyback pic is on there too.

>

> Thanx for any help.

>  Mail <------ me here

Cliff,

The schematic (Fig. 1) is missing a ground connection to the Source

terminals of each MOSFET, although the layout drawing (Fig. 2) does show

this connection. If this is not the problem, the next likely candidate

may be the PWM IC you're using - unless it's pin-for-pin compatable with

the SG3525, it may not operate correctly. Try to borrow an oscilloscope

to see if the PWM is outputing square waves to the gates of the

transistors. If you ARE getting square-wave signals to the gates of the

MOSFET's, it's possible that you may have damaged one or both of the

MOSFET's during handling through electrostatic discharge (ESD), and

they're not functioning properly. Replacement will be necessary.

Good Luck!

-- Bert --

From: "Malcolm Watts" <malcolm@directorate.wnp.ac.nz>

Organization: Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 27 Nov 1998 14:43:47 +1200

Subject: Re: Flyback Plasma Globe

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Cliff,

> Date:          Fri, 27 Nov 1998 14:03:08 +1300

> From:          Cliff Blackburn <repairco@axionet.com>

> To:            "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:       Flyback Plasma Globe

> Reply-to:      "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi I have built a plasma globe from the plans out of a Electronics now

> magazine but it doesn't work. I have posted the plans to my site at

> http://members.xoom.com/dlktdr and I would appreciate it if someone could

> look at these plans and see it there is an error or something. I would

> also like to know whether anyone has built one like this before and if it

> worked or not. The only thing I did differently is used a different chip.

Which chip did you use?  Did you consult the data sheets to check for

compatibility with the original?

The supply BTW is a pushpull forward converter, not a flyback.

> It supposedly does the same thing as the one I was supposed to use but

> maybe not. maybe the flyback primary is wound wrong. The modifyed flyback

> pic is on there too.

Looks as if the two halves of the primary are bifilar wound. Should

be fine. Do you have an oscilloscope? Check for squarewave drive to

the bases of the two MOSFETs. Check for a ramp waveform on the

timing capacitor pin of the IC.  If you used a different IC, chances

are it is not compatible with the original specified.

Malcolm

Date: Thu, 26 Nov 1998 23:10:54 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Filled capacitor orientation

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I have seen several pictures of oil-filled rolled capacitors that are used

> in horizontal position.

>

> I am near to fill my first capacitor with oil and I wonder this: doesn't

> the air that I leave inside the cap over the oil possibly get in contact

> with the capacitor layers if I turn it 90 degrees (if I use it horizontal)?

>

> Wouldn't that spoil all the work of getting the air out of the layers?

Marco,

Even commercially manufactured cylindrical caps will have an air bubble

inside the container to allow for some "give" when the capacitor heats

up and expands a bit. Once the air has been removed from between the

layers on the roll, there's really little tendancy for this air to

migrate back inside the capacitor roll - capillary action of the

dielectric fluid between the layers tends to keep it out.

-- Bert --

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 27 Nov 1998 09:08:37 +0200

Subject: RESEARCH AT HUT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I installed yesterday my TC at HUT (Helsinki University of Technology) and,

luckly, it performed well without breaking down. The audience was very impressed.

I am now laying down a tentative action plan about my PhD work. This

includes the following steps:

1. Acquire a good set of the currently available literature on Tesla Coils.

2. Write a top-down specification of one (or two) TC to be built: at least

one of the TCs would include plenty of measurement points and devices.

3. Build the TCs.

4. Measure as much as possible.

5. Device a model supporting my measurements: in practise that will be a

refinement of (and addition to) currently known formulas and theories.

6. From this initial model and set of measures pick an area of special

interest (e.g. the streamer's formation) and investigate deeply into it.

I was thinking to build one or two bigger TCs (2-coil systems) but I am

afraid that going bigger than a certain size I would get very easily

strikes to the strike rail and nothing more. Is this true?

For instance, with a TC with a secondary of 16", 48" tall (3:1):

- how long streamers do you think I could get, at  most?

- will I be limited by the distance to the strike-rail (is it a must to use

3-coil systems for long streamers)?

- how much power (kW) do you think that TC would require?

P.S. Among issues that my research is NOT going to touch are:

- home made capacitors (we'll buy them ready)

- NSTs: we'll use a pig or a commercial HV transformer.

Date: Fri, 27 Nov 1998 09:43:15 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: RESEARCH AT HUT

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

<SNIP>

> I was thinking to build one or two bigger TCs (2-coil systems) but I am

> afraid that going bigger than a certain size I would get very easily

> strikes to the strike rail and nothing more. Is this true?

Actually, with the appropriate coil length and careful management of the

e-field at the top of the coil using a single large toroid or a stacked

toroid arrangement, it's possible to make MOST of the streamers head

outwards or upwards rather than downwards to the secondary or

strikerail. With carefull design you should be able to achieve streamers

that are 3-4X the secondary coil length.

>

> For instance, with a TC with a secondary of 16", 48" tall (3:1):

>

> - how long streamers do you think I could get, at  most?

10-12 feet with no problem, and >15 feet with careful attention to

e-field management (it's actually more of an electrostatics design

problem!)

> - will I be limited by the distance to the strike-rail (is it a must to use

> 3-coil systems for long streamers)?

Either way will work. The two-coil approach would be a bit simpler to

construct and tune.

> - how much power (kW) do you think that TC would require?

You will need at least 5 KVA to even begin properly firing it up.

Maximum power handling capacity will be more a function the capabilities

of the rest of your system and their efficiencies, but a coil this size

should be capable of >15 KVA. BTW, the 3:1 H:D ratio is not especially

critical. Increasing the coilform length a bit and space-winding or

using PVC-insulated wire can increase the secondary's voltage withstand

capability so that you can run at even higher power levels.

>

> P.S. Among issues that my research is NOT going to touch are:

> - home made capacitors (we'll buy them ready)

> - NSTs: we'll use a pig or a commercial HV transformer.

Wise decision! :^)

Good Luck!

-- Bert --

From: DavidH8083@aol.com

Date: Fri, 27 Nov 1998 11:23:06 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: I'm not that brave.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Experimenters at the early part of this century seem to be made of sterner

stuff than those of us at this end.  Yesterday I managed to escape the holiday

turmoil by curling up with a recently acquired copy of THE TESLA HIGH

FREQUENCY COIL by George Haller and E. T. Cunningham (1910). In it, he

suggests a number of showy experiments to be performed with a Tesla Coil. He

begins simply enough:

"After he has become somewhat impressed with the intensity of the discharge,

the operator can approach one of the oscillators and allow the spark to play

on his bare hand. To do this without severe burning of the hand, he should

keep his hand in constant motion to prevent the spark from playing on one

point. There is only a very slight sensation felt when this current is

traversing your body and no injurious effects result so far as the authors are aware."

But then he quickly moves to:

"The next experiment is to grip one oscillator with the hand while the unit is

in operation and to have an assistant touch the operator's bare elbow with a

cotton cloth dipped in alcohol. The handkerchief immediately bursts in flame."

or, even better:

"A gas flame can be lit with the bare fingers by grasping one oscillator with

the bare hand and approaching the burner with the finger of the the other hand

so that a spark will jump to the metal tip of the burner. The writers have

made this more spectacular by letting the spark jump from the tip of the

tongue."

Call me chicken, if you will; but I just don't have the balls to try this stunt.

David Hutchison

davidh8083@aol.com

Date: Fri, 27 Nov 1998 12:09:17 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Questions about a potential transformer.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I recently bought a potential transformer that is rated a 7.2kv@1.2kva.

The transformer has a thermal burden of 30 degrees celcius at 1.2kva.

The name plate states that it has ratings of IC 15kv, BIL 110kv.

What do those two numbers mean? I would also like to know if anyone

knows how many watts I can safely get out of this transformer without

risking its "life".

So far I have tested it on my coil once. I limited the transformer to

about 10-15 amps (I do not have an ammeter so I have to guess) using a

microwave transformer with its secondary shorted. I would assume that

the coil was running between 1.15-1.725kw. I was using a .02uf cap and a

cylindrical static gap (which would get very hot). My maximum spark

length was 30 inches to ground.

Thanks for any input,

Chris

Date: Fri, 27 Nov 1998 14:43:39 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: I'm not that brave.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

DavidH8083@aol.com wrote:

>

> Experimenters at the early part of this century seem to be made of sterner

> stuff than those of us at this end.  Yesterday I managed to escape the holiday

> turmoil by curling up with a recently acquired copy of THE TESLA HIGH

> FREQUENCY COIL by George Haller and E. T. Cunningham (1910). In it, he

> suggests a number of showy experiments to be performed with a Tesla Coil. He

> begins simply enough:

>

> "After he has become somewhat impressed with the intensity of the discharge,

> the operator can approach one of the oscillators and allow the spark to play

> on his bare hand. To do this without severe burning of the hand, he should

> keep his hand in constant motion to prevent the spark from playing on one

> point. There is only a very slight sensation felt when this current is

> traversing your body and no injurious effects result so far as the authors are

> aware."

>

> But then he quickly moves to:

>

> "The next experiment is to grip one oscillator with the hand while the unit is

> in operation and to have an assistant touch the operator's bare elbow with a

> cotton cloth dipped in alcohol. The handkerchief immediately bursts in flame."

>

> or, even better:

>

> "A gas flame can be lit with the bare fingers by grasping one oscillator with

> the bare hand and approaching the burner with the finger of the the other hand

> so that a spark will jump to the metal tip of the burner. The writers have

> made this more spectacular by letting the spark jump from the tip of the

> tongue."

>

> Call me chicken, if you will; but I just don't have the balls to try this

> stunt.

>

There is a difference between having balls and having brains, you should

say you are not dumb enough to try this.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Flyback Plasma Globe

Date: Fri, 27 Nov 1998 16:48:23 -0600

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi I saw the replies from Bert and Malcolm and I have looked at your page.

This seems an overly complicated circuit for what you want to do.  The MOSFETs

are rather fragile in respect to ESD too.  I have included a drawing of a more

rugged circuit that I have built and it works fine.  also a lot of flyback T-formers

had diodes mounted in the secondary housing in series with the HV output wire.

These diodes can limit the output or keep it from working all together.  Sometimes

you can remove them, sometimes you can't.  The best flybacks are from old large B&W TVs.

[[ FLYBACK.GIF : 5660 in WINMAIL.DAT ]]

Good luck,

Roy Minnich

----------

From: 	Cliff Blackburn[SMTP:repairco@axionet.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, November 26, 1998 7:03 PM

To: 	Tesla-2 Mailing List

Subject: 	Flyback Plasma Globe

<<File: ATT00001.htm>>

Hi I have built a plasma globe from the plans out of a Electronics now

magazine but it doesn't work. I have posted the plans to my site at

http://members.xoom.com/dlktdr and I would appreciate it if someone could

look at these plans and see it there is an error or something. I would

also like to know whether anyone has built one like this before and if it

worked or not. The only thing I did differently is used a different chip.

It supposedly does the same thing as the one I was supposed to use but

maybe not. maybe the flyback primary is wound wrong. The modifyed flyback

pic is on there too.

Thanx for any help.

Mail <------ me here

Date: Sat, 28 Nov 1998 00:10:49 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Questions about a potential transformer.

Sender: tesl-err@emachine.com

mel wise wrote:

>

> I recently bought a potential transformer that is rated a 7.2kv@1.2kva.

> The transformer has a thermal burden of 30 degrees celcius at 1.2kva.

> The name plate states that it has ratings of IC 15kv, BIL 110kv.

> What do those two numbers mean? I would also like to know if anyone

> knows how many watts I can safely get out of this transformer without

> risking its "life".

>

> So far I have tested it on my coil once. I limited the transformer to

> about 10-15 amps (I do not have an ammeter so I have to guess) using a

> microwave transformer with its secondary shorted. I would assume that

> the coil was running between 1.15-1.725kw. I was using a .02uf cap and a

> cylindrical static gap (which would get very hot). My maximum spark

> length was 30 inches to ground.

>

> Thanks for any input,

> Chris

Chris,

Utilities use a system called "insulation coordination" to protect their

equipment. Every piece of transmission or distribution equipment is

rated by an Insulation Class (IC) and a Basic Impulse Level (BIL). This

allows power company engineers to design the system such that the

"weakest link" under overvoltage conditions will always be an

overvoltage arrestor (typically a lightning arrestor) connected to the

circuit. Under abnormally high voltage conditions, the arrestor will

"fire" before the voltage reach the point where it can damage the

equipment on the circuit. Your particular potential transformer is in

the 15 kV Insulation Class, and it will withstand a transient impulse of

110 kV. The transient is assumed to be a international standard

"lightning like" waveform which goes from 0 to 110 kV in 1.2 uSec, then

tails off to 50% (55 kV) of the peak value after 50 uSec. Potential

transformers are MUCH more tolerant of high voltage stress than neon

sign transformers.

While potential transformers are pretty rugged beasts, they are still

not as rugged as pole pigs, since most pt'S are insulated with a solid

[Silicone rubber/RTV or epoxy] impregnent, and don't have the benefit

convective oil cooling. Their capability of handling extended overload

conditions is thus reduced compared to a pig.  However, you still should

be able to draw 2X the current rating for short (3-5 minute) runs, and

3X (or even more) for shorter runs. However, when overloading, it's

important that you don't run for extended periods of time since it takes

a while for internally built-up heat to escape. Also, make sure you

provide a cool-down time at least twice as long as your run time to

permit hot-spot temperatures to redistribute inside the transformer.

Hope this helped, and safe coilin' to you!

-- Bert --

Sender: tesl-err@emachine.com

Date: Sat, 28 Nov 1998 11:50:13 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Unpotting

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

> 60 MA transformers have significantly larger wire, and can withstand

> more overcurrent abuse that the hair-like 30 MA secondaries. You should

> easily be able to get 90-100 MA from one of these if immersed in oil.

>

> -- Bert --

Would it be possible to rewind the secondaries of a 15kv Neon with thicker wire to

give say....7,500V at 300ma?

Just curious....

-tom

From: McFalc0n@aol.com

Date: Sat, 28 Nov 1998 14:37:33 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Unpotting

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone know if a 15KV 30MA NST could or would work if you made it 100ma?

How high could one tweak it?  ThAnx.

Date: Sat, 28 Nov 1998 14:01:30 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Unpotting

Sender: tesl-err@emachine.com

McFalc0n@aol.com wrote:

>

>         Does anyone know if a 15KV 30MA NST could or would work if you made it 100ma?

> How high could one tweak it?  ThAnx.

Unfortunately, 100 MA from a 30 MA neon is asking for a bit too much.

You should be able to get as much as 50 MA for brief run-times, but

trying to get 100 MA will quickly overheat the inner windings, causing

the secondary to rapidly fail, even with the transformer completely

immersed in cooling oil.

60 MA transformers have significantly larger wire, and can withstand

more overcurrent abuse that the hair-like 30 MA secondaries. You should

easily be able to get 90-100 MA from one of these if immersed in oil.

-- Bert --

Date: Sat, 28 Nov 1998 19:56:15 -0800

From: Cliff Blackburn <repairco@axionet.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi I just wanted to say I got the globe working. There is one small

problem though. There is arcing between components on the new flyback.

By the way the problem I was having was not with the circuit but with

the flyback. I had to replace it with another one because the old one

had a connection problem with the hv return. If you could help me to

stop the arcing I would be grateful. If you forgot where the plans were

they are on my site  http://members.xoom.com/dlktdr

thanks for any info. I was thinging I might get the new primary coated

unless there in another way. I have gotten 1/2" sparks on 1/4 power.

before arcing.

Date: Sat, 28 Nov 1998 21:58:59 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Cliff Blackburn wrote:

>

> Hi I just wanted to say I got the globe working. There is one small

> problem though. There is arcing between components on the new flyback.

> By the way the problem I was having was not with the circuit but with

> the flyback. I had to replace it with another one because the old one

> had a connection problem with the hv return. If you could help me to

> stop the arcing I would be grateful. If you forgot where the plans were

> they are on my site  http://members.xoom.com/dlktdr

> thanks for any info. I was thinging I might get the new primary coated

> unless there in another way. I have gotten 1/2" sparks on 1/4 power.

> before arcing.

Cliff,

Just where is it arcing on the transformer?

-- Bert --

Date: Sat, 28 Nov 1998 22:46:00 -0800

From: Cliff Blackburn <repairco@axionet.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert Hickman wrote:

> Cliff Blackburn wrote:

> >

> > Hi I just wanted to say I got the globe working. There is one small

> > problem though. There is arcing between components on the new flyback.

> > By the way the problem I was having was not with the circuit but with

> > the flyback. I had to replace it with another one because the old one

> > had a connection problem with the hv return. If you could help me to

> > stop the arcing I would be grateful. If you forgot where the plans were

> > they are on my site  http://members.xoom.com/dlktdr

> > thanks for any info. I was thinging I might get the new primary coated

> > unless there in another way. I have gotten 1/2" sparks on 1/4 power.

> > before arcing.

>

> Cliff,

>

> Just where is it arcing on the transformer?

>

> -- Bert --

It is arcing between the primary and the core. the secondary and the core and

old primary to the core(I cannot remove this) unless someone could tell me a

good way to remove tar.

I will try to take a pic and send it to my site so you could see but that

might take a while.

One other question I had is if anyone knows where to buy an evacuated bulb

with a spear core and thick glass.

Date: Sun, 29 Nov 1998 00:58:19 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Cliff Blackburn wrote:

>

> Bert Hickman wrote:

>

> > Cliff Blackburn wrote:

> > >

> > > Hi I just wanted to say I got the globe working. There is one small

> > > problem though. There is arcing between components on the new flyback.

> > > By the way the problem I was having was not with the circuit but with

> > > the flyback. I had to replace it with another one because the old one

> > > had a connection problem with the hv return. If you could help me to

> > > stop the arcing I would be grateful. If you forgot where the plans were

> > > they are on my site  http://members.xoom.com/dlktdr

> > > thanks for any info. I was thinging I might get the new primary coated

> > > unless there in another way. I have gotten 1/2" sparks on 1/4 power.

> > > before arcing.

> >

> > Cliff,

> >

> > Just where is it arcing on the transformer?

> >

> > -- Bert --

>

> It is arcing between the primary and the core. the secondary and the core and

> old primary to the core(I cannot remove this) unless someone could tell me a

> good way to remove tar.

> I will try to take a pic and send it to my site so you could see but that

> might take a while.

>

> One other question I had is if anyone knows where to buy an evacuated bulb

> with a spear core and thick glass.

Cliff,

The beginning (innermost turns) of the secondary winding MUST be

connected to ground. It sounds like this might not be the case, and

instead you're getting a flashover from the innermost turns of the

secondary to the semiconducting ferrite core, and then from the core

back to the primary, and finally to ground through C4 and C5. The old

primary should just be floating - just cut the old leads short. Are you

using an unsulating sleeve between the core and the new bifilar primary

winding? If not, you'll need to add one to prevent possible spark

breakdown to the core.

A Philips clear glass 40 Watt round decorative light bulb works great.

Some "roll your own" plasma experimenters have also used round Pyrex

chemistry flasks with good success. Remember to protect your eyes at all

times in case the bulb fails under vacuum!

Good luck

-- Bert --

Date: Sun, 29 Nov 1998 09:22:41 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

Sender: tesl-err@emachine.com

Cliff Blackburn wrote:

>

> Bert Hickman wrote:

>

> > Cliff Blackburn wrote:

> > >

> > > Hi I just wanted to say I got the globe working. If you forgot where the plans were

> > > they are on my site  http://members.xoom.com/dlktdr

> One other question I had is if anyone knows where to buy an evacuated bulb

> with a spear core and thick glass.

Cliff,

Most science supply type stores will sell chemixtry glassware, retorts

flasks, etc.  some tubing, valves and a pump etc would put you into a

globe or whatever....   Also there are clear glass lg base lamps with

bulb diameters near 6".

----------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: McFalc0n@aol.com

Date: Sun, 29 Nov 1998 15:29:04 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Marx Generator

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry, this isn't really about tesla coils, but does anyone have good plans,

design tips, ideas, on making marx generators?  Thanx.

From: EdwMullin@aol.com

Date: Sun, 29 Nov 1998 18:29:11 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flyback Plasma Globe

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

You may try  a lighting store, for large clear globes used to make hanging

lights etc. they can be had as large as 8 or 9 inches...

From: McFalc0n@aol.com

Date: Sun, 29 Nov 1998 18:34:17 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 1350 Watts

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Will a 10 amp variac withstand running 3 or 4 NST's (15KV 30MA each)?

Date: Sun, 29 Nov 1998 18:10:45 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 1350 Watts

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

McFalc0n@aol.com wrote:

>

>         will a 10 amp variac withstand running 3 or 4 NST's (15KV 30MA each)?

Probably so, if they all have power factor correction on them.

Chris

X-Mailer: IPAD 2.0

Date: Sun, 29 Nov 1998 19:21:26 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:50 AM 11/28/98 +0000, you wrote:

>>

>> 60 MA transformers have significantly larger wire, and can withstand

>> more overcurrent abuse that the hair-like 30 MA secondaries. You should

>> easily be able to get 90-100 MA from one of these if immersed in oil.

>>

>> -- Bert --

>

>Would it be possible to rewind the secondaries of a 15kv Neon with

>thicker wire to give say....7,500V at 300ma?

>

>Just curious....

>-tom

Tom,

No this really wouldnt be possible. Take for instance a 15kv/60ma

neon. This has a core sized for 900va. The primary winding is sized

for the expected current draw at that power level. You can draw SOME

additional power without problems but if you try to draw much more

the primary will overheat and the core will saturate.

Your example of 7500/300ma is 2250 va. Well over twice the core

rating. Just wont work even with a heavier secondary winding.

The core will saturate and losses and primary current will rise

dramatically.

The most you can draw from a 60ma neon is about 90-100ma at the rated

voltage. The 30ma neons dont do quite as well mainly due to the fact

that many 30ma neons have aluminum wire in the primary. Expect 40-45ma

from a 30ma neon. If you need more current parallel two units instead

of trying to beat it out of a single unit.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 30 Nov 1998 15:04:03 +1200

Subject:       Re: Unpotting

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I feel I really ought to say something here:

> Date sent:      Mon, 30 Nov 1998 14:21:26 +1300

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Mike Hammer <mhammer@misslink.net>

> Subject:        Re: Unpotting

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> At 11:50 AM 11/28/98 +0000, you wrote:

> >>

> >> 60 MA transformers have significantly larger wire, and can withstand

> >> more overcurrent abuse that the hair-like 30 MA secondaries. You should

> >> easily be able to get 90-100 MA from one of these if immersed in oil.

> >>

> >> -- Bert --

> >

> >Would it be possible to rewind the secondaries of a 15kv Neon with

> >thicker wire to give say....7,500V at 300ma?

> >

> >Just curious....

> >-tom

>

> Tom,

> No this really wouldnt be possible. Take for instance a 15kv/60ma

> neon. This has a core sized for 900va. The primary winding is sized

> for the expected current draw at that power level. You can draw SOME

> additional power without problems but if you try to draw much more

> the primary will overheat and the core will saturate.

Remember that load current and magnetizing current are two totally

separate entities (my transformer post of a couple of months ago).

Magnetizing current remains the same if applied voltage and primary

inductance do also. Extra load current will cause primary heating

due to load current reflection into the primary but not core

saturation.

> Your example of 7500/300ma is 2250 va. Well over twice the

> core rating. Just wont work even with a heavier secondary winding.

> The core will saturate and losses and primary current will rise

> dramatically.

Don't agree about the core bit. The heating is a copper loss, not a

core loss.

Remember also that the current rating on a NST secondary is an

RMS rating. This implies that peak current delivery is SQRT(2)*Irms.

Malcolm

X-Mailer: IPAD 2.0

Date: Sun, 29 Nov 1998 23:16:16 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:04 PM 11/30/98 +1200, you wrote:

>I feel I really ought to say something here:

>

>> Date sent:      Mon, 30 Nov 1998 14:21:26 +1300

>> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> From:           Mike Hammer <mhammer@misslink.net>

>> Subject:        Re: Unpotting

>> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>> At 11:50 AM 11/28/98 +0000, you wrote:

>> >>

>> >> 60 MA transformers have significantly larger wire, and can withstand

>> >> more overcurrent abuse that the hair-like 30 MA secondaries. You should

>> >> easily be able to get 90-100 MA from one of these if immersed in oil.

>> >>

>> >> -- Bert --

>> >

>> >Would it be possible to rewind the secondaries of a 15kv Neon with

>> >thicker wire to give say....7,500V at 300ma?

>> >

>> >Just curious....

>> >-tom

>>

>> Tom,

>> No this really wouldnt be possible. Take for instance a 15kv/60ma

>> neon. This has a core sized for 900va. The primary winding is sized

>> for the expected current draw at that power level. You can draw SOME

>> additional power without problems but if you try to draw much more

>> the primary will overheat and the core will saturate.

>

>Remember that load current and magnetizing current are two totally

>separate entities (my transformer post of a couple of months ago).

>Magnetizing current remains the same if applied voltage and primary

>inductance do also. Extra load current will cause primary heating

>due to load current reflection into the primary but not core

>saturation.

>

>> Your example of 7500/300ma is 2250 va. Well over twice the

>core rating. Just wont work even with a heavier secondary winding.

>> The core will saturate and losses and primary current will rise

>> dramatically.

>

>Don't agree about the core bit. The heating is a copper loss, not a

>core loss.

Yes agreed. In the interest of simplicity I lumped them together.

Heating is copper (or aluminum) loss and core losses stem

from eddy currents and hysteresis.

My point was that losses (whatever the source) will rise quickly

as a transformer is overloaded.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 29 Nov 1998 23:35:40 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:List archive  was New site!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:50 PM 11/29/97 -0700, you wrote:

>

>   Hello, I apologize in advance if this post is redunant,

>   I was just wondering if there is an archive (up to and

>   including the present date) of the mailing list; I need

>   to delete some email to free up space.

>   Thanks for your reply.

>

Jon,

The list archives onced edited and polished by yours truly

are placed on Dave Bronakowski's archive site as well as

on the funet.fi ftp server.

Dave's address is http://www.velocity.net/~djb/files.shtml

Funet.fi will be found at:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

These sites are up to date as of Aug 31, 1998. The archive

for Sept/Oct is not quite ready but Im working on it.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 29 Nov 1998 23:44:42 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 1350 Watts

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:10 PM 11/29/98 -0600, you wrote:

>McFalc0n@aol.com wrote:

>>

>>  will a 10 amp variac withstand running 3 or 4 NST's (15KV 30MA each)?

>Probably so, if they all have power factor correction on them.

>

>Chris

>

A typical 15kv/30ma neon will draw about 5 amps at 120V.

If you use the overwind on the variac to feed them 140V

it will be higher. Well over 6 amps.

So 3 or 4 of those will seriously overload your variac.

Adding PFC caps will lower this somewhat but will still

be over your variac rating.

Your variac will probably handle 15 amps no sweat.

You can probably pull 18-20 amp for short periods

with plenty of cool down time in between.

Keep cool.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 29 Nov 1998 22:01:35 -0800

From: Cliff Blackburn <repairco@axionet.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tesla coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Im thinking of making a tesla coil I was woundering what The best

voltage I could get out of a 15000 volt 60 ma neon transformer. also I

was woundering the diameter and sizes of the priamary and secondary

coils. I was thinking of making the primary out of 1/2" copper pipe and

the secondary out of 4" Abs pipe or 6" pvp(watever would be best). I

also woundered what the best kind of high voltage capacitors are. I was

thinking of making salt water because It is the easiest to make and I

can find everything to make them with. I also thought of making one out

of polyurithane rolled cap but I wouldent know were to get the supplies

unless someone told me. I was also wounering if I should use a rotating

gap or a static gap. Also should I use a satey gap or not?

I know these are a lot of questions but I don't know much about coils

and how to build them.

O yea. what gauge wire? I was thinking 26 (for the secondary)

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Am I correct?

Date: Sun, 29 Nov 1998 22:12:58 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I built a small TC to get my feet wet, (1.75" secondary, 15" long wound

on a cardboard tube). Due to minimum dielectric in my capacitor I am running

at a maximum of 30vac input to a 15kv/60ma nst. I am trying to calculate the

power input to the coil, and this is what I did.

I set the xfmr input at 30vac. I then measured 12.1vac across a 900 ohm resistor

which was across the xfmr output. This calculates out to a current of 13.4ma.

Then I arrived at a output voltage of 3750vac by multiplying the input, (30vac),

by 125, (the turns ratio of the xfmr). Is this procedure correct? Would the coil

input then be 3750vac x 13.4ma = 50.25 watts?

This weekend I built a 9 element, (8 gap), RQ sparkgap, .025" per gap.

The length of the spark that I could draw from the toroid increased by about 1",

getting around 9" sparks now. Boy, was I excited! Now getting arc-over to my

primary coil, (solenoid wound, the coils are parallel to the secondary).

Am in the process of building a proper, (30 degree), primary now.

This Tesla Coil stuff is so cool, I haven't been this excited

about a project for I don't know how long! The sparks that I am

getting now are awesome, I can't imagine what it will be like when

I finish my planned "real"  coil and run it at full power!

Jon Rosenstiel

jonr@pacbell.net

Date: Sun, 29 Nov 1998 22:31:55 -0800

From: Adam Pasternak <pastea2@rpi.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone have experience in winding chokes for kickback protection to

an NST?  I'm not sure what to use as forms and what gauge wire to use.

So far, I'm using a 12kV 60mA NST. Also what's the difference between

air-core and ferrite-core chokes?

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 30 Nov 1998 09:27:59 +0200

Subject: Tentative TC design: first considerations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

These are just the first steps before writing a formal specification of my

TCs to be built.

In my understanding the AC xfrms used to feed most of built TCs are just an

extra sort a complication for its modeling. My TC uses actually "pulsed DC"

from its tank supply: that is, the tank provides a voltage changing from 0

to 9 kV, 50 times a second (just to be sure everybody gets it). Because

everything works fine this proves that TCs can be fed by DC: the primary

capacitor doesn't need to be charged with varying polarity cyclically.

Now, we can imagine a tank DC supply with a bridge rectifier and a huge

flyweel capacitor (e.g. 10 uF) using e.g. a pig. The pig primary would be

current limited according to the flyweel capacitor value. This DC supply

would be connected to rest of the primary circuit (capacitor, spark gap,

primary coil) as usually, but through a second current limiter, this time

set to charge the primary capacitor in a time smaller then the time between

bangs of the rotary spark gap. This will also protect the flyweel capacitor

from discharging too much when the spark gap fires.

Having a variac on the pig primary would allow to directly set the tank

output voltage and we would be sure that the primary capacitor is always

able to get charged to that voltage before every bang.

This means that we'll know precisely at what voltage the primary cap is

charged i.e. we'll know exactly how much POWER we'll pump into the primary

coil at each bang (forget about the losses for now).

We could than easily investigate the effect of bang repetition rate without

bothering about the AC tank voltage, the bang phasing, the tank supply

impedance, etc.

What about that?

Another thing that I don't get is why TC power requirements are usually

defined as tank supply voltage and its current limit value: wouldn't it be

more exact to speak about power (Joules) required by the primary winding

(easy to calculate from primary cap value and its charging voltage) or the

energy required (multiply by the BPS) ?

Eventually the most important thing that matters (again, forget about

losses) is how much energy is stored in the primary capacitor, not its

charging voltage, to define how much energy we are pumping into the primary

coil, isn't it?

Date: Mon, 30 Nov 1998 01:01:21 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Anyone here from Blind river Ontario?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Anyone here from Blind river Ontario?

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Mon, 30 Nov 1998 01:27:38 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cliff Blackburn wrote:

>

> Im thinking of making a tesla coil I was woundering what The best

> voltage I could get out of a 15000 volt 60 ma neon transformer.

Voltage isn't as important as wattage, and you have plenty to start with.

> I was wondering the diameter and sizes of the priamary and secondary

> coils. I was thinking of making the primary out of 1/2" copper pipe and

> the secondary out of 4" Abs pipe or 6" pvp(watever would be best).

Go for the pvc as apposed to abs, my abs 3" does not preform as it

should(I think because of internal resistance) and 4" to 6" is a good

starting point, 1/2 pipe for your primary is overkill, 1/4" to 3/8" is good.

> I also woundered what the best kind of high voltage capacitors are. I was

> thinking of making salt water because It is the easiest to make and I

> can find everything to make them with. I also thought of making one out

> of polyurithane rolled cap but I wouldent know were to get the supplies

> unless someone told me.

The rolled one are the best(so I've heard) If you can get crazy

karpet(cheap childs snowsled) than you can make an inexspensive one.

I was also wounering if I should use a rotating

> gap or a static gap. Also should I use a satey gap or not?

I recommend a multigap gap, one that has several gaps in series and fan

cooled(less heat, better performance.

>

> I know these are a lot of questions but I don't know much about coils

> and how to build them.

>

Do an internet search and get as much info as possible before you begin,

be carefull because 15,000V can easily kill you.

> O yea what gauge wire I was thinking 26 (for the secondary)

I believe chips page has a chart on what wire to use per inch of

secondary diameter.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Mon, 30 Nov 1998 07:03:44 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tentative TC design: first considerations

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> These are just the first steps before writing a formal specification of my

> TCs to be built.

>

> In my understanding the AC xfrms used to feed most of built TCs are just an

> extra sort a complication for its modeling. My TC uses actually "pulsed DC"

> from its tank supply: that is, the tank provides a voltage changing from 0

> to 9 kV, 50 times a second (just to be sure everybody gets it). Because

> everything works fine this proves that TCs can be fed by DC: the primary

> capacitor doesn't need to be charged with varying polarity cyclically.

>

> Now, we can imagine a tank DC supply with a bridge rectifier and a huge

> flyweel capacitor (e.g. 10 uF) using e.g. a pig. The pig primary would be

> current limited according to the flyweel capacitor value. This DC supply

> would be connected to rest of the primary circuit (capacitor, spark gap,

> primary coil) as usually, but through a second current limiter, this time

> set to charge the primary capacitor in a time smaller then the time between

> bangs of the rotary spark gap. This will also protect the flyweel capacitor

> from discharging too much when the spark gap fires.

With careful design, this second current limiter can be a high voltage

choke. The world's biggest Tesla Coil uses rectified 3-phase power

through a "charging" choke to fix the bang size. By then adjusting the

break rate of the main rotary gap he can precisely control the power

going to the coil.

>

> Having a variac on the pig primary would allow to directly set the tank

> output voltage and we would be sure that the primary capacitor is always

> able to get charged to that voltage before every bang.

Basically correct. However, there are some other interesting wrinkles

when you look at an efficient reactive charging circuit.

>

> This means that we'll know precisely at what voltage the primary cap is

> charged i.e. we'll know exactly how much POWER we'll pump into the primary

> coil at each bang (forget about the losses for now).

Yes, if you use a rotary gap, and if the tank cap is completely

recharged between presentations.

>

> We could than easily investigate the effect of bang repetition rate without

> bothering about the AC tank voltage, the bang phasing, the tank supply

> impedance, etc.

>

Simplifies the problem a bit, doesn't it!

> What about that?

>

> Another thing that I don't get is why TC power requirements are usually

> defined as tank supply voltage and its current limit value: wouldn't it be

> more exact to speak about power (Joules) required by the primary winding

> (easy to calculate from primary cap value and its charging voltage) or the

> energy required (multiply by the BPS) ?

>

> Eventually the most important thing that matters (again, forget about

> losses) is how much energy is stored in the primary capacitor, not its

> charging voltage, to define how much energy we are pumping into the primary

> coil, isn't it?

Correct - actually, charging voltage is often designed to be much higher

to facilitate rapid recharging of the tank cap. It's actually the

breakdown voltage of the main gap and the tank capacitance that count

re: "bang" size.

-- Bert --

From: Hollmike@aol.com

Date: Mon, 30 Nov 1998 09:27:46 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cliff,

First, I would go with PVC for the secondary form.  It is cheap and easy to

get.  ABS might do well, but I haven't heard of people using it much.  You

could use from 4 -6 inch diameter with the neon and wind up to about 1000

turns.

Second, I would suggest using 24g wire for the form, unless of course, you

already have the wire :)

Third, LDPE or polypropylene are the two best choices for dielectric for the

cap.  Much better performance than the salt water cap.  Of course, you can use

the salt water cap to get started, but they tend to be lossy.

Fourth,  unless you already purchased the copper pipe, you don't need that

large diameter.  1/4 inch will do nicely.

The gap is your choice.  Either a rotary or a static gap, properly

designed, will work with this size coil.

Hope this helps.

Mike

Date: Mon, 30 Nov 1998 10:08:48 -0700

From: Dan Olsen <dan@olsen.org>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Unpotting

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just unpotted a 12kV/60 mA that a local sign shop gave me for FREE!

I put it back together and to my amazement, it still works,  now I have two

questions for the list....

First: I am wondering how many shunts I can take out (there are 8 in each of the

four groups), and how you measure the the resulting current. Do you measure the

Primary current and calculate from that or do you measure it directly?  I am a

little hesitant to hook up my $130 multi-meter to the business end of the

transformer.  Although I think that when measuring current  it would have a low

resistance and a simple amp meter should work fine(it would look just like a wire

to the circut), but what about these electronic multi-meters?

Second:  where do you get transformer oil?  The Mineral oil at the hardware store

is too expensive.($8 / quart)  I don't suppose vegetable oil would work?

p.s.  I cut-up a 2 liter Soda bottle (HDPE) to insulate the secondaries from the

core and the shunts, and it runs fine out in the air, I still should use oil to

cool it right, especially if I remove some shunts.

X-Mailer: IPAD 2.0

Date: Mon, 30 Nov 1998 11:38:03 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:08 AM 11/30/98 -0700, you wrote:

>I just unpotted a 12kV/60 mA that a local sign shop gave me for FREE!

>I put it back together and to my amazement, it still works,  now I have two

>questions for the list....

Great. Welcome to the elite group of transformer rebuilders.

>

>First: I am wondering how many shunts I can take out (there are 8 in each of the

>four groups), and how you measure the the resulting current.

Only 8 plates in each shunt? That doesnt sound like enough to me.

Most of the ones I have done have had from 18-24 plates in each shunt.

What is the manufacturer of your neon? Each manufacturer does things a

little differently.

>Do you measure the Primary current and calculate from that or do you measure it directly?

>I am a little hesitant to hook up my $130 multi-meter to the business end of the

>transformer.  Although I think that when measuring current  it would have a low

>resistance and a simple amp meter should work fine(it would look just like a wire

>to the circut), but what about these electronic multi-meters?

This is correct. Your meter IN AC CURRENT MEASURING MODE represents nearly

a short circuit. The neon goes into limiting and the actual secondary

voltage is quite low. You can then read the actual short circuit current

of the neon directly. Make sure you connect the meter firmly since a lead

coming off during the test will allow the voltage to zoom up and cause an

arc and probably damage your meter. Same thing if you accidently put your

meter on in resistance or voltage mode. Be careful.

>

>Second:  where do you get transformer oil?  The Mineral oil at the hardware store

>is too expensive.($8 / quart)  I don't suppose vegetable oil would work?

Vegetable oils contain far too much moisture and will go rancid.

Transformer oil can be purchased from most oil distributers.

Shell Diala AX is one of the most popular.

Try your local power company. Sometimes you may get a couple gallons cheap

or even free this way. If they are unwilling to sell you any then perhaps

they can tell you where they purchase their oil. Or just get out the yellow

pages and start making some calls.

For cheaper mineral oil try your local veterinarian. It comes in 2 and 5 gallon

pails for use as an animal laxative.

>

>p.s.  I cut-up a 2 liter Soda bottle (HDPE) to insulate the secondaries from the

>core and the shunts, and it runs fine out in the air, I still should use oil to

>cool it right, especially if I remove some shunts.

Immersing in oil is more for the insulation than it is for cooling.

I've run many rebuilds dry. Just beef up that insulation and they

will run dry just fine.

Of course if you put them in oil and you did a good job of rebuilding

they become very rugged and will happily take abuse that regular neons cant.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Chokes

Date: Mon, 30 Nov 1998 15:21:20 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Use the ferrite donuts from the back of a TV Picture tube...

they work great with about 150-200 winds of #24 wire...

-tom

----------

From: 	Adam Pasternak

Sent: 	Monday, November 30, 1998 1:31 AM

To: 	Tesla-2 Mailing List

Subject: 	Chokes

Does anyone have experience in winding chokes for kickback protection to

an NST?  I'm not sure what to use as forms and what gauge wire to use.

So far, I'm using a 12kV 60mA NST. Also what's the difference between

air-core and ferrite-core chokes?

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 1 Dec 1998 09:32:17 +1200

Subject:       Re: Tentative TC design: first considerations

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

Here are some answers (I'll expect a payback later when you

can answer the ones I posed on the builder's list ;) :

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Mon, 30 Nov 1998 20:27:59 +1300

> Subject:        Tentative TC design: first considerations

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> These are just the first steps before writing a formal specification of my

> TCs to be built.

>

> In my understanding the AC xfrms used to feed most of built TCs are just an

> extra sort a complication for its modeling. My TC uses actually "pulsed DC"

> from its tank supply: that is, the tank provides a voltage changing from 0

> to 9 kV, 50 times a second (just to be sure everybody gets it). Because

> everything works fine this proves that TCs can be fed by DC: the primary

> capacitor doesn't need to be charged with varying polarity cyclically.

True. But 50 times per second? Are we talking about mains input or a

deliberately tailored breakrate?

> Now, we can imagine a tank DC supply with a bridge rectifier and a huge

> flyweel capacitor (e.g. 10 uF) using e.g. a pig. The pig primary would be

> current limited according to the flyweel capacitor value. This DC supply

> would be connected to rest of the primary circuit (capacitor, spark gap,

> primary coil) as usually, but through a second current limiter, this time

> set to charge the primary capacitor in a time smaller then the time between

> bangs of the rotary spark gap. This will also protect the flyweel capacitor

> from discharging too much when the spark gap fires.

Be aware that the primary cap and limiting choke form a resonant

circuit. If your break rate is too slow, the cap will charge to a

value greater than the DC value of the storage cap and then start

coming down again unless you include a diode in series with the

arrangement. Charge time will be PI*SQRT(LC)/2  where L is the choke

value and C is the primary cap and the assumption is made that C is

very much smaller than the 10uF cap.

> Having a variac on the pig primary would allow to directly set the tank

> output voltage and we would be sure that the primary capacitor is always

> able to get charged to that voltage before every bang.

>

> This means that we'll know precisely at what voltage the primary cap is

> charged i.e. we'll know exactly how much POWER we'll pump into the primary

> coil at each bang (forget about the losses for now).

>

> We could than easily investigate the effect of bang repetition rate without

> bothering about the AC tank voltage, the bang phasing, the tank supply

> impedance, etc.

>

> What about that?

>

> Another thing that I don't get is why TC power requirements are usually

> defined as tank supply voltage and its current limit value: wouldn't it be

> more exact to speak about power (Joules) required by the primary winding

> (easy to calculate from primary cap value and its charging voltage) or the

> energy required (multiply by the BPS) ?

Power is measured in Watts, not Joules. You are speaking of capacitor

energy here. The reason why those specs are often used is because

lots of coils are powered from NSTs and NSTs have an effective "in-

built" choke which does the job of your proposed external limiting

choke. Again, one ends up with a resonant charging circuit. You

should study the equivalent circuits of these supplies.

> Eventually the most important thing that matters (again, forget about

> losses) is how much energy is stored in the primary capacitor, not its

> charging voltage, to define how much energy we are pumping into the primary

> coil, isn't it?

Fundamental! Of course where sparks are concerned, break rate is the

other fundamental. However, cap energy together with secondary

parameters determines peak possible output voltage whereas break rate

has no influence over this.

Malcolm

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 1 Dec 1998 07:06:28 +0200

Subject: Re: Tentative TC design: first considerations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MALCOLM@directorate.wnp.ac.nz on 30.11.98 23:32:17

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: Tentative TC design: first considerations

>True. But 50 times per second? Are we talking about mains input or a

>deliberately tailored breakrate?

Mains rate (Europe = 50 Hz). But this was my present design.

>Be aware that the primary cap and limiting choke form a resonant

>circuit. If your break rate is too slow, the cap will charge to a

>value greater than the DC value of the storage cap and then start

>coming down again unless you include a diode in series with the

>arrangement.

Imagine the usual DC full-wave power supply scheme: transformer, diode

bridge, big capacitor. The big capacitor (and thus the primary capacitor)

have no chance to change to a voltage higher than VAC * SQRT(2).

>Power is measured in Watts, not Joules. You are speaking of capacitor

>energy here. The reason why those specs are often used is because

>lots of coils are powered from NSTs and NSTs have an effective "in-

>built" choke which does the job of your proposed external limiting

>choke. Again, one ends up with a resonant charging circuit. You

>should study the equivalent circuits of these supplies.

Yes, sorry about the confusion. E = 0.5*C*V*V is the capacitor energy

(Joules). Discharging it you get a power (Watts) for a certain discharge

time. But I don't want to spend time with equivalent circuits of a NST.

Instead I want to isolate 2 effects:

1 - Capacitor energy single break generating voltage (and electromagnetic

field) at the secondary terminal, with no sparks

2 - Charge at the secondary terminal breaking up into streamers, and BPS

effect on it.

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Corona motor

Date: Tue, 01 Dec 1998 18:35:16 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Another easy fun thing to do with your coil....

http://www.netcomuk.co.uk/~wwl/teslamotor.html

Might possibly be useful for plotting E-field patterns ???

Date: Tue, 01 Dec 1998 14:03:45 -0500

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Corona motor

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

Cool pic's Mike. Some of your comments, thoughts at the bottom of your

page are really interesting.

Regards,

Bert W.

Mike Harrison wrote:

> Another easy fun thing to do with your coil....

> http://www.netcomuk.co.uk/~wwl/teslamotor.html

> Might possibly be useful for plotting E-field patterns ???

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Dec 1998 08:56:14 +1200

Subject:       Re: Tentative TC design: first considerations

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Tue, 1 Dec 1998 18:06:28 +1300

> Subject:        Re: Tentative TC design: first considerations

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

> MALCOLM@directorate.wnp.ac.nz on 30.11.98 23:32:17

>

>

> To:   tesla-2@emachine.com

> cc:    (bcc: Marco Denicolai/MARTIS)

> Subject:  Re: Tentative TC design: first considerations

>

>

>

>

>

> >True. But 50 times per second? Are we talking about mains input or a

> >deliberately tailored breakrate?

> Mains rate (Europe = 50 Hz). But this was my present design.

OK - the cap charges at twice the mains frequency unless the charging

circuit is resonant and the gap is set too wide.

> >Be aware that the primary cap and limiting choke form a resonant

> >circuit. If your break rate is too slow, the cap will charge to a

> >value greater than the DC value of the storage cap and then start

> >coming down again unless you include a diode in series with the

> >arrangement.

>

> Imagine the usual DC full-wave power supply scheme: transformer, diode

> bridge, big capacitor. The big capacitor (and thus the primary capacitor)

> have no chance to change to a voltage higher than VAC * SQRT(2).

Uh uh - if you include a charging/limiting choke it most certainly

can unless the choke value causes charging circuit resonance to go

way below break frequency.  You should get hold of a copy of Sargeat

and Dollinger's "High Power Electronics". It is a must-read if you

are going to do this research.

Malcolm

From: EdwMullin@aol.com

Date: Tue, 1 Dec 1998 17:55:20 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary circuit design

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have tried two ways of configuring my primary circuit:

1>  From transfromer, capacitor in parallel, spark gap in series, P-Coil in series

This configuration produces hot fat sparks in the gap and good output at

secondary terminal

2> From Xfrmer, spark gap in parallel, cap in series with Primary coil.

With this configuration the spark gap has faint blue sparks (much quieter)

and there is no output visible from the secondary terminal.

I had thought that the second way would work as well as the first, and I saw

this schematic on a web page.  I was wondering if anyone had any ideas about

the difference in output?

Coil specs:

Secondary: 3.5 in Diam, 19 in long, ~850 turns of 24 awg.Terminal: pointed rod

Primary: 15 turns 0.25 in copper tube, solenoid wound, 7.5 in diameter, tapped at 		 turn #10

Cap: 8.7nf flat plate type

Gap: Series type, operating at ~0.125 inch opening.

Xformer: 10kv/.021ma obit

X-Mailer: IPAD 2.0

Date: Tue, 01 Dec 1998 17:19:39 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary circuit design

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:55 PM 12/1/98 EST, you wrote:

>I have tried two ways of configuring my primary circuit:

>

>1> From transfromer, capacitor in parallel, spark gap in series, P-Coil in series

>      This configuration produces hot fat sparks in the gap and good output at

>	secondary terminal

>

>2> From Xfrmer, spark gap in parallel, cap in series with Primary coil.

>   With this configuration the spark gap has faint blue sparks (much quieter)

>   and  there is no output visible from the secondary terminal.

>

>I had thought that the second way would work as well as the first, and I saw

>this schematic on a web page.  I was wondering if anyone had any ideas about

>the difference in output?

>

<snip>

Ed,

There is no difference in performance in the two configurations.

They work equally well. Configuration #1 is sure death for your neon.

Since you are not getting #2 to work you must have a wiring error.

Perhaps you have the gap also in series with the primary instead

of directly across the HV outputs of the neon. That way the cap will

only charge if the gap breaks down and will act very much like you

describe. Double check your wiring because this configuration does

work and is equivalent in performance to the other.

TTYL!

Mike Hammer

mhammer@misslink.net

From: EdwMullin@aol.com

Date: Tue, 1 Dec 1998 20:20:19 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary circuit design

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I wrote:

<< >I have tried two ways of configuring my primary circuit:

>

<snip>

Mike Hammer replied:

>There is no difference in performance in the two configurations.

>They work equally well. Configuration #1 is sure death for your neon.

<snip>

Well my small Obit gave up the ghost, I was running in the first

configuration, and things got quiet all of a sudden like.  I get a low

resistance path to ground on one side and only slightly better on the other.

I am going to unpot it,  but I was wondering has anyone rebuilt an OBIT

before? are they worth the trouble?  I would assume that they are constructed

much like nst's, and would be good just for the experience...

Well live and learn!

Date: Tue, 01 Dec 1998 19:42:06 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary circuit design

Sender: tesl-err@emachine.com

EdwMullin@aol.com wrote:

>

> I have tried two ways of configuring my primary circuit:

>

> 1>  From transfromer, capacitor in parallel, spark gap in series, P-Coil in  series

>       This configuration produces hot fat sparks in the gap and good output at

>       secondary terminal

>

> 2> From Xfrmer, spark gap in parallel, cap in series with Primary coil.

>    With this configuration the spark gap has faint blue sparks (much quieter) and

>    there is no output visible from the secondary terminal.

>

> I had thought that the second way would work as well as the first, and I saw

> this schematic on a web page.  I was wondering if anyone had any ideas about

> the difference in output?

>

> Coil specs:

> Secondary: 3.5 in Diam, 19 in long, ~850 turns of 24 awg.Terminal: pointed rod

>     Primary: 15 turns 0.25 in copper tube, solenoid wound, 7.5 in diameter, tapped at                       >              turn #10

>          Cap: 8.7nf flat plate type

>          Gap: Series type, operating at ~0.125 inch opening.

>     Xformer: 10kv/.021ma obit

I suspect the difference is that the first configuration is in tune,

while the second one is not. A NST actually has a fair amount of

inter-turn capacitance "looking into" the HV output terminals. When you

have the transformer in parallel with the tank cap, you actually have a

slightly larger effective tank cap since the capacitance of the

transformer is in parallel with your actual tank cap. The other way

around, you'll have somewhat less tank capacitance, and you'll need to

increase the tank inductance by changing the primary tap outwards to

bring the system into proper tune. Otherwise from an RF and pulse

standpoint, there's really no difference between the two configurations.

However, from a practical standpoint, having the cap in parallel with

the transformere actually subjects it to significant amounts of high

voltage RF - this will lead it to an early demise... Most coilers have

best success and longer NST lifetime with the main gap sitting across

the transformer.

-- Bert --

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 2 Dec 1998 08:14:35 +0200

Subject: Re: Tentative TC design: first considerations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MALCOLM@directorate.wnp.ac.nz on 01.12.98 22:56:14

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: Tentative TC design: first considerations

>Uh uh - if you include a charging/limiting choke it most certainly

>can unless the choke value causes charging circuit resonance to go

>way below break frequency.  You should get hold of a copy of Sargeat

>and Dollinger's "High Power Electronics". It is a must-read if you

>are going to do this research.

>Malcolm

I got your idea: I must think more about this issue.

What I am wrestling with now is a rough design of secondary and primary coils:

- secondary 15" wide by 60" high, 931 turns, 73 mH, 112/62 kHz

- toroid 10" section 60" outside diameter 62 pF

- primary spiral 18" inner diameter, 12 turns, 128 uH

- --> Primary capacitor 0.056 uF

I would like to maximize the capacitor size, to be able to pump more energy

into the primary. The problem is that, having a big secondary, I must have

a big primary inner diameter too. With just 12 turn of primary my capacitor

(required to be tuned) drops down to 0.056 uF. I would have liked at least

a 0.1 uF...One possibility is to use only 8 turns (--> Cap = 0.123 uF).

With 0.056 uF and 20 kV the cap energy would go up to 11.3 Joules, while

with 0.123 uF up to 24.6 Joules.

Any suggestions anybody?

Date: Wed, 02 Dec 1998 10:19:24 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Corona motor

Sender: tesl-err@emachine.com

Mike Harrison wrote:

>

> Another easy fun thing to do with your coil....

> http://www.netcomuk.co.uk/~wwl/teslamotor.html

> Might possibly be useful for plotting E-field patterns ???

Mike,

NEAT pages!

What happens if a mickyd CD is cut to resemble a 12 tooth saw circular

saw blade, and fitted with an axial bearing?

The whole world of possible TeslaMobiles opens ....  Even a rotary

switching device for cycling E.Gnd thru such various elements...?

PS: I put my laptop on my toroid -- I didn't drop it, but it still

doesn't work anymore-- didn't even get any neat displays!

_____________________________________________

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Wed, 02 Dec 1998 14:33:47 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary circuit design

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:20 PM 12/1/98 EST, you wrote:

>I wrote:

><< >I have tried two ways of configuring my primary circuit:

> >

>

> <snip>

>

> Mike Hammer replied:

>

> >There is no difference in performance in the two configurations.

> >They work equally well. Configuration #1 is sure death for your neon.

>

><snip>

>

>Well my small Obit gave up the ghost, I was running in the first

>configuration, and things got quiet all of a sudden like.  I get a low

>resistance path to ground on one side and only slightly better on the other.

>I am going to unpot it,  but I was wondering has anyone rebuilt an OBIT

>before? are they worth the trouble?  I would assume that they are constructed

>much like nst's, and would be good just for the experience...

>

>Well live and learn!

Ed,

Sorry to hear about that. Now you know first hand the

danger involved with running the cap right across the

transformer.

Since your transformer is probably carbon track shorted

you might try baking it to melt the potting compound

and repair the carbon short. It MIGHT restore your transformer.

Personally I wouldnt unpot an OBIT. But if its your only

transformer then unpotting may be necessary. They are

built much like neons. Once unpotted you can add some

real insulation to it and end up with a better transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Dec 1998 09:03:54 +1200

Subject:       Re: Tentative TC design: first considerations

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Wed, 2 Dec 1998 19:14:35 +1300

> Subject:        Re: Tentative TC design: first considerations

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>

> MALCOLM@directorate.wnp.ac.nz on 01.12.98 22:56:14

>

> To:   tesla-2@emachine.com

> cc:    (bcc: Marco Denicolai/MARTIS)

> Subject:  Re: Tentative TC design: first considerations

Everyone who uses "limiting inductance" that is underdamped is using

the energy storage capability of these inductors whether they like it

or not. The gap firing voltage then becomes the sole arbiter of how

effectively that inductance is being used. This includes NSTs.

> What I am wrestling with now is a rough design of secondary and primary

> coils:

>

> - secondary 15" wide by 60" high, 931 turns, 73 mH, 112/62 kHz

> - toroid 10" section 60" outside diameter 62 pF

> - primary spiral 18" inner diameter, 12 turns, 128 uH

> - --> Primary capacitor 0.056 uF

>

> I would like to maximize the capacitor size, to be able to pump more energy

> into the primary. The problem is that, having a big secondary, I must have

> a big primary inner diameter too. With just 12 turn of primary my capacitor

> (required to be tuned) drops down to 0.056 uF. I would have liked at least

> a 0.1 uF...One possibility is to use only 8 turns (--> Cap = 0.123 uF).

>

> With 0.056 uF and 20 kV the cap energy would go up to 11.3 Joules, while

> with 0.123 uF up to 24.6 Joules.

I'd be interested to see whether 24 Joules causes a winding

flashover. I haven't run through the figures to estimate your Vout so

can't give a definitive answer right now.

Re the primary - you could try building one the same diameter as

the secondary and running it below the secondary. You could also try

running one *inside* the secondary. I know two people running high

power who have done this successfully. However, their coils were a

lot bigger. One was Greg Leyh's Electrum and the other was a large

coil built by Mark Barton and friends.

Malcolm

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai"<Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 3 Dec 1998 07:17:23 +0200

Subject: Re: Tentative TC design: first considerations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MALCOLM@directorate.wnp.ac.nz on 02.12.98 23:03:54

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: Tentative TC design: first considerations

>> Please respond to tesla-2@emachine.com

>??? I am responding to the list. What gives with the above message?

It might be my stupid mail program (Lotus Notes): I am forced by company

policy to use it and I DON'T recommend it!

>I'd be interested to see whether 24 Joules causes a winding

>flashover. I haven't run through the figures to estimate your Vout so

>can't give a definitive answer right now.

Me either.

>     Re the primary - you could try building one the same diameter as

>the secondary and running it below the secondary. You could also try

>running one *inside* the secondary. I know two people running high

>power who have done this successfully. However, their coils were a

>lot bigger. One was Greg Leyh's Electrum and the other was a large

>coil built by Mark Barton and friends.

I know about the Electrum, but where to find info about Mark Barton's TC?

Chuck Curran also made a coil sized about like the one I plan to do, but

where to find more data about it?

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Corona motor

Date: Thu, 03 Dec 1998 10:07:53 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Wed, 02 Dec 1998 10:19:24 -0800, you wrote:

>Mike Harrison wrote:

>>

>> Another easy fun thing to do with your coil....

>> http://www.netcomuk.co.uk/~wwl/teslamotor.html

>> Might possibly be useful for plotting E-field patterns ???

>

>Mike,

>NEAT pages!

>

>What happens if a mickyd CD is cut to resemble a 12 tooth saw circular

>saw blade, and fitted with an axial bearing?

Thought of that, bit haven't tried it yet. I suspect the difference in

sharpness between the the 'tooth' and the edge of the CD isn't enough,

bit it's worth a try.

>The whole world of possible TeslaMobiles opens ....

Yup! I've got plenty of ideas when I get time...!

Date: Thu, 03 Dec 1998 11:09:36 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Corona motor

Sender: tesl-err@emachine.com

Mike Harrison wrote:

>

> On Wed, 02 Dec 1998 10:19:24 -0800, you wrote:

>

> >Mike Harrison wrote:

> >>

> >> Another easy fun thing to do with your coil....

> >> http://www.netcomuk.co.uk/~wwl/teslamotor.html

> >> Might possibly be useful for plotting E-field patterns ???

> >

> >Mike,

> >NEAT pages!

> >

> >What happens if a mickyd CD is cut to resemble a 12 tooth saw circular

> >saw blade, and fitted with an axial bearing?

> Thought of that, bit haven't tried it yet.

I'm thinking of that styrofoam ullage: the sort of hemispheric shaped stuff-

stuck on the pointy parts of the corona motor- should use the heated air

factor to provide force?  How to keep the stuff from melting down?

Corona Motor 'drag races'?  Useful work? Shaded gap (pole) grounding?

Run the thing horizontally: use pin/'jewel' bearing: fewer balance and

friction losses?

> >The whole world of possible TeslaMobiles opens ....

> Yup! I've got plenty of ideas when I get time...!

Ah! Time and inertia! The death of creation!

----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 4 Dec 1998 09:35:09 +1200

Subject:       Re: Tentative TC design: first considerations

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Thu, 3 Dec 1998 18:17:23 +1300

> Subject:        Re: Tentative TC design: first considerations

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >I'd be interested to see whether 24 Joules causes a winding

> >flashover. I haven't run through the figures to estimate your Vout so

> >can't give a definitive answer right now.

> Me either.

lazymode/off

Without checking you 73mH calc, I come up with Ctot for the system at

90pF assuming the 62kHz final frequency which sounds a bit high, even

taking the large torus into account. Anyhow, if so, then by lumped

calculation, 24J is going to equate to Vout peak (assuming no primary

loss) of 730kV. Actually, that sounds like a go for a 60" high coil.

> >     Re the primary - you could try building one the same diameter as

> >the secondary and running it below the secondary. You could also try

> >running one *inside* the secondary. I know two people running high

> >power who have done this successfully. However, their coils were a

> >lot bigger. One was Greg Leyh's Electrum and the other was a large

> >coil built by Mark Barton and friends.

> I know about the Electrum, but where to find info about Mark Barton's TC?

> Chuck Curran also made a coil sized about like the one I plan to do, but

> where to find more data about it?

There really isn't much to know. The coil was copper tubing with an

inductance of about 12uH (way too low according to Mark in the light

of knowledge gleaned since). He was going to change it but I don't

know whether he has. I've been out of touch with him for about a year.

In the light of the Vout figure, it won't be on for your coil as the

secondary probably won't be long enough to prevent internal

flashovers and/or corona. Looks like an external primary. I'd

strongly recommend building a 2-layer or more flat spiral to squeeze

as much inductance as you can out of a length of tubing in as little

volume as possible. Insulation between layers is mandatory.

Malcolm

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 4 Dec 1998 07:27:14 +0200

Subject: Tentative TC design: 2-layer primary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MALCOLM@directorate.wnp.ac.nz on 03.12.98 23:35:09

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: Tentative TC design: first considerations

>Actually, that sounds like a go for a 60" high coil.

Good.

>Looks like an external primary. I'd

>strongly recommend building a 2-layer or more flat spiral to squeeze

>as much inductance as you can out of a length of tubing in as little

>volume as possible. Insulation between layers is mandatory.

Why a 2-layer spiral? Why the volume must be minimized?

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Corona motor

Date: Fri, 04 Dec 1998 11:07:28 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Thu, 03 Dec 1998 11:09:36 -0800, you wrote:

>Mike Harrison wrote:

>>

>> On Wed, 02 Dec 1998 10:19:24 -0800, you wrote:

>>

>> >Mike Harrison wrote:

>> >>

>> >> Another easy fun thing to do with your coil....

>> >> http://www.netcomuk.co.uk/~wwl/teslamotor.html

>> >> Might possibly be useful for plotting E-field patterns ???

>> >

>> >Mike,

>> >NEAT pages!

>I'm thinking of that styrofoam ullage: the sort of hemispheric shaped

>stuff-

>stuck on the pointy parts of the corona motor- should use the heated air

>factor to provide force?

>How to keep the stuff from melting down?

Big problem - one arc & it's carbon!

>Corona Motor 'drag races'?

Hmmm - how about horizontal rails with 'S' rotors on straight shafts

that roll along them...

> Useful work?

Yeah, Right! Have you seen those ornamental garden windmills that

power a little guy chopping wood..... Hmmm!

>Shaded gap (pole) grounding?

Not sure quite what you mean by this

>Run the thing horizontally: use pin/'jewel' bearing: fewer balance and

>friction losses?

Yes of course horizontal is easy,  but vertical mounting looks _SO_

much better when standing a safe distance away....!

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 7 Dec 1998 10:39:02 +1200

Subject:       Re: Tentative TC design: 2-layer primary?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:   tesla-2@emachine.com

> cc:    (bcc: Marco Denicolai/MARTIS)

> Subject:  Re: Tentative TC design: first considerations

>

>

>

>

> >Actually, that sounds like a go for a 60" high coil.

>

> Good.

>

> >Looks like an external primary. I'd

> >strongly recommend building a 2-layer or more flat spiral to squeeze

> >as much inductance as you can out of a length of tubing in as little

> >volume as possible. Insulation between layers is mandatory.

>

> Why a 2-layer spiral? Why the volume must be minimized?

Well you can make it as big as you like, but you are both increasing

chances of a strike to the primary and not making best possible use

of your money for a primary conductor. The two-layer technique uses

mutual induction between primary turns to maximise the inductance you

can squeeze out of a given length of copper.

Malcolm

From: Dsurfr@aol.com

Date: Sun, 6 Dec 1998 17:46:30 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: series vs. parallel caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All:     I'm in the middle of cutting up a roll of .006" LDPE to make a set

of vertical stacked plate caps & have a question on series wired caps

vs.parallel. Since the total voltage rating increases w/ each cap in series

(thus spreading out the load over each cap??) am I making a stronger cap by

building three caps w/ 5 layers of .006 Poly (.03" total)  & connecting them

in series (.018uf each - ~.006uf total) Using Mike's conservative rating of

350 V/mil this would be 10,500 V/cap or 31,500 V total. If I make two caps

using 15 layers of .006 Poly (.09" total) & connect them in parallel (.003uf

each - ~.006uf  total) 350V/mil x .09" = 31,500 V. total. I'd only have to cut

50+ more pieces of poly for the series caps & the housing would only be a bit

bigger so my question is which design will take more abuse? I'm using a 15kV

30mA neon, plates will be 7" x +7"  Al. foil (extended plate design) w/ 9" x

9" poly dielectric sheets. As long as I'm going to this much work I'd like to

make it as "bullet proof" as possible.

Also could I "fold" the layers of poly ( 9" x whatever, back on itself to make

9" x 9" layers of dielectric (cutting down on the cutting time) or will the

whole sheet react as one piece of .006" poly?

Any & all advice is more than welcome. Thanks in advance.  Jim

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: series vs. parallel caps

Date: Sun, 6 Dec 1998 18:13:59 -0500

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim,

It is generally not a good idea to fold dielectric. The folded edge

will be thicker than the other three edges. You really don't want this kind of

source of un-even-ness in your capacitor. When folding multiple sheets

the un-even0ness will become quickly apparent.

The series approach will be better than the parallel approach from the point of

view of corona production. When you have many sections in series you can

usually get the corona reduced significantly, and the corona is the source

of most cap failures. When the corona level is high it can eat right

through the dielectric a sheet at a time. Failure is then rapid.

When wiring flat caps in series within a single container, be especially

careful to make sure that there is adequate dielectric insulation between

sections. One side of the section will have a voltage difference of zero

since it is directly connected to the previous section. The OTHER side

will have a fair voltage differential present. It is easy to neglect this and

then have big problems later.

Good luck on your capacitor building!

Hope this helps.

Fr. Tom McGahee

----------

> From: Dsurfr@aol.com

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: series vs. parallel caps

> Date: Sunday, December 06, 1998 5:46 PM

>

> Hi All:     I'm in the middle of cutting up a roll of .006" LDPE to make a set

> of vertical stacked plate caps & have a question on series wired caps

> vs.parallel. Since the total voltage rating increases w/ each cap in series

> (thus spreading out the load over each cap??) am I making a stronger cap by

> building three caps w/ 5 layers of .006 Poly (.03" total)  & connecting them

> in series (.018uf each - ~.006uf total) Using Mike's conservative rating of

> 350 V/mil this would be 10,500 V/cap or 31,500 V total. If I make two caps

> using 15 layers of .006 Poly (.09" total) & connect them in parallel (.003uf

> each - ~.006uf  total) 350V/mil x .09" = 31,500 V. total. I'd only have to

cut

> 50+ more pieces of poly for the series caps & the housing would only be a bit

> bigger so my question is which design will take more abuse? I'm using a 15kV

> 30mA neon, plates will be 7" x +7"  Al. foil (extended plate design) w/ 9" x

> 9" poly dielectric sheets. As long as I'm going to this much work I'd like to

> make it as "bullet proof" as possible.

> Also could I "fold" the layers of poly ( 9" x whatever, back on itself to make

> 9" x 9" layers of dielectric (cutting down on the cutting time) or will the

> whole sheet react as one piece of .006" poly?

> Any & all advice is more than welcome. Thanks in advance.  Jim

X-Mailer: IPAD 2.0

Date: Sun, 06 Dec 1998 17:25:40 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: series vs. parallel caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:46 PM 12/6/98 EST, you wrote:

>Hi All:     I'm in the middle of cutting up a roll of .006" LDPE to make a set

>of vertical stacked plate caps & have a question on series wired caps

>vs.parallel. Since the total voltage rating increases w/ each cap in series

>(thus spreading out the load over each cap??) am I making a stronger cap by

>building three caps w/ 5 layers of .006 Poly (.03" total)  & connecting them

>in series (.018uf each - ~.006uf total) Using Mike's conservative rating of

>350 V/mil this would be 10,500 V/cap or 31,500 V total. If I make two caps

>using 15 layers of .006 Poly (.09" total) & connect them in parallel (.003uf

>each - ~.006uf  total) 350V/mil x .09" = 31,500 V. total. I'd only have to cut

>50+ more pieces of poly for the series caps & the housing would only be a bit

>bigger so my question is which design will take more abuse? I'm using a 15kV

>30mA neon, plates will be 7" x +7"  Al. foil (extended plate design) w/ 9" x

>9" poly dielectric sheets. As long as I'm going to this much work I'd like to

>make it as "bullet proof" as possible.

Jim,

You should make the 3 caps and run them in series. Your proposal

of 2 parallel caps places the entire load across each cap.

At 15kv this can cause corona along the plate edges and ends.

Even under oil. With 3 series caps each cap sees only 1/3 of

the total voltage. In this case 5kv. At this voltage corona

will not form and you will have a reliable cap.

>Also could I "fold" the layers of poly ( 9" x whatever, back on itself to make

>9" x 9" layers of dielectric (cutting down on the cutting time) or will the

>whole sheet react as one piece of .006" poly?

>Any & all advice is more than welcome. Thanks in advance.  Jim

>

I would advise against folding the poly. It will be difficult to

get the folded sheets to lay as flat as single sheets.

Good luck.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr@aol.com

Date: Sun, 6 Dec 1998 19:32:21 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: series vs. parallel caps(thanks)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Fr. Tom & Mike:  Thanks for the quick response. The series caps were what I

thought was the best way to go ( based on some previous postings from both of

you -   Fr. Tom - glad to see you're still on the list - I've missed your

postings) but I just wanted to make sure. It's a lot of poly pieces to cut

either way so a few more won't hurt!

Thanks again,   Jim

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 7 Dec 1998 08:39:30 +0200

Subject: Re: Tentative TC design: 2-layer primary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MALCOLM@directorate.wnp.ac.nz on 07.12.98 00:39:02

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: Tentative TC design: 2-layer primary?

>Well you can make it as big as you like, but you are both increasing

>chances of a strike to the primary and not making best possible use

>of your money for a primary conductor. The two-layer technique uses

>mutual induction between primary turns to maximise the inductance you

>can squeeze out of a given length of copper.

>Malcolm

Sounds good, but I have never seen a 2-layer primary design.

- Could you please describe a little bit how the two layers should be

practically wound (e.g. inside out and then back to the inside or?)

- How I can calculate the inductance of such a layout?

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 7 Dec 1998 09:21:28 +0200

Subject: A 20 kV DC tank supply

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I would like to built a 20 kV DC supply for my big coil. These are the

tentative specs:

- 20 kV DC output

- primary capacitor 0.1 uF, charged at 20kV --> 20 Joules

- maximum DC deviation: 1%

- maximum BPS: 800 Hz

If I use a 3-phase 14 kV pole pig and 6 diodes I get a ripple of 50*6 = 300 Hz.

Voltage varies between 14 kV * 1.41 = 19.8 kV and

14 kV * 1.41 * 0.866 = 17.1 kV.

I need a flywheel capacitor to provide, roughly, enough energy for 3

primary capacitor chargings (at BPS = 800 Hz, I get almost 3 bangs between

each ripple top), that is 20 * 3 = 60 Joules allowing the top voltage to

drop down only 1%. That is:

maximum voltage drop = 20kV * 0.01 = 200V

Cflywheel = dQ/dV = 3 * Cprimary * 20kV/200 = 30 uF.

And this without taking care of losses!

I have still to protect the flywheel capacitor from discharging when the

rotary gap conducts: what about using a current limiting inductor for

feeding primarty capacitor and rotary gap?

Has anybody got some good idea or hints how to build such a DC power

supply?

Help!

P.S. Another solution would be to go with a stabilized 17,1 kV, but how to

"cut away" the ripple?

X-Originating-IP: [199.71.5.50]

From: "James C." <dragonzbreath@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Storage devices

Date: Mon, 07 Dec 1998 11:58:31 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

What exactly is a "flywheel capacitor?"

Date: Mon, 07 Dec 1998 11:05:45 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tentative TC design: 2-layer primary?

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> > To:   tesla-2@emachine.com

> > cc:    (bcc: Marco Denicolai/MARTIS)

> > Subject:  Re: Tentative TC design: first considerations

> >

> >

> > >Actually, that sounds like a go for a 60" high coil.

> >

> > Good.

> >

> > >Looks like an external primary. I'd

> > >strongly recommend building a 2-layer or more flat spiral to squeeze

> > >as much inductance as you can out of a length of tubing in as little

> > >volume as possible. Insulation between layers is mandatory.

> >

> > Why a 2-layer spiral? Why the volume must be minimized?

>

> Well you can make it as big as you like, but you are both increasing

> chances of a strike to the primary and not making best possible use

> of your money for a primary conductor. The two-layer technique uses

> mutual induction between primary turns to maximise the inductance you

> can squeeze out of a given length of copper.

>

> Malcolm

Malcom,

I tend towards the 'compressed' primary-

Could turns (flat spiral) count be equivqalent if they were either:

1) "wind-out-wind-in"

or

2) two 'paralell' coils (common feed at inner turns), one atop the

other (with 2 taps?)?

Little discusion has ocurred on this approach, and my sense is that it

should be more widely discussed.

It seems obvious that a smaller Primary OD would be nice, and provide

greater induction.

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Dec 1998 08:34:51 +1200

Subject:       Re: Tentative TC design: 2-layer primary?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Mon, 7 Dec 1998 19:39:30 +1300

> Subject:        Re: Tentative TC design: 2-layer primary?

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >Well you can make it as big as you like, but you are both increasing

> >chances of a strike to the primary and not making best possible use

> >of your money for a primary conductor. The two-layer technique uses

> >mutual induction between primary turns to maximise the inductance you

> >can squeeze out of a given length of copper.

>

> >Malcolm

>

> Sounds good, but I have never seen a 2-layer primary design.

>

> - Could you please describe a little bit how the two layers should be

> practically wound (e.g. inside out and then back to the inside or?)

> - How I can calculate the inductance of such a layout?

I've bought coiled annealed tubing from plumbing merchants and used

it straight out of the pack. I comes as a two-layer spiral; starts

from the outside, coils inward, then back to the outside on the

second layer. For a support, get a sheet of acrylic with side lengths

greater than the diameter of the finished coil and cut a hole in the

centre that is two tube thicknesses wider than the required inside

diameter of the coil. You then simply thread the tubing onto the

"former" until you reach the centre of the spirals and from there,

work from centre to outside on both bottom and top layers, spacing

and hot melting the tubing onto the former as you require.

Inductance is that of two spirals mutually coupled. I've found

in practice you can get to about 3x the inductance of a single spiral.

There are formulae in various archives for the inductance of a single

layer spiral.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Dec 1998 08:38:25 +1200

Subject:       Re: A 20 kV DC tank supply

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Mon, 7 Dec 1998 20:21:28 +1300

> Subject:        A 20 kV DC tank supply

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I would like to built a 20 kV DC supply for my big coil. These are the

> tentative specs:

>

> - 20 kV DC output

> - primary capacitor 0.1 uF, charged at 20kV --> 20 Joules

> - maximum DC deviation: 1%

> - maximum BPS: 800 Hz

>

> If I use a 3-phase 14 kV pole pig and 6 diodes I get a ripple of 50*6 = 300

> Hz. Voltage varies between 14 kV * 1.41 = 19.8 kV and

> 14 kV * 1.41 * 0.866 = 17.1 kV.

>

> I need a flywheel capacitor to provide, roughly, enough energy for 3

> primary capacitor chargings (at BPS = 800 Hz, I get almost 3 bangs between

> each ripple top), that is 20 * 3 = 60 Joules allowing the top voltage to

> drop down only 1%. That is:

>

> maximum voltage drop = 20kV * 0.01 = 200V

>

> Cflywheel = dQ/dV = 3 * Cprimary * 20kV/200 = 30 uF.

>

> And this without taking care of losses!

>

> I have still to protect the flywheel capacitor from discharging when the

> rotary gap conducts: what about using a current limiting inductor for

> feeding primarty capacitor and rotary gap?

>

> Has anybody got some good idea or hints how to build such a DC power

> supply?

>

> Help!

>

> P.S. Another solution would be to go with a stabilized 17,1 kV, but how to

> "cut away" the ripple?

Smoothing. Greg Leyh usually runs large coils with such supplies. You

might ask on the pupman list. He doesn't use a smoothing cap - just a

charging choke and diode apart from the 3-phase bridge.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Dec 1998 08:43:05 +1200

Subject:       Re: Tentative TC design: 2-layer primary?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Gene,

> Date sent:      Tue, 8 Dec 1998 08:05:45 +1300

> From:           Gene Waldo <gwaldo@1stnetusa.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: Tentative TC design: 2-layer primary?

<snip>

> Malcom,

> I tend towards the 'compressed' primary-

> Could turns (flat spiral) count be equivqalent if they were either:

>  1) "wind-out-wind-in"

>

> or

>

>  2) two 'paralell' coils (common feed at inner turns), one atop the

>     other (with 2 taps?)?

Approach 1 has the disadvantage that a high voltage end of the coil

is close to the secondary. Better to wind in, then out again. Access

to the ends is then easy too.

Approach 2 has a messy connection from the outside of one spiral to

the centre of the second. All being equal in turns, spacing,

separation etc., they are electrically equivalent.

> Little discusion has ocurred on this approach, and my sense is that it

> should be more widely discussed.

>

> It seems obvious that a smaller Primary OD would be nice, and provide

> greater induction.

Cheaper for a given inductance too. I've built several. There is

nothing to stop you stacking two of these units on top to do even

better if a much higher inductance is called for.

Malcolm

From: EdwMullin@aol.com

Date: Mon, 7 Dec 1998 18:03:14 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Storage devices

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/7/98 11:59:55 AM Eastern Standard Time,

dragonzbreath@hotmail.com writes:

<< What exactly is a "flywheel capacitor?">>

I don't know either, someone care to define, please?

X-Originating-IP: [216.68.41.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Saturable reactors?

Date: Mon, 07 Dec 1998 15:10:48 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Guys,

I've been spending some quality time reading through a fine old book of mine:

Magnetic Amplifier Engineering, by Attura, (c) 1959

Anyway - this book describes the basics of a little-known-today

component called the saturable reactor.  Looks like if I had one I could

build a quite-unkillable pig controller out of it!

Here's my question - I have access to several tape-wound transformer

cores of the 220v 20amp variety (medical grade isolation xfmrs).  Does

anyone have a clue if this core material would have the required

"square" B-H curve required to make a decent mag amplifier?  I'd love to

try this sometime, and am thinking this might be the time!  :)  It was

also suggested that variac cores might be suitable for this - comments?

I'm also wondering if there'd be room to wind control windings around

the shunts of a NST.  This would create another form of saturable

reactor - similar to the large ones used in the TIG welders of the

1970's and before...

-Always amazed at what our ancient predecessors were able to engineer!

-Bill (reverse engineering the world for my amusement)

Date: Mon, 07 Dec 1998 19:49:52 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Merry Tesla Christmas

Sender: tesl-err@emachine.com

Undoubtedly there are Tesla Coil-related tunes for the Season. Here are

a few.  Any others out there?

Arc the Herald Angels Sing

A Wave in a Manger

Bring a Spark, Jeanette Isabella

O Terminalbaum

The First Coil the Angels Did Say

The Little Oscillator Boy

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Dec 1998 15:05:46 +1200

Subject:       Re: Merry Tesla Christmas

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

One comes to mind:

A Wave In A Magnifier

You could have much fun with "On the first day of Christmas.." with

appropriate Teslarian substitutions thrown in such as:

"Four caps a blowin'

And its arcing to a pear tree"

Malcolm

X-Originating-IP: [216.68.41.206]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Merry Tesla Christmas

Date: Mon, 07 Dec 1998 18:50:22 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Well let me post my thoughts...

"Silent Light" - about a solid state coil!

There is an old Hymn titled "Jacob's Ladder" - not much of a christmas

theme but oh well...

Any additions for Malcolm's 0x0c (thats 12 in decimal) Days of Christmas

- Tesla version? "Five golden rings (RQ spark gaps?)"

-Bill

-Sittin' by the fire slurping his Bourbon Ballast (new holiday HV drink!)

>One comes to mind:

>

>A Wave In A Magnifier

>

>You could have much fun with "On the first day of Christmas.." with

>appropriate Teslarian substitutions thrown in such as:

>

>"Four caps a blowin'

>And its arcing to a pear tree"

>

>Malcolm

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 8 Dec 1998 07:04:09 +0200

Subject: Flywheel capacitor (was Re: Storage devices)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sorry everybody about the "flywheel" capacitor effect. I guess I have used

a wrong terminology :(.

With flywheel capacitor I just meant a capacitor sensibly bigger than the

primary capacitor, so that when connected in parallel with it would loose a

relatively small amount of charge.

For instance, if you charge a 10 uF capacitor with 10 kV and then connect

in parallel a 0.1uF, the resulting voltage between the two's terminals will

drop only to 10 kV - 1% = 9.9 kV.

The bigger cap works as a "flywheel" in that it keeps the original power

supply peak voltage "up" (VAC * 1.41) even when the power supply rectifier

bridge is not conducting.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Dec 1998 18:24:51 +1200

Subject:       Re: Flywheel capacitor (was Re: Storage devices)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> From:           "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Tue, 8 Dec 1998 18:04:09 +1300

> Subject:        Fllywheel capacitor (was Re: Storage devices)

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Sorry everybody about the "flywheel" capacitor effect. I guess I have used

> a wrong terminology :(.

>

> With flywheel capacitor I just meant a capacitor sensibly bigger than the

> primary capacitor, so that when connected in parallel with it would loose a

> relatively small amount of charge.

>

> For instance, if you charge a 10 uF capacitor with 10 kV and then connect

> in parallel a 0.1uF, the resulting voltage between the two's terminals will

> drop only to 10 kV - 1% = 9.9 kV.

>

> The bigger cap works as a "flywheel" in that it keeps the original power

> supply peak voltage "up" (VAC * 1.41) even when the power supply rectifier

> bridge is not conducting.

It's actually not a bad use of the term. A flywheel is the mechanical

analogue of a capacitor.

Malcolm

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Merry Tesla Christmas

Date: Tue, 08 Dec 1998 13:19:06 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Tue, 8 Dec 1998 15:05:46 +1200, you wrote:

>One comes to mind:

>

>A Wave In A Magnifier

>

>You could have much fun with "On the first day of Christmas.." with

>appropriate Teslarian substitutions thrown in such as:

>

>And its arcing to a pear tree"

Two cap-a-ci-tors

Three phase power

Four coiling nerds

Five foot sparks

Six gaps-a-sparking

..

An Eleven KV Pig

From: Hollmike@aol.com

Date: Tue, 8 Dec 1998 09:27:45 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Storage devices

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/7/98 4:06:15 PM Mountain Standard Time,

EdwMullin@aol.com writes:

>

>  << What exactly is a "flywheel capacitor?">>

>

>  I don't know either, someone care to define, please?

>

I am sure someone will correct me if I am wrong, but I believe it is a

capacitor that has a tremendous capacity for energy storage.   It would be at

least ten times the capacitance of the tank circuit cap, or more(which could

be a bit expensive).  It's purpose to to maintain a constant DC voltage in a

power supply.   This can only be used with a triggered gap, such as a rotary

gap so that the current can be stopped when enough energy has been delivered

to the tank circuit.

I can see the advantages to using such a design.  First, every firing of

the gap will deliver the same energy to the tank(at least to the point where

the the number of firings per second equals the charging ability of the supply

transformer).  Second, it would eliminate the need for timing a rotary gap

with the 60Hz peaks as is necessary with a synchronous rotary gap.

Hope this helps.

Mike

From: EdwMullin@aol.com

Date: Tue, 8 Dec 1998 21:45:15 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tesla measurement questions.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have acquired a second hand oscilloscope, for purposes not related to my

coil.  But I was wondering if I would be able to use it to measure/determine

some things about my coil.  Things I would like to know, mostly because I'm

curious:)

1. Secondary resonant frequency.

2. Change in secondary Res Freq, with Toroid and other terminals attached.

3. Spark gap break rate.

4. Can I use it to determine what frequency my primary is running at and then

use it to tune my coil?

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 9 Dec 1998 16:41:45 +1200

Subject:       Re: Tesla measurement questions.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           EdwMullin@aol.com

> Date sent:      Wed, 9 Dec 1998 15:45:15 +1300

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Tesla measurement questions.

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I have acquired a second hand oscilloscope, for purposes not related to my

> coil.  But I was wondering if I would be able to use it to measure/determine

> some things about my coil.  Things I would like to know, mostly because I'm

> curious:)

>

> 1. Secondary resonant frequency.

> 2. Change in secondary Res Freq, with Toroid and other terminals attached.

> 3. Spark gap break rate.

> 4. Can I use it to determine what frequency my primary is running at and then

> use it to tune my coil?

You can use it for all of those things and more. Use a small aerial

wire on the probe and keep the probe well away from the coil.

Malcolm

Date: Tue, 08 Dec 1998 21:38:12 -0800

From: Cliff Blackburn <repairco@axionet.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Math

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was wondering if anyone knew any good sites with math eqations for tesla coil

design

Date: Wed, 09 Dec 1998 18:11:31 -0500

From: Chris Watkins <cwatkins@seidata.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Saturable reactors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

I have been thinking of trying to wind the shunts of a microwave oven sized

transformer to possibly make it's current limit variable. -- sounds like what

you are describing.

this way I could vary the transformer current on the fly, to match my cap

have you ever seen or heard of such a setup that actually worked?

I have a bunch of sodium light ballast transformers & was considering

rewinding a couple of them to output 10kv (they are shunted)

I may never try it but I just wondered if it would work.

Chris Watkins

Bill the arcstarter wrote:

> Guys,

>  I've been spending some quality time reading through a fine old book of

> mine:

> Magnetic Amplifier Engineering, by Attura, (c) 1959

>

> Anyway - this book describes the basics of a little-known-today

> component called the saturable reactor.  Looks like if I had one I could

> build a quite-unkillable pig controller out of it!

>

> Here's my question - I have access to several tape-wound transformer

> cores of the 220v 20amp variety (medical grade isolation xfmrs).  Does

> anyone have a clue if this core material would have the required

> "square" B-H curve required to make a decent mag amplifier?  I'd love to

> try this sometime, and am thinking this might be the time!  :)  It was

> also suggested that variac cores might be suitable for this - comments?

>

> I'm also wondering if there'd be room to wind control windings around

> the shunts of a NST.  This would create another form of saturable

> reactor - similar to the large ones used in the TIG welders of the

> 1970's and before...

>

> -Always amazed at what our ancient predecessors were able to engineer!

> -Bill (reverse engineering the world for my amusement)

>

Date: Wed, 09 Dec 1998 16:57:48 -0700

From: Dan Olsen <dan@olsen.org>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Saturable reactors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Chris Watkins wrote:

> Bill,

> I have been thinking of trying to wind the shunts of a microwave oven sized

> transformer to possibly make it's current limit variable. -- sounds like what

> you are describing.

>

> this way I could vary the transformer current on the fly, to match my cap

> have you ever seen or heard of such a setup that actually worked?

On my repotted NST I have discovered that I can vary the current (by up to 50%)

by pushing the shunts in and out.  (like control rods in a Nuclear Reactor)

I don't do this while it's ON,  and I can't go below the minimum.... well

I suppose you could add more shunts. You could design some mechanical

device to do this during operation.  Just don't push them out too far, or you

will have a melt-down.(not quite as bad a nuclear reactor... but I imagine it

would make you sad.)

Date: Wed, 09 Dec 1998 18:45:25 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Saturable reactors?

Sender: tesl-err@emachine.com

Dan Olsen wrote:

> On my repotted NST I have discovered that I can vary the current (by up to 50%)>

> by pushing the shunts in and out.  (like control rods in a Nuclear Reactor)

I discovered the same thing when I used the primary of a NST for the

choke in the subwoofer I built for my stereo system. I can audibly vary

the high frequency cutoff point by more than an octave.

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 10 Dec 1998 07:56:52 +0200

Subject: Quenching

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

If my big TC resonance frequency is, say, 62 kHz, its period is 1/62000 =

16 us. Two things I don't understand:

- many have always been telling that, for optimal operation, the spark gap

must quench just after the primary first oscillation period i.e. 16 us

after the bang start. (I don't remember how you call that "period"). If I

use a rotary gap, what happens in terms of performance if I run fast, small

electrodes ---> spark time less than 16 us?

- If after a certain time the gap quenches (i.e. opens) how current can

keep flowing into the primary? How the primary can still oscillate? Does it

do it through all the tank supply (across the spark gap)?

- doesn't a limited discharge time ( < 16 us) of the primary capacitor mean

also that I don't use all the energy stored in the primary capacitor for

the bang?

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Dec 1998 19:11:09 +1200

Subject:       Re: Quenching

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> If my big TC resonance frequency is, say, 62 kHz, its period is 1/62000 =

> 16 us. Two things I don't understand:

>

> - many have always been telling that, for optimal operation, the spark gap

> must quench just after the primary first oscillation period i.e. 16 us

> after the bang start. (I don't remember how you call that "period"). If I

> use a rotary gap, what happens in terms of performance if I run fast, small

> electrodes ---> spark time less than 16 us?

It *might* but this is highly unlikely.

> - If after a certain time the gap quenches (i.e. opens) how current can

> keep flowing into the primary? How the primary can still oscillate? Does it

> do it through all the tank supply (across the spark gap)?

It can't oscillate under those conditions. The tank effectively no

longer exists if the gap is interrupted.

> - doesn't a limited discharge time ( < 16 us) of the primary capacitor mean

> also that I don't use all the energy stored in the primary capacitor for

> the bang?

It does. You ideally want to quench on completion of the first

primary energy transfer - i.e. when the primary has oscillated down

to zero energy and all the energy is bottled up in the secondary.

Malcolm

Date: Thu, 10 Dec 1998 07:16:43 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quenching

Sender: tesl-err@emachine.com

Marco,

Check out the excellent articles on Tesla Coil theory that Barry Benson

mentioned yesterday. These should provide virtually all the answers you

seek:

www.peakpeak.com/~terryf/tesla/misc/

dolnger1.jpg through dolnger8.jpg

Very brief answers are also provided below.

-- Bert --

Marco Denicolai wrote:

>

> If my big TC resonance frequency is, say, 62 kHz, its period is 1/62000 =

> 16 us. Two things I don't understand:

>

> - many have always been telling that, for optimal operation, the spark gap

> must quench just after the primary first oscillation period i.e. 16 us

> after the bang start. (I don't remember how you call that "period"). If I

> use a rotary gap, what happens in terms of performance if I run fast, small

> electrodes ---> spark time less than 16 us?

Quenching should occur after most of the primary energy has transferred

to the secondary - typically after a number of primary oscillations not

just one...

>

> - If after a certain time the gap quenches (i.e. opens) how current can

> keep flowing into the primary? How the primary can still oscillate?

It doen't! However at the time of an ideal quench, little energy is left

in the primary circuit since most of it's been transferred to the

secondary or dissipated vial system losses (mainly gap).

> Does it do it through all the tank supply (across the spark gap)?

>

> - doesn't a limited discharge time ( < 16 us) of the primary capacitor mean

> also that I don't use all the energy stored in the primary capacitor for

> the bang?

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Saturable reactors?

Date: Thu, 10 Dec 1998 06:32:29 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey Chris, list,

You wrote:

>I have been thinking of trying to wind the shunts of a microwave oven sized

>transformer to possibly make it's current limit variable. -- sounds like

>what you are describing.

After reading more it turns out that regular transformer iron does not

have the square B-H curve required for a saturable reactor.  Now, that

being the case - I bet that you could wind your MOT (if there is

physical room for the windings) to 'bias-out" those shunts, and achieve

some sort of control.

>this way I could vary the transformer current on the fly, to match my cap

>have you ever seen or heard of such a setup that actually worked?

TIG welders from the 70s had a 3-legged iron core - the primary winding

was on the leftmost leg, and the welder's output was on the rightmost

leg.  The center leg has an airgap sawed through it, and also contains

the control windings.

The welder would control the current flowing through these control

windings, which would cause varying degrees of saturation of the middle

leg - thereby adjusting the output current of the transformer!

Best of luck to you, Chris.  Let us know if it works!

-Bill

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Saturable reactors?

Date: Thu, 10 Dec 1998 06:38:23 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tom wrote:

>Dan Olsen wrote:

>

>> On my repotted NST I have discovered that I can vary the current (by up to 50%)

>> by pushing the shunts in and out.  (like control rods in a Nuclear Reactor)

>

>I discovered the same thing when I used the primary of a NST for the

>choke in the subwoofer I built for my stereo system. I can audibly vary

>the high frequency cutoff point by more than an octave.

Some of the low-end stick welders have a dial on top which sets the

current.  This dial causes some form of shunt to be inserted into the

core, just like what you are describing.

The really cheap ones (like mine) just have a bunch of sockets - you

plug the cables into the proper set of sockets - and that taps the

transformer for whatever power level is required.

I'm wouldn't have thought a NST would have worked very well at audio

frequencies - the laminations are only thin enough to suppress the eddy

currents at 60hz of course.  But I suppose if this was for a Low-pass

filter then it wouldn't matter!

Thats certainly a novel use fo an NST!

-Bill

Date: Thu, 10 Dec 1998 11:44:11 -0800

From: Adam Pasternak <pastea2@rpi.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm making a quick style cylindrical gap for my coil...i have ideas but

i want some suggestions

I'm using a 12kV 60mA NST....cap is approx. 0.012uF.

1) What diameter PVC should I use

2) How many copper pipes should I use, are longer pipes better for

better heat distribution/quenching

3) Space between pipes:  it is better to overspace or underspace when tuning

total spark gap = sum of spaces between pipes?  will it function the same way?

X-Mailer: IPAD 2.0

Date: Thu, 10 Dec 1998 14:13:10 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:44 AM 12/10/98 -0800, you wrote:

>I'm making a quick style cylindrical gap for my coil...i have ideas but

>i want some suggestions

>

>

>I'm using a 12kV 60mA NST....cap is approx. 0.012uF.

>

>1) What diameter PVC should I use

For 12kv/60 a quite satisfactory gap can be fitted into

4 inch diameter PVC pipe.

>2) How many copper pipes should I use, are longer pipes better for

>better heat distribution/quenching

Electrodes cut from hard 3/4 inch copper water pipe work well.

Make them 3 inches long. Somewhat longer pipes up to 4 inches

may help at higher power levels by creating a larger thermal

mass. Above 4 inches you wont gain much. At that point it would

be better to go to larger diameter pipe for your electrodes.

Ten electrodes of 3/4 inch copper just fit in 4 inch PVC.

This provides 9 series gaps. Spaced at .025 inches each that

provides a maximum of .225 inch of gap. This places the first

and last electrodes quite close together so insert a thick plastic

barrier between them to prevent the arc from cheating.

A total spacing of .225 provides good performance on my coil

at 12kv/100ma. Provide plenty of airflow for cooling and to help

improve quenching.

Consider .250 inch the maximum you should ever run at 12kv.

>3) Space between pipes:  it is better to overspace or underspace when

>tuning.

The spacing between the individual electrodes should be fixed

at .025-.03 inch. For tuning and other uses requiring a limit

on power level use less than the maximum number of gaps.

>

> total spark gap = sum of spaces between pipes?  will it function

>the same way?

Yes the total spacing is equal to the sum of the individual

gaps. But remember you dont have to use all of the gaps.

Only use as many of the available gaps as your system will tolerate.

How do you know what your system will tolerate?

By observing your safety gap. If the safety gap is set to break

down just slightly above your rated neon voltage it will warn

you when the total gap is too wide. If you are getting lots

of safety gap firings then use 1 less main gap. If your coil is running

smoothly and is in good tune then add one additional gap to increase

primary energy level. The use of overwide gaps places great amounts

of stress on the transformer as well as your caps.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 11 Dec 1998 08:43:52 -0700

From: Dan Olsen <dan@olsen.org>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: First Light!!! & serious EMI

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got everything together last night :

12kv 60mA NST, 4.5" x 23" Secondary, 12 turn Primary

And I was THRILLED to get 12-15 inch Power Arcs!!!!!  I think I can get

more out of it.  I am shooting for 3-4 feet with more tuning, but I

was happy with that for my first go ever, having never having seen

another coil in person.

Now the problems:

I got Lots of complaints from other members of my household while my

coiling activities were going on that there was very loud static on

the phone lines, and that the touch activated lamps in the house were

countiously cycling through all of their brightness levels.  As well

as the the ethernet network of 3 computers in the house all quit

talking to each-other (hopefully the cards or hub aren't damaged).

None of the neighbors have complained (yet), but if I can't address

some of these issues, my coiling activities, are going to move from an

amusing and tolerated hobby, to a seriously discouraged activity in my house.

Any Ideas?  If plug my NST into a good surge protector, that might

keep the EMI out of the mains lines, but the phone interference...

what can be done?  How much will this diminish as my coil is tuned

better, and more of the energy goes into sparks?

Thanks for all of your help.  I learned lots of good stuff from the

list most of the time after I had ruined something, or done something

the hard-way, I would learn a better way from the list.  (that sounded

like a good-bye kind-of thank-you, hopefully it will be an

announcement of my joining (and remaining in) the ranks of coilers.

Thanks for letting me join in your Fun!

--Dan

From: Hollmike@aol.com

Date: Fri, 11 Dec 1998 10:46:51 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First Light!!! & serious EMI

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/11/98 8:43:01 AM Mountain Standard Time, dan@olsen.org

writes:

>

>  Any Ideas?  If plug my NST into a good surge protector, that might

>  keep the EMI out of the mains lines, but the phone interference...

>  what can be done?  How much will this diminish as my coil is tuned

>  better, and more of the energy goes into sparks?

>

Dan,

You could use an EMI/RFI filter on the line to prevent the interference

from entering the mains.  You could also construct a Faraday cage around the

coil to keep the radiated enery contained.

Hope this helps.

Mike

From: EdwMullin@aol.com

Date: Fri, 11 Dec 1998 21:35:45 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Shocking Losses

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was running my coil this evening, trying to get it into tune.  The best I

could get was about 10-12 inches from it.  I think there should be a lot

more...  I did notice a lot of corona and a couple of streamers emanating from

the top of the coil, so I'm sure I'm loosing energy there.  The question is,

how can I eliminate or reduce that corona?

Additionally, is there anywhere else that there could be lost energy without a

visible display?   I did see some small sparks coming from the corners of a

metal cabinet a few feet away. and I could pull sparks from the table saw with

a screw driver...

Also I noticed, after I had shut down for the evening, I was getting small

shocks from the top 3rd of the secondary,  there was no power to any part of

the circuit (mains plugs out) and I had a shunt installed across the capacitor

terminals.  Where did these shocks come from? It was repeatable, even after I

touched a grounded wire to the top terminal.

Date: Sat, 12 Dec 1998 14:11:27 +1100

From: Corey Kent <corey@enterprise.net.au>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: just a quick question

Sender: tesl-err@emachine.com

hi,

i was wondering were i could get a auto ignition coil from???

how much do they cost and so on and do i need to change it in anyway?

Date: Fri, 11 Dec 1998 21:17:00 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Shocking Losses

Sender: tesl-err@emachine.com

EdwMullin@aol.com wrote:

> Also I noticed, after I had shut down for the evening, I was getting small

> shocks from the top 3rd of the secondary,  there was no power to any part of

> the circuit (mains plugs out) and I had a shunt installed across the capacitor

> terminals.  Where did these shocks come from? It was repeatable, even after I

> touched a grounded wire to the top terminal.

ME TOO!  I thought it was the distributed capacitance in the secondary

holding a charge. Seems like it would drain off, but it doesn't.

As to your corona, what shape and size is your top terminal?

From: EdwMullin@aol.com

Date: Fri, 11 Dec 1998 22:20:30 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: just a quick question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Try asking at a local garage, sometimes they keep old ones around.  Junk yard

could be a source for cheap ones too... although then you may just be better

off bying the cheapest one at the auto parts store.   The "high performance"

coils work better at higher frequencies, but can cost upwards of $60...

Good Luck!!

Date: Fri, 11 Dec 1998 22:06:49 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Transformers for Europeans

Sender: tesl-err@emachine.com

I just had a thought for those of you in the Netherlands and elsewhere

where NSTs are hard to find.  Has anyone tried using a string of small

inexpensive transformers, putting the windings in series and parallel?

For example, take ten 120v to 12.6v power transformers.  Put the 120v

windings in series, the 12.6v in parallel, then run 120 volts through

the 12.6 v sides.  Theoretically, you should get about 12kv out, with

pretty hefty current.

I'm wondering if there is a possibility of serious danger here; is it

likely that insulation on these is rated for about 1000v?  Will it work?

I expect some pretty vigorous feedback on this one!

Date: Sat, 12 Dec 1998 00:49:33 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: just a quick question

Sender: tesl-err@emachine.com

Corey Kent wrote:

>

> hi,

> i was wondering were i could get a auto ignition coil from???

> how much do they cost and so on and do i need to change it in anyway?

Car parts store for a new one, junk yard for a cheap one.  Don't know

the price, but now that should be easy enough for you to find out.

From: Sam Laur <slaur@sekunda.pp.utu.fi>

Subject: Re: Transformers for Europeans

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 12 Dec 1998 09:14:17 +0200 (EET)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> For example, take ten 120v to 12.6v power transformers.  Put the 120v

> windings in series, the 12.6v in parallel, then run 120 volts through

First, we get 230V here, not 120V.

> I'm wondering if there is a possibility of serious danger here; is it

> likely that insulation on these is rated for about 1000v?  Will it work?

I wouldn't count on it - first of all, the "small transformers" I've seen

mostly have layered windings, i.e. the primary is beneath the secondary,

not beside it. And at either end of the "chain" you'll be getting even more

of a voltage difference between the primary and the secondary. I'd think it

wouldn't work...

From: Hollmike@aol.com

Date: Sat, 12 Dec 1998 08:42:45 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Transformers for Europeans

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/11/98 9:10:06 PM Mountain Standard Time,

tpolk@prismnet.com writes:

>

>  For example, take ten 120v to 12.6v power transformers.  Put the 120v

>  windings in series, the 12.6v in parallel, then run 120 volts through

>  the 12.6 v sides.  Theoretically, you should get about 12kv out, with

>  pretty hefty current.

>

That is if the insulation can handle it.

X-Originating-IP: [199.212.10.92]

From: "James C." <dragonzbreath@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: just a quick question

Date: Sat, 12 Dec 1998 09:57:07 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Any automotive supply place, or local garage always has many on-hand, or

you can head down to your local wrecking yard with vise-grips and and a

screwdriver and rip one out of an older car.

The one I picked up was brand new, Canadian Tire, $12

Date: Sat, 12 Dec 1998 12:03:08 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: current limiting

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To all-

I have came across a small portable welder and I need to know if it will

do the job on limiting my potential transformer. There are two input

voltages. The welder is designed to run on normal household current. One

input is 120v@15A /6v@15-70A out using a ?resistive? welding rod. The

other is 120v@20A /6v@15-70A out using a standard type welding rod.

Does anyone know if this portable unit will work to limit me to around

30A using the 15A input setting.

Chris

X-Originating-IP: [216.68.41.96]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: current limiting

Date: Sat, 12 Dec 1998 10:16:14 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Chris wrote:

>I have came across a small portable welder and I need to know if it will

>do the job on limiting my potential transformer. There are two input

>voltages. The welder is designed to run on normal household current. One

>input is 120v@15A /6v@15-70A out using a ?resistive? welding rod. The

>other is 120v@20A /6v@15-70A out using a standard type welding rod.

>Does anyone know if this portable unit will work to limit me to around

>30A using the 15A input setting.

I'm guessing this is one of those "home" mini-stick welders for the

small shop, etc.  I'm not sure what a "resistive" as opposed to normal

welding rod would be - the heat for the welding comes from the power

delivered to the arc, not due to resistive heating of the welding rod!

Some of the smaller home welders sell some really tiny welding rods

which will actually weld at 50-60 (secondary) amps on sheet metal.

If you can run your pig off of 120 volts - then using this welder on the

20 amp setting would limit your pig current to about 20 amps.

If you try to use this welder to limit the current off of 240 volts -

you'll let the smoke out of it!  The core will saturate, and it will

draw some huge current from the lines.

The bottom line is: whatever current limiting device you choose must be

able to withstand the voltage you are using.

Look in your local farm/trading post newspaper - see if you can get an

old beatup stick welder.  That will allow you to limit your pig at

240volt 10-40 amps.  I got an old 150lb beast of a stick welder for $50

a few years ago...

If worse comes to worst - you can get a new "el-cheapo" stick welder for

about $150-$200.

-Bill

From: EdwMullin@aol.com

Date: Sat, 12 Dec 1998 13:28:46 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Shocking Losses

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/11/98 10:21:15 PM Eastern Standard Time,

tpolk@prismnet.com writes:

<< EdwMullin@aol.com wrote:

> Also I noticed, after I had shut down for the evening, I was getting small

> shocks from the top 3rd of the secondary,  there was no power to any part of

> the circuit (mains plugs out) and I had a shunt installed across the capacitor

> terminals.  Where did these shocks come from? It was repeatable, even after I

> touched a grounded wire to the top terminal.

ME TOO!  I thought it was the distributed capacitance in the secondary

holding a charge. Seems like it would drain off, but it doesn't.

>As to your corona, what shape and size is your top terminal?

I was initially using a pointed terminal, for initial tuning then I went to a

6" diam ball.

From: EdwMullin@aol.com

Date: Sat, 12 Dec 1998 13:31:41 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: current limiting

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have seen a lot of postings about using welders to current limit

distribution transformers, but just how would you connect the two together to

achieve that?

Date: Sat, 12 Dec 1998 14:27:01 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Shocking Losses

Sender: tesl-err@emachine.com

EdwMullin@aol.com wrote:

>

> In a message dated 12/11/98 10:21:15 PM Eastern Standard Time,

> tpolk@prismnet.com writes:

>

> << EdwMullin@aol.com wrote:

>

>  > Also I noticed, after I had shut down for the evening, I was getting small

>  > shocks from the top 3rd of the secondary,  there was no power to any part of

>  > the circuit (mains plugs out) and I had a shunt installed across the capacitor

>  > terminals.  Where did these shocks come from? It was repeatable, even after I

>  > touched a grounded wire to the top terminal.

>

>  ME TOO!  I thought it was the distributed capacitance in the secondary

>  holding a charge. Seems like it would drain off, but it doesn't.

>

>  As to your corona, what shape and size is your top terminal?

>

>

> I was initially using a pointed terminal, for initial tuning then I went to a

> 6" diam ball.

Your terminal seems undersized for the size of your secondary.  A much

larger one stores a lot more energy and helps keep energy from draining

off.  For a cheap one, I recommend a toroid made of aluminum flex tubing

like dryer vent material.  You'll be amazed at the difference in output.

It will probably solve the corona problem, too.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 12 Dec 1998 17:13:51 EST

Subject: Re: current limiting

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sat, 12 Dec 1998 13:31:41 EST in Tesla-2

<EdwMullin@aol.com> wrote:

>I have seen a lot of postings about using welders to current limit

>distribution transformers, but just how would you connect the two

>together to achieve that?

Hi Ed, to use a welder to current limit a pole pig you short out the

welders secondary and but the welders primary in series with the pole

pig. The purpose is to current limit the pole pig. Pole

pigs unlike neon sign transformers or obits is not current limited and if

connected directly to the mains when the secondary is shorted as it is

when the spark gap fires will try to pull infinite amounts of power from

the mains and constantly blow the breakers.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

X-Originating-IP: [216.68.37.239]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: current limiting

Date: Sat, 12 Dec 1998 14:18:46 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>I have seen a lot of postings about using welders to current limit

>distribution transformers, but just how would you connect the two

together to

>achieve that?

Basically you wire your welder primary (which would normally connect to

the 240 supply of the garage) in series with the pig's 240 side.  The

welder already contains a current-limited transformer.  In operation,

you short-out the output leads of the welder.  The selection of the

welder's secondary output current controls how much current it will let

flow into the pig.

My welder, at the 180 amp (secondary) setting draws 42 amps at 240 volts

on the primary.  Meanwhile it is delivering a measured 25 volts to a

0.15 ohm load, a power of about 4200 watts.  Note, however that 42 amps

* 240 volts = 10080 vvolt*amps, quite different from 4200 watts.

In case you are wondering, my 0.15 ohm resistor is a graphite bar with

copper plated ends, about 3/16 inch diameter, about a foot long.  It

gets bright red hot in about 2 seconds at 4200 watts! :)  If anyone had

a need for anything like this, contact me as I have a whole load of

them.

At idle, the welder draws about 1 amp at 240 volts.  It contains some

sort of power factor correction capacitor inside the case.

I haven't put the 'scope on it, but surely this indicates the highly

inductive nature of the current going into the welder, saturating

various legs of the transformer, etc...

As of yet, I haven't tried running my pig in series with the welder, but

I think I will sometime this weekend.

-Bill

Date: Sat, 12 Dec 1998 18:32:21 -0600

From: Brian Christian <brianchr@tsixroads.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Plastic capacitor oil.

Sender: tesl-err@emachine.com

List,

I recently got my hand upon a .005 Mfd 30KVDC commercially made

capacitor from plastic caps.  One problem, upon shipping the glass

casing was shattered.  It leaked a thick clear, stick substance.  It is

probably a commercially made oil.  I was wondering if I could substitute

mineral oil for this, soaking the capacitor in it.  Does anyone know

anything about the substance in plastic caps, and if this is feasible?

Thank in advance,

Brian Christian

X-Mailer: IPAD 2.0

Date: Sat, 12 Dec 1998 18:49:13 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Math

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:38 PM 12/8/98 -0800, you wrote:

>I was woundering if anyone knows of any good sites with math

>equations for Tesla coil design?

>

Cliff,

You can start by looking on Dave Bronakowski's archive page.

Download the file math.txt. It has a bunch of useful formulas

and equations. Also check the file math2.txt for Wheeler's

inductance formula and Medhurst's formula for self capacitance.

Those two formulas allow you to calculate the resonant frequency

of your secondary coil quite accurately.

Also take a look at Bob Schumann's web page at:

http://www.america.com/~tesla/

Bob has a useful collection of formulas there.

For basic coil design try out the WinTesla program

from Scott Coppersmith's page:

http://members.aol.com/rscopper/index.htm

Its also available on Dave's archive page.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 12 Dec 1998 21:10:42 EST

Subject: Re: Transformers for Europeans

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Fri, 11 Dec 1998 22:06:49 -0600 Tom Polk

<tpolk@prismnet.com> wrote:

>I just had a thought for those of you in the Netherlands and >elsewhere

where NSTs are hard to find.  Has anyone tried using a >string of small

inexpensive transformers, putting the windings in >series and parallel?

>

>For example, take ten 120v to 12.6v power transformers.  Put the >120v

windings in series, the 12.6v in parallel, then run 120 volts >through

the 12.6 v sides.  Theoretically, you should get about >12kv out, with

pretty hefty current.

>

>I'm wondering if there is a possibility of serious danger here; is

>it likely that insulation on these is rated for about 1000v?  Will >it

work?

Years ago when transistors were just gaining popularity many of us would

use impedance matching transformers that would couple the low impedance

speakers (8 ohms) to the high impedance output of the audio circuits

(1000+ ohms)by wiring a group of say five or more transformers with all

the secondaries in series and all the primaries in parallel and used

vibrators to supply pulsating D.C. to the low impedance side of the bank

of transformers and get several thousand volts out of the secondaries.

Back then transformers were over built and over insulated so it would

work, but today transformers a built with very little lee-way so it

probably won't work and all you would do is burn out the insulation,

besides these did not produce enough power to drive any kind of a Tesla

coil.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

Date: Sat, 12 Dec 1998 21:02:36 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Plastic capacitor oil.

Sender: tesl-err@emachine.com

Brian Christian wrote:

>

> List,

>         I recently got my hand upon a .005 Mfd 30KVDC commercially made

> capacitor from plastic caps.  One problem, upon shipping the glass

> casing was shattered.  It leaked a thick clear, stick substance.  It is

> probably a commercially made oil.  I was wondering if I could substitute

> mineral oil for this, soaking the capacitor in it.  Does anyone know

> anything about the substance in plastic caps, and if this is feasible?

> Thank in advance,

> Brian Christian

Brian,

Most likely it was simply highly purified and inhibited mineral oil per

Plastic Capacitor's product literature. Replace it with the same and it

should be OK! BTW, these are not the type of package recommended for

Tesla Coil service, since the cap can overheat internally, causing the

glass container to explode. Also, red-colored caps use Mylar as the

dielectric, while the blue-colored ones use polypropylene.

While it MAY be possible to use glass-packaged polypropylene caps in low

power Tesla Coil service, you should definately put them inside a

container to protect you in case they overheat and explode. Mylar caps

in any package will definately NOT work properly in Tesla Coil service:

they WILL rapidly overheat, and most likely WILL explode - this has

happenned with both metal and glass-packaged Mylar caps to many an

unwary coiler!

Check out Plastic Caps' webpage at:

http://www.plasticcapacitors.com

The type of cap you've got is probably in the "OF" family of DC filter

caps - check out:

http://www.plasticcapacitors.com/typeof.html

It looks as though Plastic Caps is getting ready to offer Tesla Coil

capacitors from the looks of the bottom of their webpage (they actually

do now, but don't advertise them to any extent). Finally, for those of

you contemplating home-brew capacitor design, take a look (and take to

heart) Bill Meskan's excellent technical summaries on film capacitor

fabrication/design and capacitor specification in the Technical Bulletin

section of the web site:

http://www.plasticcapacitors.com/bulletin.html

Bill knows his stuff - he's also the owner of the company!

Safe coilin' to you, Brian!

-- Bert --

Date: Sun, 13 Dec 1998 14:15:58 +1100

From: Corey Kent <corey@enterprise.net.au>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ignition Coil Question 2.

Sender: tesl-err@emachine.com

Will i have to make any changes to the coil???

is there any instructions on how to use the coil for a jacobs ladder

and a tesla coil?

thanks

cya's

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 12 Dec 1998 22:18:19 EST

Subject: Re: Math

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sat, 12 Dec 1998 18:49:13 -0600 Mike Hammer <mhammer@misslink.net>

wrote:

>Cliff,

>You can start by looking on Dave Bronakowski's archive page.

What's the URL for Dave Bronakowski's archive page? You didn't give it in

that message.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

X-Mailer: IPAD 2.0

Date: Sun, 13 Dec 1998 00:35:25 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Math

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:18 PM 12/12/98 EST, you wrote:

>On Sat, 12 Dec 1998 18:49:13 -0600 Mike Hammer <mhammer@misslink.net>

>wrote:

>

>>Cliff,

>>You can start by looking on Dave Bronakowski's archive page.

>

>What's the URL for Dave Bronakowski's archive page? You didn't give it in

>that message.

>

Geez. What a bonehead I am these days.

Dave's URL is: http://www.velocity.net/~djb/tesla.shtml

Sorry about that.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 13 Dec 1998 02:55:13 EST

Subject: Re: Math

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sun, 13 Dec 1998 00:35:25 -0600 Mike Hammer <mhammer@misslink.net>

wrote:

>At 10:18 PM 12/12/98 EST, you wrote:

>>On Sat, 12 Dec 1998 18:49:13 -0600 Mike Hammer <mhammer@misslink.net>

>>wrote:

>>

>>>Cliff,

>>>You can start by looking on Dave Bronakowski's archive page.

>>

>>What's the URL for Dave Bronakowski's archive page? You didn't >>give

it in that message.

>>

>Geez. What a bonehead I am these days.

>Dave's URL is: http://www.velocity.net/~djb/tesla.shtml

It can happen to the best of us! Thanks for the info! BTW for the past 12

hours or so ever time I send a message to the Tesla-2 list

I get one of these:

From: System Administrator <postmaster@JDCORP.deere.com>

Subject: Undeliverable:

Your message

To:      Tesla-2 Mailing List

Subject: Re: Math

Sent:    Sat, 12 Dec 1998 21:18:19 -0600

did not reach the following recipient(s):

gorzneyrogerd@jdcorp.deere.com on Sun, 13 Dec 1998 00:21:56 -0600

The recipient name is not recognized

MSEXCH:IMS:DEERE:JDCORP:E90IMC2 0 (000C05A6) Unknown recipient

ect...

Any ideas on what gives?

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

Sender: tesl-err@emachine.com

Date: Sun, 13 Dec 1998 10:50:39 -0500

From: Tom Stock <tomstock@geocities.com>

Organization: #813

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: NST's for sale

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have 4 extra 15kv/30 ma Neons for sale...

I'd like $20 each for them (+ shipping). If you're interested let me know.

cya

-tom

Date: Sun, 13 Dec 1998 11:31:05 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New coil design *possibly*

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A while back there was a thread on what the advantages of double winding

a secondary coil would be. I got to thinking about the fact that if it

would in fact reduce the resistance of the secondary, would it also add

a little bit more power handling capability. I know that by winding a

double layer coil with both wires in parallel will reduce its resistance

by acting as one larger size conductor.

But I thought about instead of winding layer over layer, you could wind

the wires side by side. The reason I thought of winding side by side is

the fact that it would allow you to use smaller gauge wire without going

over a 1000 turns. I also thought of the fact that smaller gauge wire

such as between 20-24 awg is easier and cheaper to come by.

Tell me what everyone thinks. This can definately become a reality if

its benefits out weigh a single layer larger gauge secondary.

Chris

X-Mailer: IPAD 2.0

Date: Sun, 13 Dec 1998 11:57:23 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:error messages (was Math)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:55 AM 12/13/98 EST, you wrote:

>BTW for the past 12 hours or so ever time I send a message

>to the Tesla-2 list I get one of these:

>

>From: System Administrator <postmaster@JDCORP.deere.com>

>Subject: Undeliverable:

>

>Your message

>

>  To:      Tesla-2 Mailing List

>  Subject: Re: Math

>  Sent:    Sat, 12 Dec 1998 21:18:19 -0600

>

>did not reach the following recipient(s):

>

>gorzneyrogerd@jdcorp.deere.com on Sun, 13 Dec 1998 00:21:56 -0600

>    The recipient name is not recognized

>    MSEXCH:IMS:DEERE:JDCORP:E90IMC2 0 (000C05A6) Unknown recipient

>

>ect...

>

>Any ideas on what gives?

>

Alfred and all,

This error message and others that you may receive like

it from time to time are the unfortunate result of the

present list software we are using and the non-standard

manner in which some service providers handle bounced mail.

SOME systems ignore the error address provided and instead

route the bounced mail back to the original author of the

email. Once we resolve the problem with this subscriber

the error message will no longer show up.

I know these errors are an irritant but there is really nothing

that can be done about it in the short term. Just delete

and ignore them if and when they show up.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 13 Dec 1998 12:55:28 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New coil design *possibly*

Sender: tesl-err@emachine.com

mel wise wrote:

>

> A while back there was a thread on what the advantages of double winding

>  a secondary coil would be. I got to thinking about the fact that if it

>  would in fact reduce the resistance of the secondary, would it also add

>  a little bit more power handling capability. I know that by winding a

>  double layer coil with both wires in parallel will reduce its resistance

>  by acting as one larger size conductor.

>

>  But I thought about instead of winding layer over layer, you could wind

>  the wires side by side. The reason I thought of winding side by side is

>  the fact that it would allow you to use smaller gauge wire without going

>  over a 1000 turns. I also thought of the fact that smaller gauge wire

>  such as between 20-24 awg is easier and cheaper to come by.

>

>  Tell me what everyone thinks. This can definately become a reality if

>  its benefits out weigh a single layer larger gauge secondary.

>

>  Chris

Hi Chris!

Excellent question!

The major advantage in winding one layer over the other and then

connecting the windings in parallel is that the coil inductance and

natural resonant frequency stay virtually the same, while the DC and AC

resistances are significantly decreased. And, as you suggest, this

reduces losses, thereby increasing the Quality Factor, or "Q", of the

coil. Bert Pool is using this approach quite effectively on the 3rd coil

of his magnifier system on the main Tesla Builders List.

Winding a pair of wires side-by-side is called a bifilar winding, and

this approach is often used for RF or high-current transformers. If you

were to wind your secondary coil in this fashion, you'd end up with half

the number of "effective" turns and, since the coil's inductance is a

function of the number of turns squared, you'd end up with about 1/4 the

inductance versus a single-winding coil of the same length. Since the

self-capacitance of the coil would remain the same, the natural resonant

frequency of the secondary will now be about twice what is was before.

Although you'll have about half the DC resistance, doubling the

frequency now increase the AC (skin-effect and proximity effect) losses

of the winding. Since these losses tend to go up as the square-root of

frequency, the net effect is that your coil will have twice the resonant

frequency, and Q will increase by a factor of about 1.4 versus a

single-winding coil using the same wire gauge. This is the GOOD news!

The BAD news is that the coil's unloaded operating frequency will be

twice as high, making for poorly formed and shorter streamers versus a

lower-frequency system. Well, let's suppose we add heavier topload

capacitance so that we resonate at a lower frequency. We'd then need to

increase the primary operating voltage in order to maintain similar coil

output voltage (assuming we use the same sized tank cap).

However, it gets worse! Remember that we have _halved_ the number of

"effective" turns on the secondary. This really means that we've DOUBLED

the turn-turn voltage stress for comparable output voltages. A bifilar

wound coil will be MUCH more likely to have turn-turn flashovers than a

single-layer coil at comparable power and voltage levels.

Sorry for the rather long-winded answer to your question, but I wanted

you to understand the theory and reasoning behind it. In coiling, like

much of life, there ain't no free lunch! :^)

Safe coiling to you, Chris!

-- Bert --

From: <corr@enid.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: I'm back! :-)

Date: Sun, 13 Dec 1998 13:05:23 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well everyone, after only 4 or 5 months, but what seems like more, I have

returned from my absence on the Tesla and Tesla-2 lists. My coil output

has hit a 70 " max until I finish the new capacitors and get the spark gap

from the list moderator, Mike Hammer. With this new cap and gap setup I hope

to be able to achieve much more output than I had been getting before.

I hope to take pictures from the first run I do on this equipment.

I have gotten to the point that I hit the walls too often, and I am going to

have to move my coil outside during a still night to get the best out of it.

I am currently running 15kv at around 141ma of current on a 6.25" secondary.

For a complete list of stats, please visit my web page:

http://www.harvestcomm.net/personal/corr

Happy Coiling to everyone!

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: FREE TO GOOD HOME!!!

Date: Sun, 13 Dec 1998 13:40:47 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

I have 2 12KV 30mA NST's that need to be unpotted. One side

worked on each one, but I've checked the windings with a meter

& unpotting them should bring them back to their full life. One is

a france & the other looks like a france but has no tag on it.

If you want them email me & you pay for the shipping via UPS.

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

Date: Sun, 13 Dec 1998 14:00:15 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: FREE TO GOOD HOME!!!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Choad wrote:

>

> Hello all,

>

>     I have 2 12KV 30mA NST's that need to be unpoted. One side

> workd on each one, but I've checked the windings with a meter

> & unpoting them should bring them back to their full life. One is

> a france & the other looks like a france but has no tag on it.

> If you want them email me & you pay for the shipping via

> UPS.

>

> Choad

> Member of The Oklahoma Tesla Coil Builders

> http://www.geocities.com/SouthBeach/Pier/2756/index.htm

Choad,

I want them. I live in Texas. Area code is 75801.

E-mail me with the details.

Chris

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Request for directions [off topic]

Date: Sun, 13 Dec 1998 13:17:27 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Importance: Normal

Sender: tesl-err@emachine.com

I don't want to bother the list with this, but would appreciate either

offline discussion, or pointers to where I would find assistance.

I am trying to get a system using a combination of 555 + Ignition Coil working

[for a plasma globe type display, I hope]. I have obviously gone

wrong somewhere, as there is no HV from the coil, but one heck of

a magnetic field. I suspect I am tripping over an error in the design

of the 555 circuit, and would appreciate discussion from someone that

has done similar.

Thanks

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: FREE TO GOOD HOME!!!

Date: Sun, 13 Dec 1998 16:09:35 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I figured in the shipping and pasted it here. I'll send them

ground unless you just want them sooner.

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

Shipment Cost

Service:  Ground (residential)

Origin:  73703, US

Destination:  75801, US

Zone:   004

Weight:  55

Base Service Charge:  $16.18

Optional Service

Charge:  $0.00

--------------------------------------------------------------------

------

Total Charge:  $16.18

Other Available UPS Services

UPS Service  Guaranteed by  Total Charge  Zone

Next Day Air Early AM    $131.75    104

Next Day Air  End of Day  $106.75    104

2nd Day Air  End of Day  $57.00    204

3 Day Select  End of Day  $43.00    304

Ground (commercial)  End of Day  $15.18    004

Note:

The Next Day Air Early A.M. service may not guarantee Early AM

arrival to 75801 United States.

>> Hello all,

>>

>>     I have 2 12KV 30mA NST's that need to be unpoted. One side

>> workd on each one, but I've checked the windings with a meter

>> & unpoting them should bring them back to their full life. One is

>> a france & the other looks like a france but has no tag on it.

>> If you want them email me & you pay for the shipping via

>> UPS.

>>

>> Choad

>> Member of The Oklahoma Tesla Coil Builders

>> http://www.geocities.com/SouthBeach/Pier/2756/index.htm

>Choad,

>

>I want them. I live in Texas. Area code is 75801.

>E-mail me with the details.

>

>Chris

Sender: tesl-err@emachine.com

Date: Sun, 13 Dec 1998 20:32:34 -0500

From: Tom Stock <tomstock@geocities.com>

Organization: #813

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST's for sale

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Corey Kent wrote:

> Tom Stock wrote:

>

> > I have 4 extra 15kv/30 ma Neons for sale...

> >

> > I'd like $20 each for them (+ shipping). If you're interested let me

> > know.

> >

> > cya

> > -tom

>

> you know how much the shipping would cost to australia?

Whoops....I should've mentioned that I'm in the US (Florida).

I'm not sure but probably around $50....I'll try to remember to call the

post office today.

-tom

Date: Mon, 14 Dec 1998 15:43:08 +1100

From: Corey Kent <corey@enterprise.net.au>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST's for sale

Sender: tesl-err@emachine.com

Tom Stock wrote:

> I have 4 extra 15kv/30 ma Neons for sale...

>

> I'd like $20 each for them (+ shipping). If you're interested let me

> know.

>

> cya

> -tom

you know how much the shipping would cost to australia?

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 14 Dec 1998 09:44:37 +0200

Subject: Re: Shocking Losses

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Just static electricity: same performance from my secondary too.

EdwMullin@aol.com on 12.12.98 04:35:45

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Shocking Losses

Also I noticed, after I had shut down for the evening, I was getting small

shocks from the top 3rd of the secondary,  there was no power to any part

of the circuit (mains plugs out) and I had a shunt installed across the

capacitor terminals.  Where did these shocks come from? It was repeatable,

even after I touched a grounded wire to the top terminal.

Date: Mon, 14 Dec 1998 12:51:02 -0500

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST's for sale

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Yes I have 2 identical. I can't remember what brand right off the top of

my head, but I'll weigh them

and give you a shipping estimate when I get home (or maybe sooner).

-tom

Gene Waldo wrote:

> Tom Stock wrote:

> >

> > I have 4 extra 15kv/30 ma Neons for sale...

> >

> > I'd like $20 each for them (+ shipping). If you're interested let me

> > know.

> >

> > cya

> > -tom

>

> Tom,

> I may want 2 of them if identical?  ship cost to 91342?

>

> --

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Mon, 14 Dec 1998 10:15:36 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST's for sale

Sender: tesl-err@emachine.com

Tom Stock wrote:

>

> I have 4 extra 15kv/30 ma Neons for sale...

>

> I'd like $20 each for them (+ shipping). If you're interested let me

> know.

>

> cya

> -tom

Tom,

I may want 2 of them if identical?  ship cost to 91342?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Mon, 14 Dec 1998 16:42:23 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Secondary wire size?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have a source where I can get a 10" diameter pvc pipe and I have a

question about the size of the wire I should use. This coil is going to

be ran by a 7.2kv potential transformer, it is going to be limited to

2kw. I am going to be using a .07ufd cap. I am hoping for about 6 foot

streamers or more. But by using such power what size of wire would be

best. I currently only have a spool of 22awg and a spool of 24awg magnet

wire, both are 8lb spools. I was wondering if 22awg wire will be able to

carry that kind of power. I was considering winding the coil and then

winding another layer over that and paralleling them. I know this will

reduce the resistance and thus allow a better power transfer from the

primary to the secondary. I am also wondering that if by winding a

double layer coil with 22awg wire, wouldn't the two paralleled wires act

as one single wire of a larger size. If so it should be able to

withstand the power I want. The only other downside to using 22awg is

going to be the amount of turns. I know for certain that the winding

will exceed the 1000 turn mark (~1150 turns). Is it absolutely necessary

to stay under a 1000 turns? Will it diminish any performance?

If anyone can answer these questions, please do. I need to know if this

can work and not compromise the coils performance.

Chris Jones

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 15 Dec 1998 12:45:18 +1200

Subject:       Re: Secondary wire size?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Re your proposed coil:

> Date sent:      Tue, 15 Dec 1998 11:42:23 +1300

> From:           mel wise <mwise44@flash.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Secondary wire size?

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I have a source where I can get a 10" diameter pvc pipe and I have a

> question about the size of the wire I should use. This coil is going to

> be ran by a 7.2kv potential transformer, it is going to be limited to

> 2kw. I am going to be using a .07ufd cap. I am hoping for about 6 foot

> streamers or more. But by using such power what size of wire would be

> best. I currently only have a spool of 22awg and a spool of 24awg magnet

> wire, both are 8lb spools. I was wondering if 22awg wire will be able to

> carry that kind of power. I was considering winding the coil and then

> winding another layer over that and paralleling them. I know this will

> reduce the resistance and thus allow a better power transfer from the

> primary to the secondary. I am also wondering that if by winding a

> double layer coil with 22awg wire, wouldn't the two paralleled wires act

> as one single wire of a larger size. If so it should be able to

> withstand the power I want. The only other downside to using 22awg is

> going to be the amount of turns. I know for certain that the winding

> will exceed the 1000 turn mark (~1150 turns). Is it absolutely necessary

> to stay under a 1000 turns? Will it diminish any performance?

If it is close wound, use the #22. If it is space wound, use the #24.

The major goal is to get low frequency running to enable a high L/C

ratio in the primary. I wou;dn't bother with double winding

personally. How high is the coil?

Malcolm

Date: Mon, 14 Dec 1998 19:38:21 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary wire size?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> If it is close wound, use the #22. If it is space wound, use the #24.

> The major goal is to get low frequency running to enable a high L/C

> ratio in the primary. I wou;dn't bother with double winding

> personally. How high is the coil?

>

> Malcolm

The coil deminsions are going to be 10.5"x31.5". Calculated turns are

around 1180. Calculated inductance is around 99.76 mH. Calculated

frequency is around 125.67 khz.

Chris Jones

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 15 Dec 1998 14:49:25 +1200

Subject:       Re: Secondary wire size?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Chris,

Sounds like an interesting coil.  What terminal are you

planning to stick on it?

Malcolm

> The coil deminsions are going to be 10.5"x31.5". Calculated turns are

> around 1180. Calculated inductance is around 99.76 mH. Calculated

> frequency is around 125.67 khz.

>

> Chris Jones

>

Date: Mon, 14 Dec 1998 20:47:08 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary wire size?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm Watts wrote:

>

> Hi Chris,

>          Sounds like an interesting coil.  What terminal are you

> planning to stick on it?

>

> Malcolm

>

> > The coil deminsions are going to be 10.5"x31.5". Calculated turns are

> > around 1180. Calculated inductance is around 99.76 mH. Calculated

> > frequency is around 125.67 khz.

> >

> > Chris Jones

> >

If I can find 6" flex duct, I will make one that is in the ballpark of 6"x32"

Date: Mon, 14 Dec 1998 21:14:38 -0600

From: "Paul R. Eitson" <xyme2@earthlink.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flywheel capacitor (was Re: Storage devices)

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>It's actually not a bad use of the term. A flywheel is the mechanical

> analogue of a capacitor.

>

> Malcolm

This is an interesting statement. Would it be possible to use a flywheel in

place of a capacitor?

Paul

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 15 Dec 1998 16:41:33 +1200

Subject:       Re: Flywheel capacitor (was Re: Storage devices)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> It's actually not a bad use of the term. A flywheel is the mechanical

> analogue of a capacitor.

> >

> > Malcolm

>

>   This is an interesting statement. Would it be possible to use a flywheel in

> place of a capacitor?

>          Paul

Not directly but I suppose if one had an arrangement of magnets and

coils attached to allow the energy to be withdrawn electrically. It

would have to be physically small to allow polarity reversals at TC

type frequencies.

Malcolm

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Transformer oil

Date: Mon, 14 Dec 1998 22:58:34 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I bought 5 gallons of xfmr oil and was wondering if any of you experienced

coilers could tell me what I have.

I haven't seen this particular oil mentioned in any of the capacitor

building instructions that I have come across.

The label reads:

Insulating oil, electric

ASTM D 3487-88

Type 1

9160-00-685-0913

Gard Corp.

Kansas City, KS

Jon Rosenstiel   jonr@pacbell.net

Date: Tue, 15 Dec 1998 06:34:48 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Transformer oil

Sender: tesl-err@emachine.com

Jon Rosenstiel wrote:

>

> Hi,

> I bought 5 gallons of xfmr oil and was wondering in any of you experienced

> coilers could tell me what I have.

> I haven't seen this particular oil mentioned in any of the capacitor

> building instructions that I have come across.

>

> The label reads:

> Insulating oil, electric

> ASTM D 3487-88

> Type 1

> 9160-00-685-0913

> Gard Corp.

> Kansas City, KS

>

> Jon Rosenstiel   jonr@pacbell.net

Jon,

You've got the right type of mineral oil! ASTM stands for the American

Society for Testing and Materials, and the specific set of tests that

this oil meets is defined in an ASTM specification. This specification

(D 3487-88 - "Standard Specification for Mineral Insulating Oil Used in

Electrical Apparatus") is used by all American manufacturers of mineral

oil based transformer oils, and is your assurance that your particular

oil meets a large number of various standardized tests (covering

viscosity, flash points, dielectric breakdown, dielectric constant and

loss factor, oxidation stability, etc, etc....) and is suitible for its

intended use.

Type I means that the oil contains no additives to inhibit oxidation.

Type II oils contain additives which help protect the oil under harsher

operating conditions, such as inside high power, hot-running

transformers. Either type will work superbly for home-brew capacitors!

Keep us posted on how your system is progressing!

-- Bert --

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Xformer

Date: Tue, 15 Dec 1998 11:59:19 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Greg, All,

The transformers have found a home & are gone.

Thanks,

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

>you wrote:

>I have 2 12KV 30mA NST's that need to be unpoted. One side

>workd on each one, but I've checked the windings with a meter

>& unpoting them should bring them back to their full life. One is

>a france & the other looks like a france but has no tag on it.

>If you want them email me & you pay for the shipping via UPS.

>

Date: Tue, 15 Dec 1998 18:05:36 -0600

From: Mark Olson <molson1@isd.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: disk type spark gap?

Sender: tesl-err@emachine.com

I have 12 4" aluminum disks from a couple of old disk drives that I took

apart.  I've been

thinking about making a spark gap out of them, and I'm wondering what

people think

of this idea:

xxxxxxxxxxxxx                            1st disk

xxxxxxxxxxxx                 2nd disk

xxxxxxxxxxxxx                            3rd disk

xxxxxxxxxxxx                 4th disk

xxxxxxxxxxxxx

...

xxxxxxxxxxxx               12th disk

This is intended to show that there would be two interlaced stacks

of disks, separated by insulating spacers and placed on spindles, in

effect a rotating series gap. The edges are rounded so I would hope

that the discharge would not primarily occur at the edges of the disks.

The disks  on the outside would be connected to a brush of some sort

that could handle the discharge current (brass rod/sleeve?), and would

counterrotate (1 motor on each spindle).   If brushes are a problem I

suppose I could place thick electrodes at either end that could

withstand

the heat.

Because air could be blown through the stacks at the overlap point

(and elsewhere), the rotating disks would be easy to cool and should

withstand high power.  Since the disks are precision made they are

already balanced for ~5400 RPM, though I expect I wouldn't have

to spin them anywhere near that fast.   I would also think that the air

spun off by the disks would aid in quenching.

I realize this would look pretty cool, but would it really be that much

more effective than the venerable series cylindrical gap?  I'm hoping for

something that could be used for higher power operation without the complexity

of a synchronous rotary gap...

From: McFalc0n@aol.com

Date: Wed, 16 Dec 1998 17:43:56 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: disk type spark gap?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

sounds like a great idea.  I have taken HD's apart a lot and I might try this.

Instead of brushes for a connection, you could use a bolt gap with a heatsink

or a rotating disk like a rotary spark gap.  sounds like a neat idea.  Hope

that helps.

From: "steve" <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Power Controller

Date: Thu, 17 Dec 1998 07:55:39 -0000

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

Just a short message to say that I've finished making the control unit for

my coil (details at http://www.breakfix.demon.co.uk) and I picked up some

enameled copper wire, the last main component for the project.  There's

nothing left now but hard work!

Seasonal greetings to all!   :-)

Steve Bell   (in the UK)

P.S.  Where is the point on the globe exactly opposite to Iraq, and does

anyone have a large enough TC they can tune to earth resonance? Power up

Sadams earth and fry his computers etc!! Worth a try!

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flat plate caps

Date: Thu, 17 Dec 1998 18:08:42 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A few questions on constructing flat plate capacitors.

1. Lets say that you have a container with 8....16 plate caps wired in

series, how much distance is required between the adjacent caps that make up

the complete unit? (Cap to cap spacing).  If the dielectric of the cap is 20

mils, is 60 mils enough distance between each of the 16 units? Will there be

any detrimental interaction between the units if the spacing is too close?

2. I read somewhere that one shouldn't have a uneven number of metal plates

in a capacitor. I really don't see that this would make any difference.

Anyone care to comment?

3. How difficult would it be to construct a Plexiglas tank to hold a

capacitor? Can someone tell me where to find some tips on such a project?

Thanks,

Jon Rosenstiel   jonr@pacbell.net

From: "steve" <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat plate caps

Date: Fri, 18 Dec 1998 08:22:42 -0000

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Jon Rosenstiel <jonr@pacbell.net>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Flat plate caps

> Date: 18 December 1998 02:08

>

> A few questions on constructing flat plate capacitors.

<snip>

> 3. How difficult would it be to construct a Plexiglas tank to hold a

> capacitor? Can someone tell me where to find some tips on such a project?

>

I have made an acrylic tank for the stack plate cap I intend to build.  It

was not cheap!  I used 6mm sheet, which was the most common size, and some

fancy 2-part adhesive called 'Tensol 70'.

I wanted to be able to see the cap in operation and watch for any corona

problems etc..

If I were to build another one, it would not be the same.

Steve Bell (in the UK)

From: EdwMullin@aol.com

Date: Fri, 18 Dec 1998 12:24:28 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat plate caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/17/98 9:08:10 PM Eastern Standard Time, jonr@pacbell.net

writes:

<< A few questions on constructing flat plate capacitors.

1. Lets say that you have a container with 8....16 plate caps wired in

series, how much distance is required between the adjacent caps that make up

the complete unit? (Cap to cap spacing).  If the dielectric of the cap is 20

mils, is 60 mils enough distance between each of the 16 units? Will there be

any detrimental interaction between the units if the spacing is too close?

2. I read somewhere that one shouldn't have a uneven number of metal plates

in a capacitor. I really don't see that this would make any difference.

Anyone care to comment?

3. How difficult would it be to construct a Plexiglas tank to hold a

capacitor? Can someone tell me where to find some tips on such a project?

>>

1. In the cap that I built, I separated each segment with panes of 1/8th inch

thick glass.  I haven't noticed any bad side effects, except my cap value was

larger than planned, but I'm not sure that was caused by the glass.

3. I used a clear plastic container that I epoxied a sheet of plexiglass to it

was stronger than the original cover, to hold the bus connections.

Good luck!

Date: Fri, 18 Dec 1998 18:08:12 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat plate caps

Sender: tesl-err@emachine.com

steve wrote:

>

> ----------

> > From: Jon Rosenstiel <jonr@pacbell.net>

> > To: Tesla-2 Mailing List <tesla-2@emachine.com>

> > Subject: Flat plate caps

> > Date: 18 December 1998 02:08

> >

> > A few questions on constructing flat plate capacitors.

>

> <snip>

>

> > 3. How difficult would it be to construct a Plexiglas tank to hold a

> > capacitor? Can someone tell me where to find some tips on such a project?

> >

> I have made an acrylic tank for the stack plate cap I intend to build.  It

> was not cheap!  I used 6mm sheet, which was the most common size, and some

> fancy 2-part adhesive called 'Tensol 70'.

>

> I wanted to be able to see the cap in operation and watch for any corona

> problems etc..

>

> If I were to build another one, it would not be the same.

>

> Steve Bell (in the UK)

I built one with 1/4" Lexan (Plexiglass) 4" x 12" x 16" and my next one

will not use the same material for three reasons:

1. Not optimum thickness, needs to be 33% thicker at a minimum to

prevent bowing and having enough thickness to drive screws in

2. The mating surfaces must be very flat and smooth to achieve a

leakless joint. I followed up the acrylic glue with RTV, which was a

pain, but it worked.

3. I no longer need to see where the arcing is.  With the clear cap, I

sure did learn where corona forms and where the danger spots are.

So I'm saying, go ahead and build one if you want to learn the same

things for yourself.  My next will be 3/4" wood, sealed with

polyurethane as Mike Hammer suggests. Much easier materials to work with.

If you do plan to use 1/4", because of flexure you will need a top screw

every inch or two around the perimeter, and THAT is a pain. You  could

glue on supporting ribs, but then you'll have a lot of time invested.

If you just gotta have clear, then use thick material for the sides.

(editors note. polyurethane was not recommended for sealing cap tanks

made from wood. polyester resin of the type used for repair of fiberglass

cars and boats was the material I used and recommended)

Date: Fri, 18 Dec 1998 19:48:51 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Circuit Breakers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone know exactly what amperage a circuit breaker is designed to

trip at. The line I use to run my coil is on a 20A breaker. Will my

breaker trip right at 20A or will it run over this limit.

The reason I ask is because I have a potential transformer I would like

to run at about 20A. Originally I wanted to run it at 18A, but I am

having a hard time finding a way to get it to 18A. On the other hand I

can limit it exactly to 20A using a small welder I have, but I am afraid

that it will trip my breaker.

Chris Jones

From: <corr@enid.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Circuit Breakers

Date: Fri, 18 Dec 1998 20:49:13 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Circuit breakers rated at any given amount seldom break AT that amount.

For example, my coil (using four 15kv neons in parrallel) pulls in excess of 20A

by far, usually between 26A-30A.  However, if I run two microwave trannies in

series and draw an arc, I easily blow the breaker.

I would betcha money your 20A breaker will run your PT fine - and if not, a 30A

breaker won't cost that much at any hardware store.  You could wire from the main

150A-200A breaker and wire a dedicated 30A breaker for your PT.  This would be

the safest way to run it, because of you replace a 20A breaker with a 30A breaker,

there is a good chance the wiring on that circuit was only meant for 20A.  If you wire

a custom circuit, you can wire it for 30A easily.

I 30A breaker at Lowes here in Oklahoma costs $20-$50, while a roll of #10 (good for

25-30 amps) with 500ft on the spool is only $12.

Jeff Corr

Oklahoma Tesla Coil Builders Assoc.

corr@enid.com

>Does anyone know exactly what amperage a circuit breaker is designed to

>trip at. The line I use to run my coil is on a 20A breaker. Will my

>breaker trip right at 20A or will it run over this limit.

>The reason I ask is because I have a potential transformer I would like

>to run at about 20A. Originally I wanted to run it at 18A, but I am

>having a hard time finding a way to get it to 18A. On the other hand I

>can limit it exactly to 20A using a small welder I have, but I am afraid

>that it will trip my breaker.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R. E. Circuit Breakers

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Sat, 19 Dec 1998 01:16:52 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

When circuit breakers trip is not exact. It mostly depends on what brand,

age, type or style they are. Many older breakers are purely thermal,

now they are almost all magnetic. The thermals will really hang in there!

Federal, M-blocks, Zinsco, old square-D Will hold as much as double for

5-10 minutes! Push-a-matic, (Bull Dog), Heinemen, new Square - D, or

Siemiens will trip immediately. Breakers with HACR ratings should take

momentary overloads. (heating and air conditioning rated)

If you're looking for any old breakers contact me, I've got quite a few.

If it's not dangerous it's not fun! Kyle  break7@juno.com

X-Mailer: IPAD 2.0

Date: Sat, 19 Dec 1998 11:34:49 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat plate caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:08 PM 12/18/98 -0600, you wrote:

>steve wrote:

>>

>> ----------

>> > From: Jon Rosenstiel <jonr@pacbell.net>

>> > To: Tesla-2 Mailing List <tesla-2@emachine.com>

>> > Subject: Flat plate caps

>> > Date: 18 December 1998 02:08

>> >

>> > A few questions on constructing flat plate capacitors.

>>

>> <snip>

>>

>> > 3. How difficult would it be to construct a Plexiglas tank to hold a

>> > capacitor? Can someone tell me where to find some tips on such a project?

>> >

>> I have made an acrylic tank for the stack plate cap I intend to build.  It

>> was not cheap!  I used 6mm sheet, which was the most common size, and some

>> fancy 2-part adhesive called 'Tensol 70'.

>>

>> I wanted to be able to see the cap in operation and watch for any corona

>> problems etc..

>>

>> If I were to build another one, it would not be the same.

>>

>> Steve Bell (in the UK)

>I built one with 1/4" Lexan (Plexiglass) 4" x 12" x 16" and my next one

>will not use the same material for three reasons:

>

>	1. Not optimum thickness, needs to be 33% thicker at a minimum to

>prevent bowing and having enough thickness to drive screws in

>

>	2. The mating surfaces must be very flat and smooth to achive a

>leakless joint. I followed up the acrylic glue with RTV, which was a

>pain, but it worked.

>

>	3. I no longer need to see where the arcing is.  Whith the clear cap, I

>sure did learn where corona forms and where the danger spots are.

>

>So I'm saying, go ahead and build one if you want to learn the same

>things for yourself.  My next will be 3/4" wood, sealed with

>polyurethane as Mike Hammer suggests. Much easier materials to work

>with.

The cap tank I built and one like it for my rebult 12kv/100ma

neon were made from 1/2 inch plywood with two coats of

polyester resin on the inside to leakproof it. The polyester

resin is the same material sold for fiberglass repairs.

Others have used epoxy resin in the same fashion but its much

more expensive.

My neon tank has had no leaks over a 8 month period and the

cap tank is over 1 year old. There is a picture of my neon

tank on my web page at: http://www.misslink.net/mhammer/rebuild.htm

I made an earlier flat plate cap tank from plywood and lined it with

sheets of plexiglass. The seams were sealed with beads of RTV.

This worked quite well but was much more expensive than coating

the tank with polyester resin.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 19 Dec 1998 11:47:54 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat plate caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:08 PM 12/17/98 -0800, you wrote:

>A few questions on constructing flat plate capacitors.

>

>1. Lets say that you have a container with 8....16 plate caps wired in

>series, how much distance is required between the adjacent caps that make up

>the complete unit? (Cap to cap spacing).  If the dielectric of the cap is 20

>mils, is 60 mils enough distance between each of the 16 units? Will there be

>any detrimental interaction between the units if the spacing is too close?

>

With several caps wired in series the voltage across each individual

cap would be low so the insulation requirements would also be low.

With 16 caps in series 60 mils would surely be more than plenty.

On my 6 unit plate cap I used sheets of 3/32 plexiglass to sandwich

each cap. So there are two sheets of plexiglass between each cap.

>2. I read somewhere that one shouldn't have a uneven number of metal plates

>in a capacitor. I really don't see that this would make any difference.

>Anyone care to comment?

Consider exactly what a capacitor is. Its TWO conductors separated

by a dielectric. Now consider what a plate cap is. Its many of these

caps all connected in parallel. If have an odd number of plates this

extra plate has no matching plate to form a capacitor.

>

>3. How difficult would it be to construct a Plexiglas tank to hold a

>capacitor? Can someone tell me where to find some tips on such a project?

Plexiglass is brittle and will crack easily and would need

to be rather thick to be rigid enough. Very expensive. Lexan

would be a better choice but its nearly as expensive.

You can make a tank from plywood then line it with thin

plexiglass. Attach the plexiglass sheets to the interior

of the tank walls using RTV and fill the seams with a heavy

bead of RTV.

Or dispense with the plexiglass altogether and paint the interior

of the plywood tank with epoxy or polyester resin. I used two

coats of polyester resin on my last two tanks. Worked very well.

No leaks.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Scott_L_Hanson@notes.seagate.com

X-Lotus-FromDomain: SEAGATE@INTERNET

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 19 Dec 1998 11:23:29 -0800

Subject: Re: Flat plate caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I'm not sure I follow the odd-plate explanation....

Consider a simple flat-plate with two plates. Each conductive plate has two

faces, but only two faces are "utilized" in a two-plate capacitor (2 of 4

faces). Now add a third plate. The cap now has TWICE the capacitance of a

2-plate cap of the same plate size. Both faces of the central conductive

plate are "utilized" (4 of 6 faces).  A flat-plate cap with an odd number

of plates is perfectly feasible.

To:   "Tesla-2 Mailing List" <tesla-2@emachine.com>

cc:    (bcc: Scott L Hanson)

Subject:  Re: Flat plate caps

>2. I read somewhere that one shouldn't have a uneven number of metal plates

>in a capacitor. I really don't see that this would make any difference.

>Anyone care to comment?

Consider exactly what a capacitor is. Its TWO conductors separated

by a dielectric. Now consider what a plate cap is. Its many of these

caps all connected in parallel. If have an odd number of plates this

extra plate has no matching plate to form a capacitor.

X-Mailer: IPAD 2.0

Date: Sat, 19 Dec 1998 13:49:10 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat plate caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:23 AM 12/19/98 -0800, you wrote:

>I'm not sure I follow the odd-plate explanation....

>

>Consider a simple flat-plate with two plates. Each conductive plate has two

>faces, but only two faces are "utilized" in a two-plate capacitor (2 of 4

>faces). Now add a third plate. The cap now has TWICE the capacitance of a

>2-plate cap of the same plate size. Both faces of the central conductive

>plate are "utilized" (4 of 6 faces).  A flat-plate cap with an odd number

>of plates is perfectly feasible.

>

>

Absolutely true. But now with 3 plates does not the

cap as a whole have unequal plate areas on each side

of the dielectric? Adding a fourth plate will equalize

the plate areas. Im sure a cap with unequal plate areas

will work but it just doesnt seem right to me.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "steve" <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat plate caps

Date: Sun, 20 Dec 1998 00:04:58 -0000

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Mike Hammer <mhammer@misslink.net>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Re: Flat plate caps

> Date: 19 December 1998 19:49

>

> At 11:23 AM 12/19/98 -0800, you wrote:

> >I'm not sure I follow the odd-plate explanation....

> >

> >Consider a simple flat-plate with two plates. Each conductive plate has two

> >faces, but only two faces are "utilized" in a two-plate capacitor (2 of 4

> >faces). Now add a third plate. The cap now has TWICE the capacitance of a

> >2-plate cap of the same plate size. Both faces of the central conductive

> >plate are "utilized" (4 of 6 faces).  A flat-plate cap with an odd number

> >of plates is perfectly feasible.

> >

> >

> Absolutely true. But now with 3 plates does not the

> cap as a whole have unequal plate areas on each side

> of the dielectric? Adding a fourth plate will equalize

> the plate areas. Im sure a cap with unequal plate areas

> will work but it just doesnt seem right to me.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

The only plate area to consider when talking about a cap is the shared

overlap, which can only be equal. If you say it is unequal then you are

counting plate area not in use!

Steve Bell (in the UK)

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: A DC Coil

Date: Sun, 20 Dec 1998 03:05:32 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Hi everybody,

I was doing somethinking about a DC coil & made this diagram.

http://www.geocities.com/SouthBeach/Pier/2756/DCTeslaCoilDiagram.gif

I think I'm on the right track, but could use a little help. I think

I remember hearing theat the filter cap should be either 10 or 100

times bigger then the tank cap. Also I need to know what I should do

for a gap. I thought that a small rotory gap running at a low bps

would work. I think a static gap should work too if I switched the

gap & cap around too. I plan on using either a 12KV 60mA NST or

making a voltage doubler with some MOT's. The diodes will me either

2-3 microwave diodes in series or someother diodes that I can get.

If anyone knows where I could get some good HV diodes for this let

me know. If I need to make a filter cap for this how would I go

about making such a large value HV one? I think that a bank of

microwave caps would work in a series parallel config, but that

would take a whole lot of them as it'd have to be rated at 20Kv or

so.

Thanks,

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A DC Coil

Date: Sun, 20 Dec 1998 03:53:29 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

The link should be

http://www.geocities.com/SouthBeach/Pier/2756/dcteslacoildiagram.gif

not

http://www.geocities.com/SouthBeach/Pier/2756/DCTeslaCoilDiagram.gif

The geocities server is case sensitive. Sorry about that.

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

X-Originating-IP: [216.68.37.212]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: high freq high voltage 60kva power converter specs

Date: Sun, 20 Dec 1998 07:46:25 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Guys,

I just uploaded 5 pages of stuff on a neat high frequency power

converter used to power x-ray equipment.

The information may be found on my page at:

http://www.geocities.com/CapeCanaveral/Hangar/6160/hvtank/hvtank.html

I don't have that subpage linked in from anywhere else - so if you

forget this link you'll never find it! :)

Later.

-Bill

X-Mailer: IPAD 2.0

Date: Sun, 20 Dec 1998 11:58:02 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat plate caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:04 AM 12/20/98 -0000, you wrote:

>

>

>----------

>> From: Mike Hammer <mhammer@misslink.net>

>> To: Tesla-2 Mailing List <tesla-2@emachine.com>

>> Subject: Re: Flat plate caps

>> Date: 19 December 1998 19:49

>>

>> At 11:23 AM 12/19/98 -0800, you wrote:

>> >I'm not sure I follow the odd-plate explanation....

>> >

>> >Consider a simple flat-plate with two plates. Each conductive plate has two

>> >faces, but only two faces are "utilized" in a two-plate capacitor (2 of 4

>> >faces). Now add a third plate. The cap now has TWICE the capacitance of a

>> >2-plate cap of the same plate size. Both faces of the central conductive

>> >plate are "utilized" (4 of 6 faces).  A flat-plate cap with an odd number

>> >of plates is perfectly feasible.

>> >

>> >

>> Absolutely true. But now with 3 plates does not the

>> cap as a whole have unequal plate areas on each side

>> of the dielectric? Adding a fourth plate will equalize

>> the plate areas. Im sure a cap with unequal plate areas

>> will work but it just doesnt seem right to me.

>>

>

>The only plate area to consider when talking about a cap is the shared

>overlap, which can only be equal. If you say it is unequal then you are

>counting plate area not in use!

>

>Steve Bell (in the UK)

Steve and all.

Well now that I sit down and really think about this (unusual for me)

I can see that your are correct. Thats what I get for running off

the mouth without thinking the problem through.

So it would seem an odd number of plates is just as valid

as an even number and the old admonishments against odd numbers

were just plain wrong.

I stand corrected.

TTYL!

Mike Hammer

mhammer@misslink.net

From: NebStrebor@aol.com

Date: Sun, 20 Dec 1998 19:20:42 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: plasma globe power supply

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I had a question on the 555 timer circuit......  I built it and fired it

up. Good and bad.  First of all it works, and produces a high pitched humming

sound when running, as for the bad news, the output is less than expected.

The only problems I can think of are that I am using a 2 amp transformer

instead of 3. Would this make such a significant difference?  (my sparks are 3-4 mm

long and very spindly)

Any input is welcome,  Thanks in advance, NEB

Date: Sun, 20 Dec 1998 19:13:12 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: plasma globe power supply

Sender: tesl-err@emachine.com

NebStrebor@aol.com wrote:

>

> I had a question on the 555 timer circuit......  I built it and fired it

> up. Good and bad.  First of all it works, and produces a high pitched humming

> sound when running, as for the bad news, the output is less than expected.

> The only problems I can think of are that I am using a 2 amp transformer

> instead of 3. Would this make such a significant difference?  (my sparks are 3-4

> mm long and very spindly)

> Any input is welcome,  Thanks in advance, NEB

The reason your supply is humming is because your freq. is too low.

You are operating somewhere in audio range. You will have to decrease

the value of Ra or Rb. Also you may have decrease the value of timing

capacitor C. Da and Db will allow you to change your duty cycle from a

very low or very high value. The pic I am attaching have very high

values. Don't use these values! Your freq. will be far to low. Hope this helps.

What supply voltage are you using on the primary of your flyback

transformer?

Frankensteins Helper

Max

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\555.jpg"

(editors note. like I said. no binary attachments in archive.)

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 21 Dec 1998 09:37:11 +0200

Subject: Re: A DC Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

You can check my WWW page: I have built a TC with 2 MOTs and voltage

doubler. That is a "pulsed DC" tank supply. I am currently designing a

"real" DC tank supply but that is going to be quite expensive and big so I

don't recommend in to anybody (until you have plenty of room and money).

See http://www.saunalahti.fi/dncmrc/mytesla.html

BTW, I couldn't reach your DCTeslaCoilDiagram.gif: where is that?

choad69@geocities.com on 20.12.98 11:05:32

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  A DC Coil

Hi everybody,

I was doing somethinking about a DC coil & made this diagram.

http://www.geocities.com/SouthBeach/Pier/2756/DCTeslaCoilDiagram.gif

I think I'm on the right track, but could use a little help. I think

I remember hearing theat the filter cap should be either 10 or 100

times bigger then the tank cap. Also I need to know what I should do

for a gap. I thought that a small rotory gap running at a low bps

would work. I think a static gap should work too if I switched the

gap & cap around too. I plan on using either a 12KV 60mA NST or

making a voltage doubler with some MOT's. The diodes will me either

2-3 microwave diodes in series or someother diodes that I can get.

If anyone knows where I could get some good HV diodes for this let

me know. If I need to make a filter cap for this how would I go

about making such a large value HV one? I think that a bank of

microwave caps would work in a series parallel config, but that

would take a whole lot of them as it'd have to be rated at 20Kv or

so.

Thanks,

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 21 Dec 1998 10:08:06 +0200

Subject: Re: A DC Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Now I can see your schematics.

You won't win anything with that DC design until you manage to keep your DC

voltage up: that means a pole pig transformer, a huge capacitor (10 - 30

uF), a current limiting inductor on the DC side (1 - 3 Henry), high current

diodes (1 - 5 A), a fast rotating spark gap, possibly 3-phase AC feed for

your pig, etc. Then you could say you are using DC.

If you are not ready for all this your "DC" will float up and down at each

spark gap bang making it no more useful than a standard NST design with no

expensive diodes and no stabilizing capacitor.

Sorry about the cold shower :).

From: "steve" <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: DF for polyurethane varnish

Date: Mon, 21 Dec 1998 23:47:33 -0000

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

Does anyone know a value for dissipation factor for polyurethane varnish,

at say 1MHz?

TIA.

Steve Bell (in the UK)

From: NebStrebor@aol.com

Date: Mon, 21 Dec 1998 22:26:42 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: plasma globe power supply

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am using a 12 v power supply and my timer cap is .01uf at 1000v

Date: Mon, 21 Dec 1998 22:08:09 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: plasma globe power supply

Sender: tesl-err@emachine.com

NebStrebor@aol.com wrote:

>

> I am using a 12 v power supply and my timer cap is .01uf at 1000v

I would decrease the value of the cap to .001uf for starters. Also the

voltage rating is too high. Try a polystyrene capacitor rated at 50

volts. You are probably using a ceramic cap which has a tendancy to be a

bit unstable with temprature change and will cause the frequency to

drift. For plasma globe operation I have found that I get the best

results when the freq. is around 20Khz. This is right at the upper limit

of human hearing. Duty cycle should be around 80% at the output of the

555. Of course you can run at any duty cycle you desire to vary the

"texture" of the plasma by changing the values of Ra and Rb.

You can also alter the plasma by application of a signal to pin 5.

This will be my next project!

Frankensteins Helper

Max

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A DC Coil

Date: Tue, 22 Dec 1998 03:26:08 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Marco,

What kind of diodes & caps do you use? I currently only have

microwave parts that will handle these voltage levels. I have 9 MOT's, 6

microwave diodes & several microwave caps. I drew another pic and it's at

http://www.geocities.com/SouthBeach/Pier/2756/voltagedoubler.gif

This is of the voltage doubler I plan on using for the time being.

Thanks,

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

>Now I can see your schematics.

>You won't win anything with that DC design until you manage to keep your DC

>voltage up: that means a pole pig transformer, a huge capacitor (10 - 30

>uF), a current limiting inductor on the DC side (1 - 3 Henry), high current

>diodes (1 - 5 A), a fast rotating spark gap, possibly 3-phase AC feed for

>your pig, etc. Then you could say you are using DC.

>If you are not ready for all this your "DC" will float up and down at each

>spark gap bang making it no more useful than a standard NST design with no

>expensive diodes and no stabilizing capacitor.

>Sorry about the cold shower :).

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 22 Dec 1998 12:16:08 +0200

Subject: Re: A DC Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I use capacitors and diodes from usual microwave ovens.

Again, look at my WWW page: there you'll find schematics of my voltage

doubler. If you use my scheme you need only 2 caps and 2 diodes (not 6 caps

and 4 diodes as you use). Or you can use 4 diodes for more protection (as I

did).

http://www.saunalahti.fi/dncmrc/tank.html

Good Luck!

choad69@geocities.com on 22.12.98 11:26:08

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: A DC Coil

Marco,

What kind of diodes & caps do you use? I currently only have microwave

parts that will handle these voltage levels. I have 9 MOT's, 6 microwave diodes

& several microwave caps. I drew another pic and it's at

http://www.geocities.com/SouthBeach/Pier/2756/voltagedoubler.gif

This is of the voltage doubler I plan on using for the time being.

Thanks,

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A DC Coil

Date: Tue, 22 Dec 1998 16:23:17 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Marco,

On your page you say you get 6.5KvDC from your supply. By just

using a bridge & some caps I could get 6Kv from 2 MOT's in series. I

don't see any doubling from the circuit.I just see AC being

rectified. I'm assuming that your MOT's are giving you 2.3Kv each.

By using the circuit I drew I could get a much desired higher

voltage. Right now I'm thinkg I may go with the bridge as it will

provide a better DC level tho.

Choad

From: EdwMullin@aol.com

Date: Tue, 22 Dec 1998 17:52:02 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A DC Coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Question about these Voltage doubler circuits, how much current can they put out?

It seems that if two MOT's are putting out ~8kv DC through the doubler then

they would have to have a high current to get a decent power supply for tesla

use. Is this correct? or is there some other advantage to using a DC Supply?

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 23 Dec 1998 07:39:18 +0200

Subject: Re: A DC Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

choad69@geocities.com on 23.12.98 00:23:17

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: A DC Coil

>Marco,

>    On your page you say you get 6.5KvDC from your supply. By just

>using a bridge & some caps I could get 6Kv from 2 MOT's in series. I

>don't see and doubleing from the circuit.I just see AC being

>rectified. I'm assuming that your MOT's are giving you 2.3Kv each.

>By using the circuit I drew I could get a much desired higher

>voltage. Right now I'm thinkg I may go with the bridge as it will

>provide a better DC level tho.

>Choad

Sorry, but my scheme IS A VOLTAGE DOUBLER. The scheme is not even mine, it

is used in every microwave oven to get 4 kV from one usual 2 kV MOT. By

connecting two MOT/diode/cap doubler with ground as the center tap you can

get (as I did) 8 kV pulsed DC. If you instead invert the phase of one of

the two MOTs you'll get 4kV DC (yes, REAL DC).

If you just put 2 MOTs secondary in series you will stress them to 4 kV AC,

while they are planned to withstand only 2 kV! Some efforts, some fail...

I say I get 6.5 kV because I have got also a third MOT in series with the

primaries (as a current limiter) that actually also makes some 20% voltage

on the 220V side. That's why.

X-Lotus-Fromdomain: MARTIS

From: "Marco Denicolai" <Marco.Denicolai@tellabs.fi>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 23 Dec 1998 07:45:43 +0200

Subject: Re: A DC Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

EdwMullin@aol.com on 23.12.98 00:52:02

To:   tesla-2@emachine.com

cc:    (bcc: Marco Denicolai/MARTIS)

Subject:  Re: A DC Coil

>Question about these Voltage doubler circuits, how much current can

>they put out?

I haven't yet had a chance of measuring (or calculating) it.

>It seems that if two MOT's are putting out ~8kv DC through the doubler then

>they would have to have a high current to get a decent power supply for tesla

>use. Is this correct? or is there some other advantage to using a DC Supply?

MOTs do put out high current: from 250 to 500 mA. And doubling capacitors

20 times bigger than the primary capacitor (1 uF to 0.05 uF) ensure that

they have some charge left also after charging the primary cap.

In my design the target was NOT to use DC (which is, anyway, PULSED in my

tank supply): the target was to have a robust alternative to weak NSTs. My

tank supply witstands well RF and current peaks: I never broke a MOT, nor a

doubler capacitor. I broke twice the diodes but never after using two in series.

Date: Tue, 22 Dec 1998 22:22:43 -0800

From: Cliff Blackburn <repairco@axionet.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Hey

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was Just woundering what is the best frequency to run a flyback

transformer at would be.

Date: Wed, 23 Dec 1998 00:59:46 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Plasma Globe

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I know this doesn't relate to tesla coils, but I need some help on

something. I have a plasma globe that I bought awhile back. It has an

8" diameter globe with a black base. When I bought the plasma globe it

emitted lots of purplish pink streamers everywhere. Now on the other

hand it doesn't seem to look as good as it once did before. I don't know

what the problem with it is. It still emits streamers but there are less

of them and they don't seem to be as bright in color.

The ciruit is a 555 type circuit. The flyback transformer is a fairly

small one. Here are my questions. First, could it be the flyback going

bad? Second, could it be that the circuitry is degrading in performance.

Third, the circuit has a 5k ohm trimmer in it, could this be my problem.

It has been adjusted to get peak performance in the globe.

If anyone might know what the problem is please reply to me off list.

Thanks

Chris

Date: Wed, 23 Dec 1998 12:18:13 +0100

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Hey

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cliff Blackburn wrote:

>

> I was Just woundering what is the best frequency to run a flyback

> transformer at would be.

Depends on the flyback. I guess the best frequency is achieved using

the circuit with the two 2N3055's. With this circuit I get much longer

sparks than with any other design.

Bye,

Stefan

Date: Wed, 23 Dec 1998 11:00:14 -0600

From: mel wise <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Items For sale

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have gotten my forsale page up and running. Prices on some items are

negotiatable.

http://www.geocities.com/SiliconValley/Haven/3841

Enjoy,

Chris

From: Scott_L_Hanson@notes.seagate.com

X-Lotus-FromDomain: SEAGATE@INTERNET

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 23 Dec 1998 13:37:03 -0800

Subject: Re: Items For sale

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello -

Very interested in the 12kV/90 neon and the potential XFMR.

Where are you located? Economics of the transaction depend on the shipping

charges.

Please call me ASAP at 714-751-4858 (Costa Mesa, southern Caslifornia) or

E-mail to sierrapk@net999.com

Thanks!!!

Scott Hanson

mel wise <mwise44@flash.net> on 12/23/98 09:00:14 AM

To:   "Tesla-2 Mailing List" <tesla-2@emachine.com>

cc:    (bcc: Scott L Hanson)

Subject:  Items Forsale

I have gotten my forsale page up and running. Prices on some items are

negotiatable.

http://www.geocities.com/SiliconValley/Haven/3841

Enjoy,

Chris

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Christmas Coil Saga

Date: Wed, 23 Dec 1998 19:28:52 -0500

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A Christmas Story for Coilers

by Fr. Tom McGahee

The story begins innocently enough...

Meanwhile, out in the garage,Bill the ********** was

busily lashing together his latest concoction: In

keeping with the Christmas Spirit, Bill was building

a rather large Christmas Decoration for the front

yard. To the uninitiated it looked simply like a large

aluminum Christmas tree mounted on a pedestal of some

sort. In reality, the aluminum Christmas tree was

merely the Topload on Bill's latest Tesla Coil. Bill

knew full well that the geometry of the tree did not

really lend itself well to being an EFFICICIENT

topload. But that was all-right. Bill was more

interested in shock value than he was in efficiency.

Actually, the pedestal was the "extra" coil of Bill's

experimental Magnifier Tesla Coil. According to his

calculations the "extra" coil should be capable

of throwing arcs at least 6 feet long.

He chuckled to himself as he struggled to move the

"extra" coil and Christmas tree topload out of the

garage and onto the front lawn. His wife was currently

curled up in front of the fireplace with a book and two

cats. Bill hated cats, but for the sake of his wife

he tolerated their presence in his home. Bill strapped

the coil/tree assembly to the top of the little

American Flyer sled, and wrestled it into position on

the front lawn. The final position was about 7 feet

away from the shoveled walkway that led from the

street to his front door. Bill always used the side

door, but he had In-Laws coming over tonight, and

*they* always used the Front Door. He figured the

6 foot arcs would be hurled mostly upward, and there

would be no chance of the arcs actually coming any

where near the In-Laws as they came down the little

shoveled path that he had prepared.

He carefully positioned a couple of dozen old

polyethylene buckets to hold the transmission

line, which was a couple of lengths of 1/2 inch

copper tubing that he had brazed together. He

giggled to himself as he fastened the transmission

line in place and attached it to the base of the

"extra" coil. Then he attached the other end of the

transmission line to the anti-corona ring of his

secondary coil.

The secondary coil consisted of a single layer of

high voltage wire wrapped in a solonoid fashion

around a fair-sized plastic garbage can. This in turn

was nestled inside a piece of slightly damp cardboard

sonotube that was several inches larger in diameter

than the garbage can. The primary was several turns of

heavy battery cable that Bill had "borrowed" from work.

The high voltage capacitors were homebrew poly caps

in sections of PVC pipe and under mineral oil

(actually an off-brand of cattle laxative that he

had seen mentioned by Gary Weaver on the Tesla List.)

Not being able to find any decent clear poly, Bill

had managed to scrape up a mixture of "construction"

grade poly (that had lots of interesting things

imbedded in it), and some smooth and shiny BLACK

poly that he thought might work well, despite his

fleeting concern that it might contain large

quantities of carbon in the form of lampblack. What

the heck, what did he have to lose by trying???

Dozens of these capacitors were wired in series -

parallel to achieve the required voltage and

capacitance rating. Just a few days before, he had

discovered a mistake in his calculations and had

realized that he still needed more capacitance.

Not wanting to spend any more money and time on poly

caps, he decided to revert to making Beer Bottle

Caps. He had tried making Soda Bottle Caps once, but

they were not nearly so much fun to make as Beer

Bottle Caps.

Bill had invited a couple dozen of his closest friends

over for a Bring Your Own Booze - Beer Bash. His wife

finally busted up the party at 4 AM Sunday morning.

Bill emptied out whatever beer still remained

in the bottles (I leave it to your own imagination

to figure out just how he did this). After many many

trips to the bathroom he finally had all the

empty beer bottles he needed. There were a fair

number of aluminum cans as well. These he put

in the plastic garbage bag in the kitchen.

While his wife wasn't looking he stole all the

aluminum foil and corn oil and salt that he could

find and proceeded to build a couple of tub-fulls

of beer bottle caps, which he then wired into the

existing capacitor grid using hundreds of little

Radio Shack clip leads.

The spark gap was an old circular saw blade that

was missing a couple of teeth. Why bother to pay

big bucks to that Wingate guy for a precision

built and properly balanced tungsten gapped rotary

spark gap with G-10 fiberglass wheel when you

could make your own for next to nothing?

Bill's own idea of a rotary spark gap was powered

by an ancient single phase AC motor that Bill had

scrounged from the local dump for a few bucks. The

motor and circular saw blade were connected

via a belt and pulley arrangement

since the shaft of the AC motor was somewhat bent,

and could not reliably direct-drive the saw blade.

As it was, when the spark gap motor was powered on,

the wooden base to which the entire Spark Gap

assembly was bolted would shake all over the place

and make an awful racket. Bill had jerry-rigged a

device that allowed him to vary the phasing on the

spark gap by rotating the motor by pulling on a

four foot long two-by-four. Bill had wanted to use

a ten-foot-pole, but I told him that nobody would

want to come near it with a ten-foot pole. That

seems to have convinced him.

Looking through the archives, Bill

had found a posting by John Freau on how to convert

small AC motors into fully synchronous motors.

Oblivious to the fact that the conversion pertained

only to SMALL AC motors, Bill modified his motor

anyhow, and found that after filing away large

chunks of his rotor that the modification only made

his motor lopsided. Now it REALLY jumped around when

he turned on the power. So he held the rotary spark

gap assembly wooden base plate as still as he could

by temporarily holding it down with a couple of old

lawn mower engines that he had hanging around.

He made a mental note to drop John a nasty note

telling him how useless his modification had been.

Back a few feet from the Spark Gap was the One Eared

Pole Pig. Thick high voltage cables snaked across

the floor from the Pig to the Spark Gap and the rest

of the Tesla Coil. Not wanting to cut the cables,

(which he had borrowed from work without asking)

Bill had left each cable its original length of

fifty feet.

The Pig was fed a diet of 220 VAC from a 100 amp

Service Line. Now, the Pole Piggie was only

rated at 10KVA, but Bill had read somewhere on

the List that you could actually push a Pig to

two or three times its rated power capacity if

you kept the run short, (so that you didn't boil

off all the oil). Bill planned to test this theory

tonight.

Now, Bill SHOULD have had a number of things that

he didn't. Such as common sense and an ON/OFF switch.

Being something of a cheapskate, he had decided not

to bother with installing a silly little thing like

an ON/OFF switch, because the guy at the dump wanted

more than two bucks for the ones he had in the big

box marked "Electrical Stuff". He knew that he

needed something to limit the current to the Pole

Pig, so he decided to wire a couple of toaster

ovens and a string of Christmas tree bulbs in

*parallel* with the primary of the Pole Pig. He

could have SWORN that he had read a post somewhere

(maybe on the Tesla-2 List) about putting some sort

of a load in series or parallel or something with

the transformer primary.

Bill knew that a variac was really a "must", but

he didn't have one. But he had read a post that

seemed to imply that you could modify a three phase

AC motor to act as some kind of a variable

transformer. Sneaking into the dump under cover of

darkness, he liberated a 400 pound three phase

AC motor that had once seen service in an office

building as the elevator motor. A few whacks with

an axe in just the right places and he had

de-commisioned one set of windings. He knew he

only needed two. But which two? He hoped it was the

two that still remained. He welded two metal stubs

to the casing and then welded a three foot length

of one inch diameter solid steel rod to the rotor

shaft. Now the motor shaft could only turn 90

degrees.

It was still a minute or two before the In-Laws were

scheduled to arrive. One last check and Bill was

ready for an operational test. He turned on the

spark gap motor. Whump!      Whump!     Whump!

Whump! Whump WhumpWHUMPwhumpawhumpawumpawupa...

Yes, the spark gap assembly was a bit, uh,

Vibrational, but seemed to be holding together OK.

Bill lined the plug up with the socket (remember,

he had no ON/OFF switch), and rammed the plug into

the socket. BZZZZSHHHT! The spark gap lit up with

bright actinic light and would have fried Bill's

eyeballs in no time at all if not for the fact that

Bill (always safety conscious) had quickly put on

a pair of welding goggles. Now he could look at

the spark gap arc with impunity. Which he did.

Unfortunately, the goggles were so dark that that

is ALL that Bill could see.

Carefully shielding his eyes with his

left hand, he used his right hand to gingerly

lift the goggles and look at the Christmas tree

on the front lawn.

Outside, the Christmas tree came to life with a

pale glow of pink and blue corona that fuzzed out

for about two feet. But no arcs. No streamers.

What a bummer! He stuffed another wad of cotton

in each ear so that he could think again,

and taking a deep breath of the ozone-soaked air,

he groped his way over towards the modified

three phase motor. Grabbing a hold of the metal

rod, he strained to change the angle of the rotor.

Suddenly a forest of fierce white arcs as thick as

his arm broke out between the primary and the

secondary. DARN!

He released the control rod. TWANG!!!! The rod

slammed itself into the short stub that acted as a

stop. Bill yanked the plug out of the socket.

The only sound was the whappawhappawhapa of the

rotary gap assembly, and the insistent

buzzing that was only in Bill's ears. DARN! The

secondary was arcing to the primary. Maybe if

he added a capacitive load to the secondary he

could get this sucker to stop arcing. He looked

around for something... anything... to use

as a capacitive load. His eyes came to rest on

the leering sharp-toothed smile of his butane

tank work of art, the Halloween Tank-O-Lantern.

To amuse the neighbors and anyone else foolish

enough to approach his house at Halloween, he

had fashioned a gruesome Tank-O-Lantern by using

a cutting torch to fashion leering eyes and

drooling teeth from the once-smooth surface of

the butane tank. The various burn marks from

the cutting torch operations made the tank look

even more sinister when the light played on

it just right.

Grabbing the Tank-O-Lantern, he managed to get

it to sit on top of the existing anti-corona

ring. It wobbled a bit, but what the heck.

For the second time that evening, Bill rammed the

plug into the socket. Again the spark gap burst into

life. The horrendous roar of the spark gap beat

against his ears as he grabbed the control rod and

pulled. A fierce blue corona outlined the eyes and

teeth of the leering Tank-O-Lantern, and as he

gleefully peeked out from under the welding

goggles, Bill could plainly see that beautiful two

to three foot arcs were issuing forth from the

branches of the aluminum Christmas tree. Not

bad. The system was obviously a little out of tune,

and he didn't have much more time before the

In-Laws would arrive, so he would just have to run

it the way it was. But he wanted the tree to look

a bit more Christmas-sy so he made a quick trip

to the attic and got some really awful Christmas

ornaments, (the kind his In-Laws liked), to

decorate the tree.

Unknown to Bill, while he was out decorating

the tree, his wife came out to the garage with a

bag full of garbage that she didn't want in her

kitchen for her parents to see. Looking

around in the garage she spied what she knew was

a plastic garbage can sitting inside what

looked like a cardboard container of some kind.

Seeing no other garbage container around,

she decided to dump the load of trash into the

plastic garbage can. So she did. And with

interesting results.

Bill was totally unaware that the coupling and

inductance of his coil had been changed slightly

by the addition of various beer cans and tuna

fish containers that had been dumped into the

core of his beloved experimental Magnifier Coil.

The addition of the glass Christmas tree ornaments

had little effect on the capacitance of the

Christmas Tree Topload, but Bill's last-minute

addition of a large copper toilet ball to the very

*top* of the tree had changed the isotropic

capacitance of the total topload just enough that

the "extra" coil and topload were in perfect tune.

When Bill saw his In-Laws exit their car and begin

their trip down the shoveled walkway that led

within a few feet of the Christmas Coil, he

stationed himself next to the power outlet and

waited until they were at just the right spot.

He jammed the plug into the socket and ran over

to the control rod. Little beads of sweat broke

out on his forehead as he pulled back on the

control rod. Meanwhile, inside the garbage can

secondary, RF induction heating was taking place

on the cat food tin cans. The heat caused the

garbage to shift suddenly, and in that instant

a wonderful serendipity took place. For a few

fleeting cycles, PERFECT resonance was achieved!

Megawatts of energy happily surged back and forth

in the slipshod tank circuit of the amazing

Christmas Coil. Phase angles slipped past one

another invisibly and fell in-synch. Due to a

couple of missing teeth on the makeshift rotary

spark gap's circular saw blade the caps ceased

to fire for a moment, and the capacitor bank

experienced an Anomalous Resonant Rise. An

instant later the excessive voltage caused a

particularly massive dump of energy into the

primary circuit at precisely the right phase

angle, and the resulting surge in energy passed

from the base of the wildly glowing Tank-O-

Lantern down the copper tubing transmission

line, which looked as though it were ringed

with fire. The transmission line was just

exactly the right length to allow the electrical

wave travelling down it to slam into the base

of the "extra" coil precisely at a zero voltage,

MAX current node. The "extra" coil and the

Christmas Tree Topload with round copper toilet

ball were exactly matched to the impedance

required, and the massive driving force of

Megawatts of resonant energy caused the

"extra" coil to react like a spring that had

been hit hard with a hammer. The resulting

jump in energy caused the voltage at the

Christmas Tree to exceed the breakdown voltage

of the winter air. With a mad, screeching

KaBOOOM the air broke down and a single

solitary streamer launched itself into the

cold dark night. Up, UP, *UP* it climbed,

and then, seeking the path of least

resistance, it curved over. Escaping the

intense electrostatic forces that existed at the

surface of the aluminum tree, the mighty bolt

of lightning swerved around and headed for the

nearest conductive object it could find.

The In-Laws would have been toasted alive were

it not for the one object that caught the Arc's

attention. Beyond the In-Laws, a good twenty

feet from where the Lightning Bolt had launched

itself from the infamous Copper Toilet Ball was

an old fashioned lamp post. It put out a dim

but cheery quantity of light that seemed

to beckon to the Wayward Lightning Bolt. Like a

giant white arm, the lightning bolt swerved around

and slammed into the cheery lamp post.

PHHHHHHHT! KABLAMMMMM! Like a gigantic flash lamp

the lightning bolt lit up the night with an

instant of blinding whiteness and a deafening

BLAM that reverberated in the In-Law's ears long

after their knees had stopped shaking.

A flash of light. A moment of raw, awesome beauty,

and then DARKNESS as all the electricity for blocks

around ceased to flow.

It is a Christmas that Bill will always remember.

It is a Christmas that his wife and In-Laws will

never let him forget.

*****

This is just a story. Any resemblance to any persons

living or dead named Bill is merely coincidental.

As the author of this tale I want to say that the

story is not meant to be a plan for how to build

your own Arcstarter -er- I mean, Magnifier Coil.

The theory contained herein is only partially

believeable, and is not meant to be a factual

accounting of what actually happens in a Tesla Coil.

I hope you enjoy this little romp in the spirit

in which it was written.

I wrote it especially for my good friend (at least

he WAS my good friend), Bill the Arcstarter Pollack.

But, as the disclaimer said, it is not actually

ABOUT him. Even though he does hate cats and

builds Tank-O-Lanterns, and dabbles in Tesla Coils.

This story is just about a Bill who happens to

be an awful lot LIKE Bill Pollack. The real Bill

would *never* borrow anything from work without

asking. He is beyond reproach. He is my friend.

Written by

Fr. Tom McGahee

Tesla List and Tesla-2 List Member

Date: Wed, 23 Dec 1998 18:56:19 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Plasma Globe

Sender: tesl-err@emachine.com

mel wise wrote:

>

> I know this doesn't relate to tesla coils, but I need some help on

> something. I have a plasma globe that I bought awhile back. It has an

> 8" diameter globe with a black base. When I bought the plasma globe it

> emitted lots of purplish pink streamers everywhere. Now on the other

> hand it doesn't seem to look as good as it once did before. I don't know

> what the problem with it is. It still emits streamers but there are less

> of them and they don't seem to be as bright in color.

>

> The ciruit is a 555 type circuit. The flyback transformer is a fairly

> small one. Here are my questions. First, could it be the flyback going

> bad? Second, could it be that the circuitry is degrading in performance.

> Third, the circuit has a 5k ohm trimmer in it, could this be my problem.

> It has been adjusted it to get peek performance in the globe.

>

> If anyone might know what the problem is please reply to me off list.

>

> Thanks

> Chris

I may be wrong but I think pressure of the gas in the globe is

increasing. This may be caused by improper baking out of the glass

during manufacture. I hope this helps.

Frankensteins Helper

Max

From: McFalc0n@aol.com

Date: Sat, 26 Dec 1998 23:19:29 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: X-RAY PICTURES

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone know how to make or find a tube to make x-rays from a tesla coil?

Film?  I need to see what is inside something...  Does anyone have any

experience or ideas? Thanx.

Date: Fri, 25 Dec 1998 00:42:05 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re:Which circuit?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What configuration of my spark gap and capacitors should I use to

provide the best protection for my NST. I know performance stays the same.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: X-RAY PICTURES

Date: Sun, 27 Dec 1998 03:26:14 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Sat, 26 Dec 1998 23:19:29 EST, you wrote:

>	Does anyone know how to make or find a tube to make x-rays from a tesla coil?

>Film?  I need to see what is inside something...

If you just want to take an X-ray of a specific item, go to your local

dentist/vet etc. Much cheaper & safer!

From: Hollmike@aol.com

Date: Sun, 27 Dec 1998 10:29:46 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: X-RAY PICTURES

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/27/98 8:27:07 AM Mountain Standard Time,

wwl@netcomuk.co.uk writes:

> >	Does anyone know how to make or find a tube to make x-rays from a tesla  coil?

> > Film?  I need to see what is inside something...

>

>  If you just want to take an X-ray of a specific item, go to your local

>  dentist/vet etc. Much cheaper & safer!

>

Another possibility is a company that X-Rays pipes for flaws.  Their setup may

be better designed for such work.

Date: Sun, 27 Dec 1998 16:30:50 +0100

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: X-RAY PICTURES

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

McFalc0n@aol.com wrote:

>

>         Does anyone know how to make or find a tube to make x-rays from a tesla coil?

> Film?  I need to see what is inside something...  Does anyone have any

> experience or ideas? Thanx.

Some light bulbs are evacuated. If you put one of these on your TC you

will notice a greenish glow on the inner surface of the bulb. X-Rays!

I don't have an idea how intense they are but I guess they will have a

pretty short wavelength due to the high acceleration voltage.

Hope this helps,

Stefan

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: X-ray Pictures

Date: Sun, 27 Dec 1998 14:06:01 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Check out this site:

http://www.mathematik.uni-marburg.de/~kronjaeg/hv/index.html#xray

Jon Rosenstiel   jonr@pacbell.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Fair Radio Transformers

Date: Mon, 28 Dec 1998 10:49:47 CST

From: Kent Vander Velden <graphix@iastate.edu>

Sender: tesl-err@emachine.com

X-Mailer: IPAD 2.0

On the tesla mailing list someone pointed out a nice potential

transformer that Fair Radio is selling.  The transformer is a 6.3kv

500mA type.  Would one of these transformers work well for a Tesla

coil?  The part that that I am not sure of is the lower voltage.

At the moment I use two 15kV 60mA neons in parallel for a total of

1800W.  It would seem that if the transformer from Fair Radio was

current limited to 300mA the same power output would be achieved.

Also, is it correct to assume that this transformer is likely built

tougher than a neon?

Thanks.

---

Kent Vander Velden

kent@iastate.edu

From: DavidH8083@aol.com

Date: Mon, 28 Dec 1998 11:56:30 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re:  Fair Radio Transformers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/28/98 11:50:48 AM, you wrote:

<<  On the tesla mailing list someone pointed out a nice potential

transformer that Fair Radio is selling.  The transformer is a 6.3kv

500mA type. >>

I don't think it is a potential x-former.  I think it is a plate x-former.

Keep in mind it weighs 90 lbs.  Unless you are building a big tube coil, you

would probably be better off with the Neons.

David Hutchison

davidh8083@aol.com

Date: Mon, 28 Dec 1998 11:26:18 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Fair Radio Transformers

Sender: tesl-err@emachine.com

Kent Vander Velden wrote:

>

>   On the tesla mailing list someone pointed out a nice potential

> transformer that Fair Radio is selling.  The transformer is a 6.3kv

> 500mA type.  Would one of these transformers work well for a Tesla

> coil?  The part that that I am not sure of is the lower voltage.

> At the moment I use two 15kV 60mA neons in parallel for a total of

> 1800W.  It would seem that if the transformer from Fair Radio was

> current limited to 300mA the same power output would be achieved.

>

>   Also, is it correct to assume that this transformer is likely built

> tougher than a neon?

>

>   Thanks.

>

> ---

> Kent Vander Velden

> kent@iastate.edu

Kent,

This transformer is definitely not a potential transformer, but is more

likely simply a plate transformer (most likely from a vacuum tube

transmitter). Since it's rated at 500 mA and weighs 90 pounds, it

undoubtedly is more robust than a Neon Sign Transformer (NST) with

respect to secondary wire size and overload capability. However, with an

insulation breakdown voltage of 7500 volts, it's no better than the

insulation rating of your 15 kV 60 mA NST's, and it could actually be

poorer in this regard.

In comparison, a potential transformer (used in the electric power

industry) in the 5 - 8.7 kV voltage class would typically have an

insulation-breakdown rating of 60-75 kV(!). Potential transformers are

tough little cookies! Note also that the Fair Radio plate transformer

has no built-in current limiting, so you'll need to use external

ballasting.

-- Bert --

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: X-RAY PICTURES

Date: Mon, 28 Dec 1998 12:43:01 -0600

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Have a look here.

http://www.noah.org/science/x-ray/index.html

Roy Minnich

----------

From: 	McFalc0n@aol.com[SMTP:McFalc0n@aol.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Saturday, December 26, 1998 10:19 PM

To: 	Tesla-2 Mailing List

Subject: 	X-RAY PICTURES

Does anyone know how to make or find a tube to make x-rays from a tesla coil?

Film?  I need to see what is inside something...  Does anyone have any

experience or ideas? Thanx.

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Muffin fan

Date: Mon, 28 Dec 1998 20:40:49 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I have a 120Vac, 14.5W muffin fan on my spark gap, can the fan speed be

controlled with a dimmer made for incandescent lights, or must one use a

variac?

Jon Rosenstiel   jonr@pacbell.net

Date: Mon, 28 Dec 1998 23:02:27 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Muffin fan

Sender: tesl-err@emachine.com

Jon Rosenstiel wrote:

>

> Hi,

> I have a 120Vac, 14.5W muffin fan on my spark gap, can the fan speed be

> controlled with a dimmer made for incandescent lights, or must one use a

> variac?

>

> Jon Rosenstiel   jonr@pacbell.net

Sure, dimmer should work fine with such low power. Don't expect it to be

very linear, though.

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Muffin fan

Date: Tue, 29 Dec 1998 01:39:32 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Tue, 29 Dec 1998 00:59:39 -0600, you wrote:

>I must disagree - I've tried it and it DOES NOT work.  The output from

>a light dimmer switch is NOT AC.  It will burn up your AC motor in seconds

>and then blow the fuse and burn out the dimmer switch because to DC

>that is a short.

>

>>> Jon Rosenstiel   jonr@pacbell.net

>>Sure, dimmer should work fine with such low power. Don't expect it to be

>>very linear, though.

>>

>

It depends on the type of motor - many types can be controlled with a

dimmer, but you may need to put a snubber across the dimmer (100n cap

+ 100R resistor in series) due to the inductive load.

Large motor loads can sometimes cause problems due to the inductive

phase shift causing it to only fire on alternate half cycles,

producing DC which saturates the inductance & generally results in

smoke!. If in doubt use a motor speed controller designed for motor

loads rather than a dimmer.

From: "Jeff Corr" <corr@enid.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Muffin fan

Date: Tue, 29 Dec 1998 00:59:39 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I must disagree - I've tried it and it DOES NOT work.  The output from

a light dimmer switch is NOT AC.  It will burn up your AC motor in seconds

and then blow the fuse and burn out the dimmer switch because to DC

that is a short.

>> Jon Rosenstiel   jonr@pacbell.net

>Sure, dimmer should work fine with such low power. Don't expect it to be

>very linear, though.

>

Date: Mon, 28 Dec 1998 23:36:15 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: tc photos

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Last time I took pictures of my coil in operation I used 1000 isa

film, they turned out very well with realistic sparks and the tc itself

being visable. What do you guys use for film?

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Tue, 29 Dec 1998 11:10:49 -0600

From: mel wise / chris <mwise44@flash.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Secondary coating

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Is anyone familiar with a product called PlastDip? Does anyone know any

specific dielectric properties of this stuff. I have 2 cans of this

which I bought a Wal-Mart. I am not real sure of any advantadges of

using this stuff, but I do know that is very durable and will last a

long time. I have personally used polyurethane before but I do not like

its long term effects. From experience it does begin to chip and peel

off. So I thought about this stuff and was wondering if it would make a

suitable secondary coating.

The only ill effect I am worried about is what it is made of. According

to be label it contains Naphtha, Hexane, Toluene, and Methyl Ethyl

Ketone. Especially I am wondering about Naphtha, Toluene and Methyl

Ethyl Ketone. They are all solvents and I am wondering if they would

have any damaging effects to the enamel coating on magnet wire.

Any input on this would be very helpful.

Chris

Date: Tue, 29 Dec 1998 11:23:51 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Muffin fan

Sender: tesl-err@emachine.com

Jeff Corr wrote:

>

> I must disagree - I've tried it and it DOES NOT work.  The output from

> a light dimmer switch is NOT AC.  It will burn up your AC motor in seconds

> and then blow the fuse and burn out the dimmer switch because to DC

> that is a short.

>

> >> Jon Rosenstiel   jonr@pacbell.net

> >Sure, dimmer should work fine with such low power. Don't expect it to be

> >very linear, though.

> >

I have used dimmer switches successfully on refrigerator blower motors

and other small AC motors; with my dimmers, the output is AC, chopped to

vary the duty cycle. Maybe it depends on the type of dimmer switch you

use.  If you haven't bought one, a ceiling fan control should work.

Date: Tue, 29 Dec 1998 13:06:52 -0500

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: tc photos

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I used 400 film, with exposures of around 1 second, in the dark with a

flashlight pointed at the coil.

-tom

http://www.geocities.com/capecanaveral/1348/

From: Hollmike@aol.com

Date: Tue, 29 Dec 1998 13:25:11 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: neon surgery

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, all,

It seems that my first attempt at unpotting a 15/60 neon was a success.

The only damage that I caused was a small nick in one of the primary leads

while chipping out some of the tar - no big deal there :o)

The cause of carbon tracking in neons seems to be due to a poor design,

at least in Franceformers.  I noticed that where they decided to ground

the secondaries is at the top of the transformer core just a short distance

away from the high voltage leads(unless this particular one was inadvertantly

mounted upside-down).

The question I have is:  Has anyone tried using petroleum jelly for

repotting a neon?  I have plenty of transformer oil, but in the event of a

containment breach, I would have a serious mess to deal with(I presently am

forced to do my coiling in a spare bedroom for lack of a garage).  I heard

somewhere that petroleum jelly is a good insulating material, but would like

confirmation if possible, before considering this option.  It would be easy

enough to heat it and liquify it in order to allow it to seep into the nooks

and crannies then let it cool and re-gel.

Comments?

Mike

Date: Tue, 29 Dec 1998 13:42:49 -0500

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: neon surgery

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What a neat idea! After all, it's just another petroleum product right?

It probably won't be cheaper though...I did find some motor oil that claims

that it has absoloutley no additives (detergents, etc)...I've been using it in

bottle caps...I might

try it in my plate caps in the near future...

-tom

>     The question I have is:  Has anyone tried using petroleum jelly for

> repotting a neon?  I have plenty of transformer oil, but in the event of a

> containment breach, I would have a serious mess to deal with(I presently am

> forced to do my coiling in a spare bedroom for lack of a garage).  I heard

> somewhere that petroleum jelly is a good insulating material, but would like

> confirmation if possible, before considering this option.  It would be easy

> enough to heat it and liquify it in order to allow it to seep into the nooks

> and crannies then let it cool and re-gel.

>    Comments?

> Mike

>

>

Date: Tue, 29 Dec 1998 14:42:29 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: neon surgery

Sender: tesl-err@emachine.com

Hollmike@aol.com wrote:

>

> Mike, all,

>     It seems that my first attempt at unpotting a 15/60 neon was a success.

> The only damage that I caused was a small nick in one of the primary leads

> while chipping out some of the tar - no big deal there :o)

>      The cause of carbon tracking in neons seems to be due to a poor design,

> at least in Franceformers.  I noticed that the where they decided to ground

> the secondaries is at the top of the transformer core just a short distance

> away from the high voltage leads(unless this particular one was inadvertantly

> mounted upside-down).

>     The question I have is:  Has anyone tried using petroleum jelly for

> repotting a neon?  I have plenty of transformer oil, but in the event of a

> containment breach, I would have a serious mess to deal with(I presently am

> forced to do my coiling in a spare bedroom for lack of a garage).  I heard

> somewhere that petroleum jelly is a good insulating material, but would like

> confirmation if possible, before considering this option.  It would be easy

> enough to heat it and liquify it in order to allow it to seep into the nooks

> and crannies then let it cool and re-gel.

>    Comments?

> Mike

>

>

Mike,

Petroleum jelly (Vaseline) or petroleum gel is technically known as

"white petrolatum, USP". This is actually a combination of a fine

crystalline paraffin wax and mineral oil. Its dielectric properties lie

somewhere between paraffin and mineral oil (both being excellent, low

loss dielectrics), and it has been used in the electric power industry

for years to help insulate low voltage cables and as an impregnent in

kraft paper power factor correction (PFC) capacitors.

Since it would liquify at a relatively low temperature and has such good

dielectric properties, it would be a MUCH better alternative than the

bitumen that was originally used in the NST. And it would also be easier

to contain than liquid mineral oil.

Good luck, and let us know how it works out!

-- Bert --

From: "Jeff Corr" <corr@enid.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Muffin fan

Date: Tue, 29 Dec 1998 15:50:40 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Perhaps I should offer my exact setup when this did not work.

I was using a light dimmer switch first of all, rated at 600W.

The load on the switch was a microwave oven transformer fan.

As I turned it up it got very hot and when I tried to turn the blade

with my hand, it was as if a large brake was holding it still.  It got

very hot, very fast.  That fan never worked again.

>I have used dimmer switches successfully on refrigerator blower motors

>and other small AC motors; with my dimmers, the output is AC, chopped to

>vary the duty cycle. Maybe it depends on the type of dimmer switch you

>use.  If you haven't bought one, a ceiling fan control should work.

X-Mailer: IPAD 2.0

Date: Tue, 29 Dec 1998 23:57:26 +0000

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: MOT's

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All,

have just cleared out the local microwave oven shop of all their old junk,

and now have about 50 MOTS!!

so if any UK coilers want any get in touch off list. I remember someone

talking about using 8 off them in some configuration where they don't see

more than there design voltage but I can't find the reference, can someone

point me in the right direction please. I know it was one of the European

coilers as mot's are so much easier to get than neons so there is more

incentive to try and use them.

many thanks

bob golding

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOT's

Date: Tue, 29 Dec 1998 22:36:27 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Tue, 29 Dec 1998 23:57:26 +0000, you wrote:

>Hi All,

>	have just cleared out the local microwave oven shop of all their old junk,

>and now have about 50 MOTS!!

>so if any UK coilers want any get in touch off list. I remember someone

>talking about using 8 off them in some configuration where they don't see

>more than there design voltage but I can't find the reference, can someone

>point me in the right direction please. I know it was one of the European

>coilers as mot's are so much easier to get than neons so there is more

>incentive to try and use them.

>

>many thanks

>bob golding

Hi, Bob - can you hang on to about 5 for me.

When I get time, I'm, intending to put some work into investigating

how to make MOTs work well for TC's, as especially outside the US,

where neons seem to abound, they're probably the most easily

obtainable HV source wherever you are in the world.

Using a pair shouldn't be a problem, and if the cores are isolated,

three should be pretty OK before you need to go dunking them in oil.

The main problem with the low-ish voltage seems to be quenching as far

as I can make out. Maybe a rotary gap would be useful here?

I saw one reference somewhere to using MW diodes and caps in a voltage

multiplier arrangement for a DC coil.

Date: Tue, 29 Dec 1998 19:04:30 -0600

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOT's

Sender: tesl-err@emachine.com

bob golding wrote:

>

> Hi All,

>         have just cleared out the local microwave oven shop of all their old junk,

> and now have about 50 MOTS!!

> so if any UK coilers want any get in touch off list. I remember someone

> talking about using 8 off them in some configuration where they don't see

> more than there design voltage but I can't find the reference, can someone

> point me in the right direction please. I know it was one of the European

> coilers as mot's are so much easier to get than neons so there is more

> incentive to try and use them.

>

> many thanks

> bob golding

My question is if you could put the primaries in parallel and the

secondaries in series to get your required output.

From: Hollmike@aol.com

Date: Tue, 29 Dec 1998 20:21:26 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOT's

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/29/98 5:10:09 PM Mountain Standard Time, yubba@clara.net

writes:

> Hi All,

>  	have just cleared out the local microwave oven shop of all their old junk,

>  and now have about 50 MOTS!!

>

Nice Catch!!,

Mike

X-Originating-IP: [216.68.40.52]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: neon surgery

Date: Tue, 29 Dec 1998 19:28:59 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike Hollingsworth wrote:

>    The question I have is:  Has anyone tried using petroleum jelly for

>repotting a neon?  I have plenty of transformer oil, but in the event of a

>containment breach, I would have a serious mess to deal with

Somebody out there was mixing melted vasoline with diala-ex or mineral

oil to make a compound slightly thinner than just solid vaseline.

I think that using a substance so thick that it won't flow is asking for

trouble!  I've also heard of people who have encapsulated various HV

equipment in parrafin, only to have it break down.

-Bill (just got back from vacation and am still recovering!)

From: DavidH8083@aol.com

Date: Tue, 29 Dec 1998 22:58:31 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary coating

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/29/98 12:11:03 PM, mwise44@flash.net writes:

<<The only ill effect I am worried about is what it is made of. According

to be label it contains Naphtha, Hexane, Toluene, and Methyl Ethyl

Ketone. Especially I am wondering about Naphtha, Toluene and Methyl

Ethyl Ketone. They are all solvents>>

I am astounded that this stuff is being sold at all.  All of these items on

the EPA hit list and have long been banned for general consumer use. If you

dare to use this stuff make sure it is in an EXTREMELY well-ventilated area,

and good luck disposing of any excess.

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOT's

Date: Wed, 30 Dec 1998 06:08:21 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Tue, 29 Dec 1998 19:04:30 -0600, you wrote:

>bob golding wrote:

>>

>> Hi All,

>>         have just cleared out the local microwave oven shop of all their old junk,

>> and now have about 50 MOTS!!

>> so if any UK coilers want any get in touch off list. I remember someone

>> talking about using 8 off them in some configuration where they don't see

>> more than there design voltage but I can't find the reference, can someone

>> point me in the right direction please. I know it was one of the European

>> coilers as mot's are so much easier to get than neons so there is more

>> incentive to try and use them.

>>

>> many thanks

>> bob golding

>

>My question is if you could put the primaries in parallel and the

>secondaries in series to get your required output.

No - the insulation won't stand it. Remember MOTs are built very

cheaply, so the insulation will probably be only just adequate - I

doubt they would stand much more than twice their rating, which

assuming the core is insulated from both ends of the secondary,

implies no more than 4 with a centre ground, but you'd have very

little margin to handle RF kickback from the TC - safety gaps and/or

big MOVs would be essential.

From: Hollmike@aol.com

Date: Wed, 30 Dec 1998 07:56:35 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: neon surgery

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 12/29/98 8:30:11 PM Mountain Standard Time,

arcstarter@hotmail.com writes:

>

>  I think that using a substance so thick that it won't flow is asking for

>  trouble!  I've also heard of people who have encapsulated various HV

>  equipment in parrafin, only to have it break down.

>

>  -Bill (just got back from vacation and am still recovering!)

>

Hi Bill,

I was figuring on heating the petroleum jelly until it is liquid before

potting the neon(if I actually decide to go this route).  If I keep it hot for

a good long time, it should seep into the trannie better than the tar would.

It would also be sort of self healing if a carbon track were to form since it

would melt at a much lower temperature than tar.

I used paraffin for preventing arcs between the copper plates used to bank

a bunch of 30kV transmitter caps.  It held up very well for about 3 years.  My

capacitor bank finally suffered a carbon track across one of the caps, but it

was the one cap closest to the surface of the paraffin.  I think the paraffin

was degraded by corona over time and finally broke down.  I imagine that if I

had about 1/4 inch more covering the bank, that wouldn't have happened.

Mike

Date: Thu, 31 Dec 1998 00:24:19 +1100

From: Corey Kent <corey@enterprise.net.au>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ignition Coil.

Sender: tesl-err@emachine.com

what type of ignition coil do i need? one for the older type of cars or

one for the newer type of cars?

there is a difference in case you didnt know.

and has anyone on this list made a TC out of a ignition coil??

thanx

cya's

From: "Chris Crochet" <ccrochet@earthling.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary coating

Date: Wed, 30 Dec 1998 11:53:12 -0600

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

><<The only ill effect I am worried about is what it is made of. According

>to be label it contains Naphtha, Hexane, Toluene, and Methyl Ethyl

>Ketone. Especially I am wondering about Naphtha, Toluene and Methyl

>Ethyl Ketone. They are all solvents>>

>

>I am astounded that this stuff is being sold at all.  All of these items on

>the EPA hit list and have long been banned for general consumer use. If you

>dare to use this stuff make sure it is in an EXTREMELY well-ventilated area,

>and good luck disposing of any excess.

Considering I can buy three out of these four solvents in nearly pure form

at the local hardware shops, and I have numerous paints and paint strippers

containing one or more of these substances, I doubt they're banned for

general consumer use.

Inhaling toxic fumes in LA,

Chris Crochet

Date: Wed, 30 Dec 1998 10:54:58 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary coating

Sender: tesl-err@emachine.com

mel wise / chris wrote:

>

> Ia anyone familiar with a product called PlastDip?

> The only ill effect I am worried about is what it is made of. According

> to be label it contains Naphtha, Hexane, Toluene, and Methyl Ethyl  Ketone.

> Any input on this would be very helpful.

>

> Chris

Chris,

MEK, may be a solvent for the enamel-- as may toluene. I'd test some of

the mtl on a sample of your 2ndary wire.  Mostly, if you paint over the

coil, there should not be any serious effects. Most enamels are fairly

hardy and should survive.

Consider a tacky underlayer (need to assess drying rate) of the mtl as

you do your winding-- would mechanically stabalize the coil, prior to

any final overcoats.

----------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Wed, 30 Dec 1998 11:26:11 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: TESLA-2@emachine.com

Subject: Self-tuning Xmas display

Sender: tesl-err@emachine.com

Bill,

While you may have no connection to the obvious fiction pubished by Fr.

T. McGahee, you may have insights to the tank and associated circuitry

circuitry he described. I would be interested in a diagram, if

available.  The tank-o-lantern as top load has potential for the next

halloween...?

Could the system capacitance be achieved by design, or is a Frau and

random dumping of trash into system components required? What would be

the effect of the actual cat as a system component, in lieu of merely

the cat food cans?

Also, I lack in-laws as the specified ground potential sources, but

there is an adequate supply of ajacent kinder.... Can we rely on the

skin effect to reduce wrongfull death litigations?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Wed, 30 Dec 1998 14:30:22 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Which circuit? lets try this again.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike N. wrote:

>

> What configuration of my spark gap and capacitors should I use to

> provide the best protection for my NST. I know performance stays the

> same.

>

> --

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

From: McFalc0n@aol.com

Date: Wed, 30 Dec 1998 18:01:02 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: flyback trans from mac monitor

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

does anyone know where I can get good plans for rewinding or using a flyback

from an old macintosh monitor.  I have it in good condition still wound.

Should I keep it so?  Any suggestions?

X-Mailer: IPAD 2.0

Date: Wed, 30 Dec 1998 19:29:07 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Muffin fan

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:40 PM 12/28/98 -0800, you wrote:

>Hi,

>I have a 120Vac, 14.5W muffin fan on my spark gap, can the fan speed be

>controlled with a dimmer made for incandescent lights, or must one use a

>variac?

>

>Jon Rosenstiel   jonr@pacbell.net

>

Jon,

In my experience speed control on a gap fan is not

really of any benefit. When using a fan to blow air

through the gap you need all the airflow you can get.

However when using a vacuum cleaner motor to suck air

through the gap some form of speed control is worthwhile.

Best operation of a vacuum gap occurs at relatively low

vacuum cleaner speeds. Exactly why this happens Im not

quite sure. Perhaps it has to do with the lower air

pressure inside the gap housing.

So I wouldnt bother with the speed control on your

muffin fan.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 30 Dec 1998 19:39:52 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Which circuit? lets try this again.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:30 PM 12/30/98 -0800, you wrote:

>Mike N. wrote:

>>

>> What configuration of my spark gap and capacitors should I use to

>> provide the best protection for my NST. I know performance stays the

>> same.

>>

>> --

Mike,

There is only one configuration you should consider.

This is where the spark gap is paralleled across the

HV outputs of the neon.

The "other" common configuration showing the cap across

the transformer is sure death for a neon. The main cap,

spark gap and primary coil form a pulse driven LC oscillator.

The cap will of necessity have very high RF voltages across it.

Those high RF voltages would also appear across your neon

secondary if the cap was in parallel. With the spark gap

paralleled across the neon there is a measure of protection

due the the energized gap forming a short circuit across the

neon secondary. This will keep the voltages low while the gap

is conducting.

This protection is not perfect. There can still be some ringing

that occurs when the gap quenches. So the parallel gap wont

guarantee neon protection but be assured that the parallel

cap arrangement will toast your neon in short order.

Dont forget the safety gap.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Jon Rosenstiel" <jonr@pacbell.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Which circuit? lets try this again.

Date: Wed, 30 Dec 1998 18:19:35 -0800

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

How much protection would a saftey gap across the xfmr provide? Doesn't the

saftey gap by necessity have to have a wider gap than the main? If the main

is as close as possible, (wiring length), to the xfmr wouldn't the main gap

fire before the saftey? What am I missing here?

Jon

-----Original Message-----

From: Mike Hammer <mhammer@misslink.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Wednesday, December 30, 1998 5:44 PM

Subject: Re: Which circuit? lets try this again.

>At 02:30 PM 12/30/98 -0800, you wrote:

>>Mike N. wrote:

>>>

>>> What configuration of my spark gap and capacitors should I use to

>>> provide the best protection for my NST. I know performance stays the

>>> same.

>>>

>>> --

>

>Mike,

>

>There is only one configuration you should consider.

>This is where the spark gap is paralleled across the

>HV outputs of the neon.

>

>The "other" common configuration showing the cap across

>the transformer is sure death for a neon. The main cap,

>spark gap and primary coil form a pulse driven LC oscillator.

>The cap will of necessity have very high RF voltages across it.

>Those high RF voltages would also appear across your neon

>secondary if the cap was in parallel. With the spark gap

>paralleled across the neon there is a measure of protection

>due the the energized gap forming a short circuit across the

>neon secondary. This will keep the voltages low while the gap

>is conducting.

>

>This protection is not perfect. There can still be some ringing

>that occurs when the gap quenches. So the parallel gap wont

>guarantee neon protection but be assured that the parallel

>cap arrangement will toast your neon in short order.

>

>Dont forget the safety gap.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  neon surgery

Date: Thu, 31 Dec 1998 04:08:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

H>    The question I have is:  Has anyone tried using petroleum jelly for

H>repotting a neon?  I have plenty of transformer oil, but in the event of a

I have a friend who has a humongous x-ray transformer made a long

time ago.  It had beautiful bakelite and brass terminals on the top of

the unit.  It is about 48" x 18" x 20", and weighs about 120 pounds.  It

requires a current regulator, and when you fire it up, it produces a

roaring arc between the electrodes.  The air gap breakdown distance

between pointed electrodes is about 10 inches. Quite a performer.

Without the regulator it eats 50 amp fuses instantly.

Anyway, back on topic, this beautiful transformer is insulated by

about 10 gallons of Vaseline.  I can attest to the quantity of

Vaseline, because some years ago, I had to remove all of it to fix a

broken wire between a couple of the windings.

As an insulator, it's great.  It's flexible, of course, and conforms

to the windings even if the case flexes or the temperature changes.  I

removed the Vaseline using a "heat gun" made from a blow torch and a

section of 2 inch pipe. I replaced it by melting the stuff on a hotplate

and pouring it back into the case.   I prepared the windings by running

DC through them to heat them up so that the Vaseline would not congeal

upon hitting them, thereby leaving air gaps in the Vaseline.

After the Vaseline was in place and still liquid, I increased the DC

through the windings to force any trapped air from the windings.

Following that, I let the transformer cool down for several days - yep,

it took that long - and did a smoke (gads; I HATE that word!) test.

Everything was fine, and the transformer still graces his living room

today.

Ralph

Date: Wed, 30 Dec 1998 23:34:45 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Which circuit? lets try this again.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer wrote:

>

> At 02:30 PM 12/30/98 -0800, you wrote:

> >Mike N. wrote:

> >>

> >> What configuration of my spark gap and capacitors should I use to

> >> provide the best protection for my NST. I know performance stays the

> >> same.

> >>

> >> --

>

> Mike,

>

> There is only one configuration you should consider.

> This is where the spark gap is paralleled across the

> HV outputs of the neon.

>

> The "other" common configuration showing the cap across

> the transformer is sure death for a neon. The main cap,

> spark gap and primary coil form a pulse driven LC oscillator.

> The cap will of necessity have very high RF voltages across it.

> Those high RF voltages would also appear across your neon

> secondary if the cap was in parallel. With the spark gap

> paralleled across the neon there is a measure of protection

> due the the energized gap forming a short circuit across the

> neon secondary. This will keep the voltages low while the gap

> is conducting.

>

> This protection is not perfect. There can still be some ringing

> that occurs when the gap quenches. So the parallel gap wont

> guarantee neon protection but be assured that the parallel

> cap arrangement will toast your neon in short order.

>

> Dont forget the safety gap.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Actually in an early popular electronics magazine (1973 iirc)it shows

both the gap and cap in parallel, it produced 1' streamers( I think it

was made from bear skins and stone knives)!!!, we've come a long way

since that.

Thanks Mike

X-Mailer: IPAD 2.0

Date: Thu, 31 Dec 1998 01:48:35 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Which circuit? lets try this again.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:19 PM 12/30/98 -0800, you wrote:

>Mike,

>How much protection would a safety gap across the xfmr provide?

The safety gap is the last line of defense. It must

be set to break down just over the neon rating.

>Doesn't the safety gap by necessity have to have a wider gap

>than the main?

Yes. Or maybe more properly stated the main gap should ALWAYS

be less than the safety gap.

>If the main is as close as possible, (wiring length), to the

>xfmr wouldn't the main gap fire before the safety?

Ideally yes. But when the gap quenches there can be some

energy still left in the system. This can sometimes delay

quenching or even sometimes fire the safety gap. This is

often caused by energy stored in bypass and/or RF chokes.

These components can sometimes do more harm than good.

The safety gap is also very helpful in handling the surges from

streamer hits into the primary coil.

>What am I missing here?

I dont think you are missing a thing.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 31 Dec 1998 08:17:20 -0500

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: flyback trans from mac monitor

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Finally ! Someone has found a good use for a Mac. (sorry for the off topic post)

Bert W.

McFalc0n@aol.com wrote:

>         does anyone know where I can get good plans for rewinding or using a flyback

> from an ol macintosh monitor.  I have it in good condition still wound.

> Should I keep it so?  Any suggestions?

From: Hollmike@aol.com

Date: Thu, 31 Dec 1998 09:04:25 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Which circuit? lets try this again.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jon,

This may interest you.  It is some testing of RC filter networks performed

by Terry Fritz.

http://www.peakpeak.com/~terryf/tesla/experiments/rcfilter/rcfilter.html

Design of a Neon Sign Transformer Protection Network for Tesla Coil

Primary Circuit applications

Hope it helps,

Mike Hollingsworth

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: no-tag-request@junoaccmail.org

Date: Thu, 31 Dec 1998 12:08:06 EST

Subject: Re: neon surgery

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Tue, 29 Dec 1998 13:25:11 EST Hollmike@aol.com wrote:

>    The question I have is:  Has anyone tried using petroleum

>jelly for repotting a neon?  I have plenty of transformer oil, but

>in the event of a containment breach, I would have a serious mess

>to deal with(I presently am forced to do my coiling in a spare

>bedroom for lack of a garage).  I heard somewhere that petroleum

>jelly is a good insulating material, but would like confirmation

>if possible, before considering this option.  It would be easy

>enough to heat it and liquify it in order to allow it to seep into

>the nooks and crannies then let it cool and re-gel.

I have never used vaseline or petroleum jelly as an insulator and I have

no idea what it's dielectric strength is let alone it's power factor, BUT

I do know you can mix petroleum jelly with mineral oil and achieve any

consistency you want from the relatively thin oil to the very viscous

petroleum jelly. Just remember to mix them very well and heat the mixture

and allow all the air bubbles to rise out of the mix.

Alfred A. Skrocki Sr.

Alfred.Skrocki.Sr@JUNO.COM

Date: Thu, 31 Dec 1998 12:36:36 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: flyback trans from mac monitor

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I use the flyback ferrite cores as choke cores.

Bert Wikkerink wrote:

>

> Finally ! Someone has found a good use for a Mac. (sorry for the off topic post)

>

> Bert W.

>

> McFalc0n@aol.com wrote:

>

> >         does anyone know where I can get good plans for rewinding or using a flyback

> > from an ol macintosh monitor.  I have it in good condition still wound.

> > Should I keep it so?  Any suggestions?

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Sender: tesl-err@emachine.com

Date: Thu, 31 Dec 1998 22:48:36 -0500

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: flyback trans from mac monitor

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Ok, I'll make up for my off topic post....

You can use the ferrite doughnut from the back of the picture tube as a

choke...just pop it off, remove the wire, and wrap about 150 turns of #24

wire.....

-tom

Coiler wrote:

> *chuckle* you guys are

> a) on a religious argument that cannot be won.

> b) forgetting the *true* bug resistant OS (what can I say, been

>    a unix hack since 1984, when we still measured uptime in terms

>    of months, not weeks.)

> c) Forgetting this is a *TESLA* list, not an OS flamewar list.

>

> Lets get back to arcs and sparks, shall we?

>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: flyback trans from mac monitor

Date: Thu, 31 Dec 1998 22:49:00 -0800

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

*chuckle* you guys are

a) on a religious argument that cannot be won.

b) forgetting the *true* bug resistant OS (what can I say, been

a unix hack since 1984, when we still measured uptime in terms

of months, not weeks.)

c) Forgetting this is a *TESLA* list, not an OS flamewar list.

Lets get back to arcs and sparks, shall we?

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

(editors note. Michael was referring to a flame war that started on the

list on the PC vs MAC debate. very boring and rather childish and deleted

from the archives)